3GPP TSG-RAN WG2 Meeting #129bis                                                             R2-2503424

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

The objectives of Sub-band full duplex (SBFD) which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

RAN1 has sent the RRC parameter list in R1-2503155 and the RRC running CR is under discussion at [Post129bis][212][SBFD] Running CR for 38.331 (Huawei).

We are going to discuss the open issues and newly spotted issues in R1-2503155 and the RRC running CR:

- The RAN2 impacts on the open issues at #212 post email discussion.

- The data structure design of L1 UE-to-UE CLI measurement and reporting, L1 CLI-RSSI measurement and reporting and L1 SRS-RSRP measurement and reporting.

- LS to RAN1 on the observed issues in R1-2503155.

2. Discussion

2.1 Spotted issues in RRC running CR

- How to use PUCCH-CSI-ResourceExt is FFS

The existing running CR includes the pucch-CSI-ResourceListExt-r19 in CSI-ReportConfig IE as below, and put FFS on the IE PUCCH-CSI-ResourceExt.

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

***skip the unchanged******

]],

[[

nrofReportedCLImeasureResources-r19  ENUMERATED {n1, n2, n3, n4}                                            OPTIONAL,    -- Need R

pucch-CSI-ResourceListExt-r19        SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-ResourceExt-v19xy        OPTIONAL,    -- Need R

symbolType-r19                       ENUMERATED {sbfd, non-sbfd}                                            OPTIONAL    -- Need R

]]

}

[Editor’s note: How to use PUCCH-CSI-ResourceExt is FFS]

According to the RRC parameter list, the IE PUCCH-CSI-Resource should be extended to include symbolType as below. However the IE PUCCH-CSI-Resource is not extendable because of the original data design.

| symbolType | New | Configures the valid symbol type for PUCCH carrying P-CSI or SP-CSI for Configuration 1. Network does not configure this field if Configuration 2 is enabled for the UL BWP. | {sbfd, non-sbfd} | Per UL BWP In PUCCH-CSI-Resource | UE-specific | Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - For SP-CSI on PUCCH: o Option 2: the valid symbol type for PUCCH carrying SP-CSI is configured in semiPersistentOnPUCCH in CSI-ReportConfig. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for Type 1 CG PUSCH is configured in rrc-ConfiguredUplinkGrant in Con-figuredGrantConfig. - The valid symbol type for PUCCH configured for SR is configured in SchedulingRequestRe-sourceConfig. - The valid symbol type for PUCCH carrying P-CSI is configured in periodic in CSI-ReportConfig. |
| --- | --- | --- | --- | --- | --- | --- |

The same issue happens to reportQuantity in CSI-ReportConfig. The IE reportQuantity is not extendable either.

| reportQuantity | Existing | This field indicates the report quantity configured for a given CSI report | cli-RSSI' or 'cli-SRS-RSRP' | Per DL BWP In CSI-ReportConfig | UE-specific |
| --- | --- | --- | --- | --- | --- |

So these IEs should be extended in the right way. According to the existing running CR –v05, however the reportQuantityCLI is a new IE which doesn’t follow RAN1 parameter list.

Proposal 1-1: Support the running CR that pucch-CSI-ResourceList is extended directly in CSI-ReportConfig with PUCCH-CSI-ResourceExt-v19xy.

Proposal 1-2: RAN2 agree that the IE reportQuantity is extended directly in CSI-ReportConfig. Merge the TP1 below in to the RRC running CR.

------------------------------------TP1 on CSI-ReportConfig -------------------------------------------------------------------------------------------------------------------------------------------

.

.

.

]],

[[

nrofReportedCLImeasureResources-r19  ENUMERATED {n1, n2, n3, n4}                                            OPTIONAL,    -- Need R

pucch-CSI-ResourceListExt-r19        SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-ResourceExt-v19xy        OPTIONAL,    -- Need R

symbolType-r19                       ENUMERATED {sbfd, non-sbfd}                                            OPTIONAL,    -- Need R

reportQuantity-r19                	CHOICE {

cli-RSSI                              NULL,

cli-SRS-RSRP                          NULL,

} OPTIONAL,    -- Need R

]]

}

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19 is present, UE shall ignore reportQuantity (without suffix). Network does not configure reportQuantity-r17 or reportQuantity-r18 or reportQuantity-r19 together with reportQuantity-r16.

- How to use Uplink-powerControl-v19xy

Three new IEs are defined in Uplink-powerControl-r17 which is not extendable.

It is not specified how to use the Uplink-powerControl-v19xy in existing RRC running CR yet.

The TP how to use the Uplink-powerControl-v19xy should be specified in IE ServingCellConfig.

Proposal 2: RAN2 agree to use Uplink-powerControl-v19xy in ServingCellConfig and merge the TP2 into the RRC running CR.

---------------------------------------------TP2 on ServingCellConfig ----------------------------------------------------------------------------------------------------------------------

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

.

.

.

[[

mimoParam-v1850                     SetupRelease {MIMOParam-v1850}                                          OPTIONAL    -- Need M

]],

[[

mimoParam-v19xy                     SetupRelease {MIMOParam-v19xy}                                          OPTIONAL    -- Need M

]]

.

.

.

MIMOParam-v1850 ::= SEQUENCE {

additionalTDDConfig-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF  AdditionalTDDConfig-perPCI-ToAddMod-r18

OPTIONAL, -- Cond 2TA-TDD-Only

additionalTDDConfig-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17

OPTIONAL  -- Need N

}

MIMOParam-v19xy ::= SEQUENCE {

additionalUplinkPowerControlToAddModList-r19  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-v19xy        OPTIONAL,   -- Need N

additionalUplinkPowerControlToReleaseList-r19 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    	OPTIONAL,   -- Need N

...

}

2.2 Design of CLI measurement and reporting

- How to design CSI-ResourceConfig

The IE cli-MeasurementResourceSetList includes cli-RSSI-MeasurementResourceSetList or srs-RSRP-MeasurementResourceSetList according to the RRC parameter list as below:

| L1 UE-to-UE CLI measurement and reporting | cli-MeasurementResourceSetList | New | List of references to SRS-RSRP measurement resources set(s) or CLI-RSSI measurement resources set(s) used for L1 SRS-RSRP measurement and reporting or L1 CLI-RSSI measurement and reporting. | srs-RSRP-MeasurementResourceSetList or cli-RSSI-MeasurementResourceSetList | Per DL BWP In CSI-ResourceConfig |
| --- | --- | --- | --- | --- | --- |
| L1 CLI-RSSI measurement and reporting | cli-RSSI-MeasurementResourceSetList | New | List of references to CLI-RSSI measurement resources set(s) for L1 CLI-RSSI measurement and reporting in a CLI measurement resource set. Contains up to maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. FFS: maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig. | A length-N list of IDs of CLI-RSSI measurement resourceSet. N is FFS. | Per DL BWP In cli-MeasurementResourceSetList |
| L1 CLI-RSSI measurement and reporting | srs-RSRP-MeasurementResourceSetList | New | List of references to SRS-RSRP measurement resources set(s) used for L1 SRS-RSRP measurement and reporting in a CLI measurement resource set. Contains up to maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. FFS: maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig. | A length-N list of IDs of SRS-RSRP measurement resource set. N is FFS. | Per DL BWP In cli-MeasurementResourceSetList |

So the data structure of CSI-ResourceConfig is designed as below:

- CSI-ResourceConfig

  - cli-MeasurementResourceSetList

    - cli-RSSI-MeasurementResourceSetList

      - SEQUENCE OF CLI-RSSI-MeasurementResourceSetId

    - srs-RSRP-MeasurementResourceSetList

      - SEQUENCE OF SRS-RSRP-MeasurementResourceSetId

Proposal 3: RAN2 discuss the TP3 on CSI-ResourceConfig and merge it into the RRC running CR:

---------------------------------------------TP3 on CSI-ResourceConfig------------------------------------------------------------------------

CSI-ResourceConfig ::=      SEQUENCE {

]] ,

[[

cli-MeasurementResourceSetList-r19        CHOICE {

cli-RSSI-MeasurementResourceSetList-r19 SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig-r19)) OF CLI-RSSI-MeasurementResourceSetId-r19,

srs-RSRP-MeasurementResourceSetList-r19 SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig-r19)) OF SRS-RSRP-MeasurementResourceSetId-r19,

...

} OPTIONAL	-- Need R

]]

}

- How to design CSI-MeasConfig

The data structure of CSI-MeasConfig can be found as below according to the RRC parameter list:

- CSI-MeasConfig

  - cli-RSSI-MeasurementResourceSetToAddModList

    - CLI-RSSI-MeasurementResourceSet

      - cli-RSSI-MeasurementResourceSetId

      - cli-RSSI-MeasurementResourceIdList

      - aperiodicTriggeringOffset

  - cli-RSSI-MeasurementResourceSetToReleaseList

  - cli-RSSI-MeasurementResourceToAddModList

    - CLI-RSSI-MeasurementResource

      - cli-RSSI-MeasurementResourceId

      - startSymbol

      - nrofSymbols

      - startPRB

      - nrofPRBs

      - cli-RSSI-PeriodicityAndOffset

      - qclInfo-Periodic-CLI-RSSI-MeasurementResource

  - cli-RSSI-MeasurementResourceToReleaseList

  - srs-RSRP-MeasurementResourceSetToAddModList

    - SRS-RSRP-MeasurementResourceSet

      - srs-RSRP-MeasurementResourceSetId

      - srs-RSRP-MeasurementResourceIdList

      - aperiodicTriggeringOffset

  - srs-RSRP-MeasurementResourceSetToReleaseList

  - srs-RSRP-MeasurementResourceToAddModList

    - SRS-RSRP-MeasurementResource

      - srs-RSRP-MeasurementResourceId

      - srs-Resource-19

      - qcl-InfoPeriodicSRS-RSRP-MeasurementResource

  - srs-RSRP-MeasurementResourceToReleaseList

However there is no such data structure design in the existing RRC running CR. So we proposed:

Proposal 4-1: RAN2 agree the data structure of CSI-MeasConfig including these lists:

- cli-RSSI-MeasurementResourceSetToAddModList

- cli-RSSI-MeasurementResourceSetToReleaseList

- cli-RSSI-MeasurementResourceToAddModList

- cli-RSSI-MeasurementResourceToReleaseList

- srs-RSRP-MeasurementResourceSetToAddModList

- srs-RSRP-MeasurementResourceSetToReleaseList

- srs-RSRP-MeasurementResourceToAddModList

- srs-RSRP-MeasurementResourceToReleaseList

Proposal 4-2: RAN2 agree the data structure as below if proposal 4-1 is agreed:

- cli-RSSI-MeasurementResourceSetToAddModList

  - CLI-RSSI-MeasurementResourceSet

    - cli-RSSI-MeasurementResourceSetId

    - cli-RSSI-MeasurementResourceIdList

    - aperiodicTriggeringOffset

- cli-RSSI-MeasurementResourceToAddModList

  - CLI-RSSI-MeasurementResource

    - cli-RSSI-MeasurementResourceId

    - startSymbol

    - nrofSymbols

    - startPRB

    - nrofPRBs

    - cli-RSSI-PeriodicityAndOffset

    - qclInfo-Periodic-CLI-RSSI-MeasurementResource

- srs-RSRP-MeasurementResourceSetToAddModList

  - SRS-RSRP-MeasurementResourceSet

    - srs-RSRP-MeasurementResourceSetId

    - srs-RSRP-MeasurementResourceIdList

    - aperiodicTriggeringOffset

- srs-RSRP-MeasurementResourceToAddModList

  - SRS-RSRP-MeasurementResource

    - srs-RSRP-MeasurementResourceId

    - srs-Resource

    - qcl-InfoPeriodicSRS-RSRP-MeasurementResource

Proposal 4-3: RAN2 discuss and merge the TP4 on CSI-MeasConfig into the RRC running CR:

------------------------------------------------TP4 on CSI-MeasConfig------------------------------------------------------

CSI-MeasConfig ::=                  SEQUENCE {

.

.

.

]] ,

[[

cli-RSSI-MeasurementResourceToAddModList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResources-r19)) OF 	CLI-RSSI-MeasurementResource-r19   	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResources-r19)) OF 	CLI-RSSI-MeasurementResourceId-r19	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceSetToAddModList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF CLI-RSSI-MeasurementResourceSet-r19   	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceSetToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF CLI-RSSI-MeasurementResourceSetId-r19	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceToAddModList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResources-r19)) OF 	SRS-RSRP-MeasurementResource-r19   	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceToReleaseList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResources-r19)) OF 	SRS-RSRP-MeasurementResourceId-r19	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceSetToAddModList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResourceSets-r19)) OF SRS-RSRP-MeasurementResourceSet-r19   	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceSetToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF SRS-RSRP-MeasurementResourceSetId-r19	OPTIONAL -- Need N

]]

Proposal 4-4: The corresponding elements in the TP4 will be defined as separate IEs in RRC:

- CLI-RSSI-MeasurementResource,

- CLI-RSSI-MeasurementResourceId,

- CLI-RSSI-MeasurementResourceSet,

- CLI-RSSI-MeasurementResourceSetId,

- SRS-RSRP-MeasurementResource,

- SRS-RSRP-MeasurementResourceId,

- SRS-RSRP-MeasurementResourceSet ,

- SRS-RSRP-MeasurementResourceSetId.

2.3 LS to RAN1 on the observed issues in R1-2503155

Zero should not be included in the range of resourceSetCLI, according to the description: Entry number in cli-MeasurementResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

| L1 UE-to-UE CLI measurement and reporting | 38.214 | resourceSetCLI | New | CLI measurement resource set for L1 UE-to-UE CLI measurements. Entry number in cli-MeasurementResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on). | Integer the range of 0..maxNrofCLI-MeasurementResourceSetsPerConfig. maxNrofCLI-MeasurementResourceSetsPerConfig is FFS. | Per DL BWP In CSI-AssociatedReportConfigInfo->resourcesForChannelCLI | UE-specific | Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

So RAN1 should revise the RRC parameter list.

Proposal 5: RAN2 to send an LS to RAN1 to indicate the spotted issue in RRC parameter list:

- The range of resourceSetCLI should start from 1 instead of 0.

2.4 Others

There is a working assumption at RAN2#129bis meeting.

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

It can be confirmed since it follows the legacy rules.

Proposal 6: Confirm the Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

Spotted issues in RRC running CR

Proposal 1-1: Support the running CR that pucch-CSI-ResourceList is extended directly in CSI-ReportConfig with PUCCH-CSI-ResourceExt-v19xy.

Proposal 1-2: RAN2 agree that the IE reportQuantity is extended directly in CSI-ReportConfig. Merge the TP1 below in to the RRC running CR.

------------------------------------TP1 on CSI-ReportConfig ---------------------------------------------------------------

.

.

.

]],

[[

nrofReportedCLImeasureResources-r19  ENUMERATED {n1, n2, n3, n4}                                            OPTIONAL,    -- Need R

pucch-CSI-ResourceListExt-r19        SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-ResourceExt-v19xy        OPTIONAL,    -- Need R

symbolType-r19                       ENUMERATED {sbfd, non-sbfd}                                            OPTIONAL,    -- Need R

reportQuantity-r19                	CHOICE {

cli-RSSI                              NULL,

cli-SRS-RSRP                          NULL,

} OPTIONAL,    -- Need R

]]

}

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19 is present, UE shall ignore reportQuantity (without suffix). Network does not configure reportQuantity-r17 or reportQuantity-r18 or reportQuantity-r19 together with reportQuantity-r16.

Proposal 2: RAN2 agree to use Uplink-powerControl-v19xy in ServingCellConfig and merge the TP2 into the RRC running CR.

---------------------------------------------TP2 on ServingCellConfig -----------------------------------------------------

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

.

.

.

[[

mimoParam-v1850                     SetupRelease {MIMOParam-v1850}                                          OPTIONAL    -- Need M

]],

[[

mimoParam-v19xy                     SetupRelease {MIMOParam-v19xy}                                          OPTIONAL    -- Need M

]]

.

.

.

MIMOParam-v1850 ::= SEQUENCE {

additionalTDDConfig-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF  AdditionalTDDConfig-perPCI-ToAddMod-r18

OPTIONAL, -- Cond 2TA-TDD-Only

additionalTDDConfig-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17

OPTIONAL  -- Need N

}

MIMOParam-v19xy ::= SEQUENCE {

additionalUplinkPowerControlToAddModList-r19  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-v19xy        OPTIONAL,   -- Need N

additionalUplinkPowerControlToReleaseList-r19 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    	OPTIONAL,   -- Need N

...

}

Design of CLI measurement and reporting

Proposal 3: RAN2 discuss the TP3 on CSI-ResourceConfig and merge it into the RRC running CR:

---------------------------------------------TP3 on CSI-ResourceConfig------------------------------------

CSI-ResourceConfig ::=      SEQUENCE {

]] ,

[[

cli-MeasurementResourceSetList-r19        CHOICE {

cli-RSSI-MeasurementResourceSetList-r19 SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig-r19)) OF CLI-RSSI-MeasurementResourceSetId-r19,

srs-RSRP-MeasurementResourceSetList-r19 SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig-r19)) OF SRS-RSRP-MeasurementResourceSetId-r19,

...

} OPTIONAL	-- Need R

]]

}

Proposal 4-1: RAN2 agree the data structure of CSI-MeasConfig including these lists:

- cli-RSSI-MeasurementResourceSetToAddModList

- cli-RSSI-MeasurementResourceSetToReleaseList

- cli-RSSI-MeasurementResourceToAddModList

- cli-RSSI-MeasurementResourceToReleaseList

- srs-RSRP-MeasurementResourceSetToAddModList

- srs-RSRP-MeasurementResourceSetToReleaseList

- srs-RSRP-MeasurementResourceToAddModList

- srs-RSRP-MeasurementResourceToReleaseList

Proposal 4-2: RAN2 agree the data structure as below if proposal 4-1 is agreed:

- cli-RSSI-MeasurementResourceSetToAddModList

  - CLI-RSSI-MeasurementResourceSet

    - cli-RSSI-MeasurementResourceSetId

    - cli-RSSI-MeasurementResourceIdList

    - aperiodicTriggeringOffset

- cli-RSSI-MeasurementResourceToAddModList

  - CLI-RSSI-MeasurementResource

    - cli-RSSI-MeasurementResourceId

    - startSymbol

    - nrofSymbols

    - startPRB

    - nrofPRBs

    - cli-RSSI-PeriodicityAndOffset

    - qclInfo-Periodic-CLI-RSSI-MeasurementResource

- srs-RSRP-MeasurementResourceSetToAddModList

  - SRS-RSRP-MeasurementResourceSet

    - srs-RSRP-MeasurementResourceSetId

    - srs-RSRP-MeasurementResourceIdList

    - aperiodicTriggeringOffset

- srs-RSRP-MeasurementResourceToAddModList

  - SRS-RSRP-MeasurementResource

    - srs-RSRP-MeasurementResourceId

    - srs-Resource

    - qcl-InfoPeriodicSRS-RSRP-MeasurementResource

Proposal 4-3: RAN2 discuss and merge the TP4 on CSI-MeasConfig into the RRC running CR:

------------------------------------------------TP4 on CSI-MeasConfig------------------------------------------------------

CSI-MeasConfig ::=                  SEQUENCE {

.

.

.

]] ,

[[

cli-RSSI-MeasurementResourceToAddModList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResources-r19)) OF 	CLI-RSSI-MeasurementResource-r19   	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResources-r19)) OF 	CLI-RSSI-MeasurementResourceId-r19	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceSetToAddModList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF CLI-RSSI-MeasurementResourceSet-r19   	OPTIONAL, -- Need N

cli-RSSI-MeasurementResourceSetToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF CLI-RSSI-MeasurementResourceSetId-r19	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceToAddModList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResources-r19)) OF 	SRS-RSRP-MeasurementResource-r19   	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceToReleaseList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResources-r19)) OF 	SRS-RSRP-MeasurementResourceId-r19	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceSetToAddModList	SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasurementResourceSets-r19)) OF SRS-RSRP-MeasurementResourceSet-r19   	OPTIONAL, -- Need N

srs-RSRP-MeasurementResourceSetToReleaseList	SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-MeasurementResourceSets-r19)) OF SRS-RSRP-MeasurementResourceSetId-r19	OPTIONAL -- Need N

]]

Proposal 4-4: The corresponding elements in the TP4 will be defined as separate IEs in RRC:

- CLI-RSSI-MeasurementResource,

- CLI-RSSI-MeasurementResourceId,

- CLI-RSSI-MeasurementResourceSet,

- CLI-RSSI-MeasurementResourceSetId,

- SRS-RSRP-MeasurementResource,

- SRS-RSRP-MeasurementResourceId,

- SRS-RSRP-MeasurementResourceSet ,

- SRS-RSRP-MeasurementResourceSetId.

LS to RAN1 on the observed issues in R1-2503155

Proposal 5: RAN2 to send an LS to RAN1 to indicate the spotted issue in RRC parameter list:

- The range of resourceSetCLI should start from 1 instead of 0.

Others

Proposal 6: Confirm the Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #120bis

- RAN2 Chair Notes WG2 #129bis

- R1-2503154 LS on Rel-19 higher layers parameters list Post RAN1#120bis Ericsson