3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503428

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Further Consideration on XR-specific RLC Enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the open issues on timely RLC retransmissions including enhanced polling and autonomous retransmission, and avoiding unnecessary retransmission for XR, our views on these issues are given in the following part.

Discussion

Timely RLC retransmissions for XR

In RAN2#129bis meeting, the agreement on timely retransmissions on RLC layer is as follows [1].

Autonomous retransmissions and polling enhancements For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission. Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

[RLC-9]The triggering condition

For FFS, when an SDU is being transmitted over the air in an active HARQ process, it may not be necessary to immediately trigger a new RLC retransmission when remaining time threshold is reached. Some proposed to define a new timer for a RLC PDU which is started when a RLC PDU is submitted to lower layers. But it is confusing that after a new defined discardTimer, another prohibit timer is introduced. One simple way is that the network defines the discardTimer appropriately which would consider the initial HARQ transmission including the remaining time.

Proposal 1: (RLC-9) The network defines the discardTimer appropriately which would consider the initial HARQ transmission including the remaining time.

[RLC-7] Autonomous Retransmission coexists with ARQ

In current running CR at RAN2#129bis Post Email [512], there is a FFS issue as below:

38.322 Running CR for RAN2#129bis Post Email discussion Editor’s Note: It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission.

Similar to the legacy procedure, the autonomous retransmission will have higher priority than transmission of not previously transmitted RLC SDUs or RLC SDU segments. For the autonomous retransmission and legacy retransmission, there is no significant benefit forseen when the priority among them is considered. It is suggested they will be transmitted in the order of SN of RLC SDU or segment.Since the number of autonomous retransmission(s) could not be used to deduce that channel quality is bad, it is not necessary to increase the RETX_COUNT when autonomous retransmission is performed.

Proposal 2: (RLC-7) For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

Proposal 3: (RLC-7) It is not necessary to increase the RETX_COUNT when autonomous retransmission is performed.

[RLC-8]One or multiple enhanced polling:

38.322 Running CR for RAN2#129bis Post Email discussion Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

For above FFS issue on polling, more remaining time is needed for receiving status reporting, so there is not so much time remained for multiple pollings when the remaining time is limited. Since there is only one time of autonomous retransmission, so, one enhanced polling is preferable, which is simple and sufficient in the congestion scenario.

Proposal 4:  (RLC-8) Only a single enhanced polling will be triggered per RLC SDU.

Avoid unnecessary retransmission

[RLC-6]The impact on RLF

38.322 Running CR for RAN2#129bis Post Email discussion Editor’s Note: FFS whether there are any RLF detection impacts when avoiding unnecessary retransmissions is introduced.

In legacy RLC layer procedure, the following procedure is applied for RLF.  It can be seen that the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

When the autonomous retransmission is introduced, the number of autonomous retransmission(s) could not be used to deduce that channel quality is bad enough to indicate the RLF to high layer, it is not necessary to increase the RETX_COUNT when autonomous retransmission is performed, there is no impact to spec.

Proposal 5: (RLC-6) There is no specification impact foreseen for RLF.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: (RLC-9) The network defines the discardTimer appropriately which would consider the initial HARQ transmission including the remaining time.

Proposal 2: (RLC-7) For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

Proposal 3: (RLC-7) It is not necessary to increase the RETX_COUNT when autonomous retransmission is performed.

Proposal 4:  (RLC-8) Only a single enhanced polling will be triggered per RLC SDU.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

Proposal 5: (RLC-6) There is no specification impact foreseen for RLF.

Reference

- R2-2502985, Report from session on R19 XR, Huawei, April 2025.