3GPP TSG-RAN WG2 Meeting #130	R2-2503440

# St Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

# Introduction

In RAN2#129bis meeting, following was agreed on XR rate control [1]:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In RAN#106 meeting, downlink congestion indication (in addition of the uplink) was discussed, and slide#3 of RP-243305 [2] is endorsed, with related content as below:

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

In this contribution, we discus open issues of XR rate control.

# Discussion

## (MAC-11) DL rate control

In “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)” [3], one open issue is: [MAC-11] Whether to apply the same design for UL rate control design to DL data transmission.

In RAN plenary, following was concluded on DL rate control:

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

RAN2 discussion is focused on the bit rate table design. The rate control MAC CE can be naturally extended to DL similarly to legacy Recommended Bit Rate MAC, where an Uplink/Downlink (UL/DL) field is used, as below:

TS 38.321 Figure 6.1.3.20-1: Recommended bit rate MAC CE

-	Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink;

As the main purpose of DL rate control is to adjust the DL data rate at the communication peer based on the adjustment of UL data rate, the range / granularity of uplink bit rate recommended by SA4 is applicable for DL rate control, i.e. there is no need to consider the specific requirements of bit rate of DL XR traffic.

It is therefore proposed that RAN2 to confirm that same design for UL rate control design is applicable to DL data transmission, and inform SA2/SA4/RAN3 about the agreement.

Proposal 1: (MAC-11) RAN2 confirm that same design for UL rate control design is applicable to DL data transmission, and inform SA2/SA4/RAN3 about the agreement.

## (MAC-09) Rate control in DC

In “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)” [3], one open issue is: [MAC-09] How UE should handle the Rate Control MAC CE in DC configuration.

In DC scenario, there are three types of bearers: MCG bearer, SCG bearer, and split bearer. TS 37.340 Figure 4.2.2-2 is copied below for reference.

Figure 4.2.2-2: Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in MR-DC with 5GC (NGEN-DC, NE-DC and NR-DC).

For rate control in DC configuration, a gNB can only control its responsible bearers. For example, MN can control MCG bearer and split bearer, but not SCG bearer. Similarly, SN can control SCG bearer and split bearer, but not MCG bearer. This is the principle used by other MAC CEs in DC configuration. For example, for Duplication Activation/Deactivation MAC CE, following is defined in TS 38.321 clause 6.1.3.11:

Di: This field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity.

Split bearer can be used for XR services to boost the throughput. If both MN and SN send MAC CEs for rate control of the same QoS flow(s) mapped to split bearer, some discussion might be needed on UE behavior regarding which data rate to be applied for the split bearer or DRB as indicated in the MAC CEs, e.g. whether UE follows latest MAC CE from MN or SN, or UE should consider the MAC CEs jointly. Given that each gNB sends MAC CE to control the physical bit rate, one straightforward approach is that for split bearer, the aggregated bit rate provided by MN and SN is indicated to upper layers.

Proposal 2: (MAC-09) In DC configuration, UL rate control MAC CE is only applicable to QoS flows served in the MAC entity. For split bearer, the MAC entity indicates the aggregated bit rate provided by MN and SN to upper layers.

## (MAC-04) QoS flow identification

In RAN2#129bis meeting, RAN2 agreed that “From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. FFS how QFI is indicated, e.g. with DRB ID + QFI ID.” A related open issue is captured in “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)” [3]: [MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier.

To indicate QoS flow, at least QFI is needed. Signalling QFI alone is not sufficient since QFI is only unique with a PDU session. In NR SDAP header, PDU Session ID is not signalled since one DRB can only contain data from one PDU Session, as from ASN.1 below. In MAC CE, indication of PDU Session ID / DRB ID / LCID is needed to uniquely identify a QoS flow. Considering QFI has 6 bits, the signalling overhead to indicate QoS flow is 14 / 12 / 12 bits when using PDU Session ID / DRB ID / LCID, respectively.

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

...

}

PDU-SessionID ::=   INTEGER (0..255)

In addition to signalling overhead, the main difference among these options is how to capture the restriction in Proposal 2 that UL rate control MAC CE is only applicable to QoS flows served in the MAC entity. For PDU Session ID and DRB ID, some specification work is needed. Since LCID is bound to MAC entity, additional specification efforts are not needed in case of DC.

Table 1: Comparison of QoS flow indication options

|  | PDU Session ID + QFI | DRB ID + QFI | LCID + QFI |
| --- | --- | --- | --- |
| Signalling overhead | 14 bits | 12 bits | 12 bits |
| In DC configuration, whether additional specification efforts are needed to ensure that UL rate control MAC CE is only applicable to QoS flows served in the MAC entity | Yes | Yes | No |

From above discussion, it can be seen that LCID + QFI approach has less signalling overhead and is simpler to support DC configuration. In addition, LCID is also used in legacy Recommended bit rate MAC CE. It is therefore proposed to use LCID + QFI to indicate QoS flow.

Proposal 3: (MAC-04) QoS flow is indicated with LCID + QFI.

## (MAC-05, MAC-06) Rate control MAC CE format

In “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)” [3], one open issue is: [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows.

A single Rate Control MAC CE indicating multiple QoS flows can save some signalling overhead (e.g. MAC subheader) at the cost of additional standardization efforts. It is suggested to go with single QoS flow approach for simplicity.

Proposal 4: (MAC-05) Rate Control MAC CE includes bit rate for single QoS flow.

Another open issue is [MAC-06] Any additional fields that should be included in the Rate Control MAC CE. From previous proposals, following fields are needed: LCID, QFI, UL/DL, Bit Rate. In RAN#129 meeting, RAN2 agreed that “We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed”. In MAC running CR of “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)”, 8 bit table is designed to cover the 100 kbps to 40 Mbps bit range proposed by SA4, and consequently multiplier is not needed.

Proposal 5: (MAC-06) Rate Control MAC CE includes the following fields: LCID, QFI, UL/DL, Bit Rate.

A related open issue is [MAC-07] Format of the Rate Control MAC CE. Based on Proposal 5 above, an example Rate Control MAC CE format is shown in Figure 1 below.

Figure 1: Rate Control MAC CE format

## (RRC-1, MAC-08, MAC-10) Bit rate query

In “[POST129bis][503][XR] RRC running CR and open issues (Huawei)” [4], one open issue is Issue 1: FFS how to indicate whether bit rate query is enabled based on which granularity (QoS flow level or DRB level). Given that RAN2 agreed that rate control is indicated per QoS flow level, it is natural to have bit rate query enabling / disabling in QoS flow level. In addition, since UL QoS flow to DRB mapping can be changed either due to RRC reconfiguration or from reflective mapping, DRB level control of bit rate query is not consistent with QoS flow level rate control.

Proposal 6: (RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

In “[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)” [3], there are two open issues related to bit rate query: [MAC-08] Handling of triggered UL rate queries (e.g. multiplexing, transmission and cancelation, etc) and [MAC-10] Behavior of bitRateQueryProhibitTimer. It is straightforward to following legacy Recommended bit rate query procedure, with the change of logical channel to QoS flow.

Proposal 7: (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

An example TP is as follows:

The MAC entity may transmit a UL Rate Control MAC CE to the serving gNB to query available bit rate or request a desired bit rate for a QoS flow and for a direction (i.e. for uplink or downlink). If the MAC entity is requested by upper layers to do so, it shall:

1>	if a bit rate query for this QoS flow and this direction has not been triggered:

2>	trigger a bit rate query for this QoS flow, direction.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	for each bit rate query that the bit rate query procedure determines has been triggered and not cancelled:

2>	if bitRateQueryProhibitTimer for the QoS flow and the direction of this bit rate query is configured, and it is not running; and

2>	if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Rate Control MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the Rate Control MAC CE for the QoS flow and the direction of this bit rate query;

3>	start the bitRateQueryProhibitTimer for the QoS flow and the direction of this bit rate query;

3>	cancel this bit rate query.

# Conclusion

In this contribution, we discuss open issues of XR rate control, and propose the following:

Proposal 1: (MAC-11) RAN2 confirm that same design for UL rate control design is applicable to DL data transmission, and inform SA2/SA4/RAN3 about the agreement.

Proposal 2: (MAC-09) In DC configuration, UL rate control MAC CE is only applicable to QoS flows served in the MAC entity. For split bearer, the MAC entity indicates the aggregated bit rate provided by MN and SN to upper layers.

Proposal 3: (MAC-04) QoS flow is indicated with LCID + QFI.

Proposal 4: (MAC-05) Rate Control MAC CE includes bit rate for single QoS flow.

Proposal 5: (MAC-06) Rate Control MAC CE includes the following fields: LCID, QFI, UL/DL, Bit Rate.

Proposal 6: (RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

Proposal 7: (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

# References

[1] R2-2502985, Session chair (Huawei), “Report from session on R19 XR”

[2] RP-243305, Nokia (moderator), “Release 19 XR enhancements - Way forward on WID scope revision”

[3] R2-2503361, Qualcomm Incorporated, “List of open issues in MAC”

[4] R2-2503788, Huawei, HiSilicon, “Summary of [POST129bis][503][XR] RRC running CR (Huawei)”