3GPP TSG-RAN WG2 Meeting #130	R2-2503441

# St Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

# Introduction

In RAN2#129bis meeting, following was agreed regarding other aspects of SBFD [1]:

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration.

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

In this contribution, we discuss RAN2 aspects of SBFD.

# Discussion

## (RRC-2) Coexistence between LTM and SBFD

In “[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)” [2] R2-2504444, Samsung, "MAC running CR for Evolution of NR duplex operation: SBFD"

[3], there is one open issue as follows:

[Proposal for RRC-2] RAN2 to decide whether ” Early UL synchronization with an LTM candidate cell” and/or ”RACH-based LTM” can be supported in SBFD symbols and the RRC signallings if to support. (based on companies’ meeting contributions, 11/16 companies prefer not to support.)

Firstly, supporting SBFD and LTM together might be beyond the scope of the duplex and mobility WIs.

Secondary, the motivation of supporting LTM related RACH on SBFD symbols is not clear. The main motivation of RACH on SBFD symbols is to reduce latency. However, early UL synchronization with an LTM candidate cell is performed before handover, and is not related to the service interruption time during handover (LTM cell switch). Therefore “early UL synchronization with an LTM candidate cell” in SBFD symbols does not bring the latency reduction for mobility. Although RACH-based LTM in SBFD symbols might bring some latency reduction gain, the most gain comes from RACH-less LTM.

Thirdly, there are some standardization efforts to support this. For example, UE should be aware of RO type, RACH configuration, and SBFD time / frequency configuration before performing early UL synchronization to candidate cell in SBFD symbols.

In summary, we don’t think standardization efforts are justified to support “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM” in SBFD symbols.

Proposal 1: (RRC-2) “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM” are not supported in SBFD symbols.

## SP CLI measurement resource set activation/deactivation MAC CE

In “[Post129bis][213][SBFD] Running CR for 38.321 (Samsung)”, MAC running CR [2] has the following:

SP CLI measurement resource set ID: This field contains either an index of SRS-ResourceConfigCLI containing Semi Persisten SRS-RSRP measurement resources as specified in TS 38.331 [5], indicating the Semi Persistent SRS-RSRP measurement resource set, or an index of RSSI-ResourceConfigCLI containing Semi Persistent CLI-RSSI measurement resources as specified in TS 38.331 [5], indicating the CLI-RSSI measurement resource set, which shall be activated or deactivated. The length of the field is [FFS] bits;

Editor’s Note: The field length of 6-bit in the figure is tentative, and only for illustration purpose. It will be aligned with further RAN1 agreement(s).

SP CLI measurement resource set ID is related to the following fields in RRC running CR [4] below:

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

...

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

[[

cli-MeasResourceSetList-r19                    CHOICE {

cli-RSSI-MeasResourceSetList-r19    CHOICE {

CLI-RSSI-MeasResourceSetList-r19,

CLI-RSSI-MeasResourceSet-r19

},

srs-RSRP-MeasResourceSetList-r19    CHOICE {

SRS-RSRP-MeasResourceSetList-r19,

SRS-RSRP-MeasResourceSet-r19

}

}

]]

}

CLI-RSSI-MeasResourceList-r19 ::=       SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResources-r19)) OF CLI-RSSI-MeasResource-r19

CLI-RSSI-MeasResourceSetList-r19 ::=    SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSets-r19)) OF CLI-RSSI-MeasResourceSet-r19

SRS-RSRP-MeasResourceList-r19 ::=       SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResources-r19)) OF SRS-RSRP-MeasResource-r19

SRS-RSRP-MeasResourceSetList-r19 ::=    SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSets-r19)) OF SRS-RSRP-MeasResourceSet-r19

CLI-RSSI-MeasResourceSet-r19 ::=    SEQUENCE {

aperiodicTriggeringOffset-r19       INTEGER (0..31)                 OPTIONAL,   -- Need R

cli-RSSI-MeasResourceSetId-r19      CLI-RSSI-MeasResourceSetId      OPTIONAL,   -- Need R

cli-RSSI-MeasResourceIdList-r19     SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourcesPerSet-r19) ) OF CLI-RSSI-MeasResourceId                          OPTIONAL,   -- Need R

...

}

SRS-RSRP-MeasResourceSet-r19 ::=      SEQUENCE {

aperiodicTriggeringOffset-r19         INTEGER (0..31)         OPTIONAL,   -- Need R

srs-RSRP-MeasResourceSetId-r19        SRS-RSRP-MeasResourceSetId     OPTIONAL,   -- Need R

srs-RSRP-MeasResourceIdList-r19       SEQUENCE (SIZE (1.. maxNrofSRS-RSRP-MeasResourcesPerSet-r19) ) OF SRS-RSRP-MeasResourceSetId                          OPTIONAL,   -- Need R

...

}

From above, it can be seen that SP CLI measurement resource set ID refers to either cli-RSSI-MeasResourceSetId-r19 or srs-RSRP-MeasResourceSetId-r19.

Proposal 2: In SP CLI measurement resource set activation/deactivation MAC CE, field SP CLI measurement resource set ID refers to either cli-RSSI-MeasResourceSetId-r19 or srs-RSRP-MeasResourceSetId-r19.

## Interference issue

In Rel-19 SBFD, gNB can receive in UL and transmit in DL simultaneously. An example is shown in Figure 1 below. In Case 1, the DL reception may cause interference to the PUSCH reception in network side. In Case 2, neighbor cell’s DL transmission may also cause interference to the PUSCH reception in network side, i.e., CLI. Although the interference issue above could be mitigated to certain extent by implementation as well as CLI handling techniques, the PUSCH transmitted in SBFD symbols still suffers more interference than PUSCH transmitted in non-SBFD symbols. RAN2 needs to discuss whether such PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Figure 1: Interference issue of PUSCH in SBFD symbols

Note that quality difference between SBFD and non-SBFD symbols for RACH was already identified by RAN1, which agreed the following in RAN1#118bis meeting:

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

One potential enhancement to consider PUSCH quality difference between SBFD and non-SBFD symbols is to make LCP aware whether PUSCH resource contains SBFD symbols, e.g. during the step of selection of logical channel.

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

# Conclusion

In this contribution, we discuss RAN2 aspects of SBFD, and propose the following:

Proposal 1: (RRC-2) “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM” are not supported in SBFD symbols.

Proposal 2: In SP CLI measurement resource set activation/deactivation MAC CE, field SP CLI measurement resource set ID refers to either cli-RSSI-MeasResourceSetId-r19 or srs-RSRP-MeasResourceSetId-r19.

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

# References

[1] R2-2502982, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others"

[2] R2-2504444, Samsung, "MAC running CR for Evolution of NR duplex operation: SBFD"

[3] R2-2503866, Huawei, HiSilicon, "Remaining RRC open issues in feature SBFD"

[4] R2-2503862, Huawei, HiSilicon, "RRC running CR for Evolution of NR duplex operation (SBFD)"