3GPP TSG RAN WG2 Meeting #130         	                          R2-2503449

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Discussion on open issues of AI/ML air LCM

Document for:	Discussion and Decision

# Introduction

In RAN2 #129bis meeting, RAN2 agreed Option B is provided via OtherConfig, although detailed RRC parameter list is waiting for further RAN1 input.

Considering both Option A and Option B are supported to config/report functionality applicability, in this contribution, we further discuss the association between Option A and Option B (especially when both options are configured). Furthermore, it is not clear what is the UE behavior upon receiving configuration from Option B, especially how UE reports its applicability reporting.

Additionally, based on current running CRs, there are still some remaining open issues on applicability reporting and UE data collection request, including inapplicability cause value, activation for non-applicable functionality when it becomes available, etc.

In this contribution, we further discuss the above remaining open issues of AI/ML air LCM.

# Discussion

## Applicability Configuration/Reporting (RRC-1/7/8)

Applicability reporting based on Option B (RRC-7)

During RAN2 #129bis meeting, RAN2 assumes Option B configuration can be provided via OtherConfig. However, it is not clear how UE reports its applicable/non-applicable functionalities upon receiving configuration from Option B.

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Note that following applicability reporting message was agreed in RAN2 #129 meeting:

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Since RAN2 aims to design a unified procedure for Option A and B, same as Option A, UE can also use the initial applicability report in RRCReconfigurationComplete. UAI is used to send updated functionality applicability.

Proposal 1: (RRC-7) Upon receiving configurations from Option B, same as Option A, UE sends the initial applicability report in RRCReconfigurationComplete. UAI is used to send updated functionality applicability.

Explicit inapplicability cause value (RRC-1)

During RAN2 #129bis meeting, explicit inapplicability cause value was discussed and RAN2 reached following agreement:

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

During the discussion, it was clarified that the inapplicability cause value would be helpful for network to know model availability at UE-side, where network can release the corresponding inference configuration directly if model is not available at UE-side or network can trigger the corresponding model transfer/delivery towards UE/UE-side OTT server for two-sided model.

It was further discussed that the model at UE side may be temporarily not available and network doesn’t have to release the corresponding inference configuration, as such configuration could be applicable later on when model becomes available. On the other hand, if the model will not be available or will be available after a long time, network may decide to release such configuration first for resource saving over the air interface.

Therefore, to assist network’s decision, UE can further indicate how long its model will become available, together with inapplicability cause value ‘model is not available’. Then, it is up to network whether the corresponding inference configuration can be released or not according to the ‘waiting duration’ of model availability at UE side.

Proposal 2: (RRC-1) UE reports non-applicability cause as ‘model is not available’, optionally with when (how long) model will be available.

Co-existence/Association between Option A and Option B (RRC-8)

As mentioned by RAN1 LS [1], the applicability report is based on A) and/or B). This implies it is possible for network to provide 1) only Option A, 2) only Option B, 3) both Option A and Option B.

It is further observed that ‘Applicability reporting procedure’ has been widely used in current RRC running CR:

1> if the received otherConfig includes applicabilityReportConfig; 2> if applicabilityReportConfig is set to setup: 3> consider itself to be configured to report applicability information of configurations subject to the applicability determination procedure in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to report applicability information of configurations subject to the applicability determination procedure;

In our understanding, during previous RAN2 discussion, both Option A and Option B falls into the procedure of UE applicability determination. Therefore, further clarification on RAN2 agreement should be considered:

Proposal 3: (RRC-8) Applicability determination procedure includes both Option A and Option B. Network can provide applicabilityReportConfig for Option A and/or Option B. Co-existence of Option A (via CSI-ReportConfig) and Option B (via OtherConfig) in same RRCReconfiguration message is supported.

Recalling that RAN2 agreed that UE can only provide UAI reporting if it is allowed to do so based on configuration in OtherConfig (i.e., applicabilityReportConfig).

- “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

RAN2 further agreed the initial applicability reporting is provided by UE via RRCReconfigurationComplete message, at least for Option A.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

However, it is observed that, if the network configuration on whether UE is allowed to do applicability reporting is only for UAI reporting, it is possible that UE can only report initial applicability via RRCReconfigurationComplete, but not allowed to provide updated applicability via UAI if it’s not configured to do so. This means, if there’s an applicable functionality becomes inapplicable, UE cannot send its updated applicablity reporting to network. In our understanding, such inconsistent applicability reporting (i.e., UE can only provdie initial applicaiblity but not the following-up updates) may lead to performance downgradation. Furthermore, RAN2 further agrees not to introduce prohibit timer for this applicability reporting via UAI, which hints that network can always get the up-to-date applicaiblity information from UE.

- No prohibit timer is introduced

If UE is not allowed to do UAI reporting and without real-time information of UE’s model non-applicaiblity, network may activates the non-applicable functionalities at UE side, while there’s no way for UE to activate such functionality since it’s currently not applicable.

Observation 1: If UE only reports initial applicability reporting via RRCReconfigurationComplete but not allowed to report updates via UAI, network may wrongly activates non-applicable functionalities (initailly applicable becomes non-applicable) at UE-side.

There are two alternatives to solve the inconsistency of UE initial/updated applicability reporting:

- Network shall always allow UE to do UAI reporting

- Network configuration on whether UE is allowed to provide applicability reporting is applicable for both initial reporting (via RRCReconfigurationComplete) and updated reporting (via UAI)

The selection among above two alternatives come to the question whether there’s a case that network only provides inference configuration but not need UE to provide applicability reporting immediately. In our understanding, this is possible since network may only provide potential feasible inference configurations to UE at the beginning for UE to check applicability, but not ready to activate AI/ML related functionalities immediately (especially for periodic CSI). Based on this assumption, network can provide inference configuration first, but not configure UE to report its applicability reporting. Later on, network can configure UE is allowed to do applicability reporting (for both initial and updated reporting) via OtherConfig.

Proposal 4: UE is allowed to report initial and updated applicability reporting consistently, that is:

- if applicabilityReportConfig is set to setup, UE considers itself to be configured to report applicability information of configurations subject to the applicability determination procedure via RRCReconfigurationComplete and UAI;

- else, UE considers itself not to be configured to report applicability information of configurations subject to the applicability determination procedure via either RRCReconfigurationComplete or UAI;

Furthermore, when both Option A and Option B are provided in Step 3, it is not clear to UE whether there’s any association between Option A and Option B.

It is observed that RAN1 has defined different functionalities for beam management:

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] |
| --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] |

Based on different use cases/feature groups defined by RAN1, network may need to configure multiple applicability reporting configuration to UE. Hence, applicability configuration ID can be introduced within applicabilityReportConfig to differentiate different use cases/feature groups, so that UE can decide/report applicability for different use cases independently.

Proposal 5: Introduce applicability report configuration ID inside ApplicabilityReportConfig.

With this, the CSI-ReportConfigId(s) of the corresponding applicability reporting can be further included in applicabilityReportConfig. Additionally, UE can also report the applicability report configuration ID in applicability reporting, either via RRCReconfigurationComplete or UAI.

An example of signaling structure is shown as below:

ApplicabilityReportConfig-r19 ::= SEQUENCE { applicabilityReportConfigList-r19 ApplicabilityReportConfigList-r19 OPTIONAL } AppllicabilityReportConfigList-r19 ::= SEQUENCE (SIZE (1..maxApplicabilityReport-r19)) OF applicabilityReportInformation-r19 s ApplicabilityReportInformation-r19 ::= SEQUENCE { applicableReportConfigId ApplicableReportConfigId, -- association to Option A, if present, (list of) Option A is configured. associatedCSI-ReportConfigId CSI-ReportConfigId(or list) OPTIONAL, -- Configuration of Option B applicabilityReportPartialInformation-r19 SetupRelease { ApplicabilityReportPartialInformation } OPTIONAL } ApplicabilityReportPartialInformation ::= SEQUENCE { [option B IE 1] OPTIONAL, [option B IE 2] OPTIONAL, [option B IE 3] OPTIONAL … }

During applicability reporting, when both Option A and Option B are provided in Step 3, UE can provide applicable/non-applicable report configuration IDs provide in ApplicabilityReportConfig. UE further includes the applicable CSI-ReportConfigId(s) of Option A and/or whether Option B is applicable or not under each reported configuration ID.

Proposal 6: (RRC-8) To support coexistence between Option A and Option B:

- During configuration, under applicableReportConfigId, network includes CSI-ReportConfigId(s) of Option A for that applicableReportConfigId, and/or inference related parameters of Option B that are associated with applicability report configuration ID.

- During reporting, UE reports the applicable/non-applicable report configuration ID in applicability reporting (via RRCReconfigurationComplete or UAI). UE further includes the applicable/non-applicable CSI-ReportConfigId(s) of Option A and/or whether Option B is applicable or not under each reported configuration ID.

## Other open Issues on applicability reporting (RRC-4/5/6)

Activation/Deactivation of applicable functionality reporting via UAI (RRC-4)

It was agreed in RAN2 #129 meeting that the periodic CSI reporting is activated autonomously by UE while reporting RRCReconfiguationComplete in Step 4.

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

However, UE can also report a non-applicable functionality becomes applicable via UAI as a follow-up change, as discussed earlier. If a non-applicable functionality becomes applicable, and UE reports its applicability via UAI, it not clear whether the periodic CSI reporting of such non-applicable functionality can be autonomously activated by UE or not.

Note that UAI is normally considered as an assistance information, but not a configuration response. The periodic CSI reporting is not desired to be activated autonomously by UE upon reporting via UAI. Hence, different from UE considers its periodic reporting is activated upon reporting RRCReconfigurationComplete, UE should wait for an explicit activation/deactivation (e.g. a new MAC CE) indication from network to activate its periodic CSI reporting.

Proposal 7: (RRC-4) If a non-applicable functionality becomes applicable and reported by UE via UAI, network activates periodic CSI reporting via a new MAC CE explicitly. FFS on the detail design of new MAC CE.

Reporting of inapplicable periodic beam prediction (RRC-5)

It was questioned that UE may continuously report periodic beam prediction even after UE becomes inapplicable. Recalling RAN2 agreed UE can report its functionality becomes from non-applicable to applicable via UAI without any prohibit timer, which means UE can report the change of applicability of any functionality immediate to network.

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - No prohibit timer is introduced

Observation 2: (RRC-5) UE can report inapplicability of its functionality immediately, since there’s no prohibit timer for UAI reporting.

Furthermore, as UE shall maintain all inference configurations until network explicitly releases it, UE may continuously report periodic beam prediction to network. However, in our understanding, upon receiving the inapplicability UAI reporting from UE, network explicitly knows which functionality is not applicable, where network can release such configuration directly.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Based on above observations, network receiving periodic beam prediction after inapplicability reporting will not happen.

Proposal 8: (RRC-5) Upon receiving UAI reporting containing inapplicable functionality, network shall release configuration of the corresponding inapplicable functionality immediately. Network receiving periodic beam prediction after inapplicability reporting will not happen.

Applicability configuration handling during RLF, RRC_IDLE/INACTIVE (RRC-6)

Similar as legacy RRC configuration handling, UE can keep inference configuration during RRCReestablishment or when UE transits to RRC_INACTIVE state. When UE goes to RRC_IDLE, all inference configuration will be released.

Proposal 9: (RRC-6) Same as other RRC configuration, UE keeps all applicability configurations during RRCReesblishment and when UE goes to RRC_INATIVE state. UE releases all applicability configurations when UE goes to RRC_IDLE state.

However, considering there’s no urgency for network to immediately enable UE-side model for beam prediction, there’s no need to report applicability via RRCReestablishmentComplete and RRCResumeComplete.

Proposal 10: (RRC-6) Applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is not supported.

## Open issues on UE data collection request (RRC-3)

Following agreements were made for UE data collection request for UE-side model in RAN2 #129bis meeting:

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Furthermore, RAN1 agreed to configure CSI-ResourceConfigId for Set A/B and associated ID for data collection purpose:

Agreement RAN1#120 For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS

It is straightforward that the candidate data collection configuration and UE data collection request considers the above information as baseline.

Proposal 11: (RRC-3) DataCollectionPreferenceConfig (i.e., list of candidate data collection configurations) and UAI reporting of UE data collection request includes:

- CSI-ResourceConfigId of Set A

- CSI-ResourceConfigId of Set B

- One/two associated IDs (up to whether Set B is equal/subset of Set A or not)

# Conclusion

In this contribution, we further discuss the remaining open issues of AI air LCM, with following proposals and observations:

Applicability Configuration/Reporting/Cause Value

Proposal 1: (RRC-7) Upon receiving configurations from Option B, same as Option A, UE sends the initial applicability report in RRCReconfigurationComplete. UAI is used to send updated functionality applicability.

Proposal 2: (RRC-1) UE reports non-applicability cause as ‘model is not available’, optionally with when (how long) model will be available.

Co-existence/Association between Option A and Option B (RRC-8)

Proposal 3: (RRC-8) Applicability determination procedure includes both Option A and Option B. Network can provide applicabilityReportConfig for Option A and/or Option B. Co-existence of Option A (via CSI-ReportConfig) and Option B (via OtherConfig) in same RRCReconfiguration message is supported.

Observation 1: If UE only reports initial applicability reporting via RRCReconfigurationComplete but not allowed to report updates via UAI, network may wrongly activates non-applicable functionalities (initailly applicable becomes non-applicable) at UE-side.

Proposal 4: UE is allowed to report initial and updated applicability reporting consistently, that is:

- if applicabilityReportConfig is set to setup, UE considers itself to be configured to report applicability information of configurations subject to the applicability determination procedure via RRCReconfigurationComplete and UAI;

- else, UE considers itself not to be configured to report applicability information of configurations subject to the applicability determination procedure via either RRCReconfigurationComplete or UAI;

Proposal 5: Introduce applicability report configuration ID inside ApplicabilityReportConfig.

Proposal 6: (RRC-8) To support coexistence between Option A and Option B:

- During configuration, under applicableReportConfigId, network includes CSI-ReportConfigId(s) of Option A for that applicableReportConfigId, and/or inference related parameters of Option B that are associated with applicability report configuration ID.

- During reporting, UE reports the applicable/non-applicable report configuration ID in applicability reporting (via RRCReconfigurationComplete or UAI). UE further includes the applicable/non-applicable CSI-ReportConfigId(s) of Option A and/or whether Option B is applicable or not under each reported configuration ID.

Other open issues

Proposal 7: (RRC-4) If a non-applicable functionality becomes applicable and reported by UE via UAI, network activates periodic CSI reporting via a new MAC CE explicitly. FFS on the detail design of new MAC CE.

Observation 2: (RRC-5) UE can report inapplicability of its functionality immediately, since there’s no prohibit timer for UAI reporting.

Proposal 8: (RRC-5) Upon receiving UAI reporting containing inapplicable functionality, network shall release configuration of the corresponding inapplicable functionality immediately. Network receiving periodic beam prediction after inapplicability reporting will not happen.

Proposal 9: (RRC-6) Same as other RRC configuration, UE keeps all applicability configurations during RRCReesblishment and when UE goes to RRC_INATIVE state. UE releases all applicability configurations when UE goes to RRC_IDLE state.

Proposal 10: (RRC-6) Applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is not supported.

UE data collection request

Proposal 11: (RRC-3) DataCollectionPreferenceConfig (i.e., list of candidate data collection configurations) and UAI reporting of UE data collection request includes:

- CSI-ResourceConfigId of Set A

- CSI-ResourceConfigId of Set B

- One/two associated IDs (up to whether Set B is equal/subset of Set A or not)

# Reference

[1] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model