3GPP TSG RAN WG2 Meeting #130         	                          R2-2503451

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.4

Source:	Xiaomi

Title:	Discussion on RAN impact of UE-side data collection

Document for:	Discussion and Decision

# Introduction

Following way forward on UE-side data collection was agreed in RAN #106 meeting:

For SA From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilement of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5 RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now. For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed General RAN will make a final decision on way forward taking into account SA inputs when available. RAN will have a checkpoint in Q2, 2025

In this contribution, we continue discuss remaining open issues on UE-side data collection.

# Discussion

## RAN involvement in Configuration and Collection

For UE-side model data collection, RAN2 has reached the following agreement in RAN2 #129bis meeting:

- Agreements on data collection configuration

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

To our understanding, the procedure can be summarized as below:

- NW first provides a list of candidate configurations.

- UE indicates its preferred configuration based on network configuration via UAI. This indicates that:

  - Network first has the full controllability on whether UE is allowed to collect data or not by providing the corresponding configuration.

  - NW also has the full controllability to further ignore/reject UE requested preferred configuration by not configuring UE with such measurement configuration, although it is not desired from UE point of view.

- NW provides configuration for UE-side data collection. This means network has full visibility of what data is being collected by a UE based on measurement configuration.

- UE sends start indication to NW, indicating it starts to perform UE-side data collection, where network has the full visibility on when UE is performing UE-side data collection. If network doesn’t want UE to perform UE-side data collection anymore, network can always release UE-side data collection configuration directly, which means network has full controllability of when data is being collected.

Based on above, network already have full visibility and controllability on UE-side data collection configuration and when UE is performing UE-side data collection.

Proposal 1: NG-RAN has full visibility on UE-side data collection configuration (i.e., what data UE collects) and full visibility/controllability on whether UE starts/stops UE-side data collection.

## UP data transfer RAN impact

Solution 2 (First termination entity inside the CN)

According to RP-243316 and RP-242389, SA2 has started to study feasibility and candidate solutions for UP data transfer via CN. After April meeting, SA2 has identified the following architecture requirement and key issues on UP data transfer via CN [1]:

### 4.2 Architectural Requirements For the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training, the following architectural requirements shall be considered: - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - The MNO has full visibility for standardized data. - Roaming case is not supported in this release NOTE 1: The requirements to study UP Data Collection (for Option2) are documented in RAN LSs RP-243316 and RP-242389. NOTE 2: The requirements elaborated in LSs RP-242389 and R2-2411152 will be considered for investigating and evaluating potential solutions documented for this key issue. NOTE3: Full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported. For the enhancements of 5GC analytics to support efficient performance of user plane, the following architectural requirements shall be considered: - The study shall not address how UPF detects wrong or malformed or duplicated packets. - This study shall have neither RAN nor UE impacts. - Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF. - The existing UPF event exposure should be reused as much as possible. ## X.Y Key Issue #X – Transfer of data over UP for UE data collection ### X.Y.1 Description This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server. This key issue will investigate the following aspects: - How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection. - How to support in 5GC the full visibility of standardized UE data contents. - How 5GC can support the full controllability of standardized UE data transfer. - How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC. - Whether and how the UE knows about what the standardized data to be transferred are.

It is observed that SA2 will work on how to support standardized transfer of standardized data over UP, meanwhile there’s might be RAN impacts identified by SA2 during their discussion. Therefore, we suggest RAN2 to wait for SA2 conclusion on how to standardized transfer of standardized data over UP via CN.

Proposal 2: For UP data transfer via CN, RAN2 wait for SA2 conclusion on data transfer via UP, including initiation, termination, management of data transfer (PDU session establishment); differentiation between UE-side data collection and regular UE traffic (QoS); full visibility of standardized data (mapping between configuration and data content),etc.

Solution 3 (First termination entity inside the OAM)

As for UE-side data collection when first termination entity inside the OAM, following agreements were reached during RAN2 #129bis meeting:

- From RAN2 point of view UP tunnel between UE and gNB is not considered for 5GA R19/20, as currently the UP tunnel doesn’t terminate in the gNB 4 From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA Rel-19/20

According to the above agreement, it is clear that UP tunnel between UE and gNB will not be supported in 5GA Rel-19 and Rel-20. Without this UP tunnel, it is impossible to support UP data transfer between UE and OAM. Hence, we suggest to deprioritize Solution 3 via UP tunnel.

Proposal 3: UP data transfer via OAM (i.e., Solution 3 via UP tunnel) is not considered as a candidate solution in 5GA Rel-19/20.

## CP data transfer RAN impact

Using CP to transfer massive size of dataset has been extensively discussed and RAN2 has not reached consensus on its feasibility:

6 For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility: - OTA solution 1a: gNB -> UE via CP - OTA solution 2: CN -> UE via gNB - OTA solution 3: OAM -> UE via gNB

Though NW dataset/model parameter was discussed under the assumption that CP is used for DL dataset transfer, the same constraint on RRC message size also applicable to UL, as UL RRC only support 5 more segments than DL RRC. Therefore, in our understanding, RAN2 also has no consensus on whether data transfer over CP for UE-side data transfer with massive size is feasible or not. Therefore, we suggest to deprioritize discussion on Solution 2/3 via CP.

Observation 1: Restriction using CP for massive data size transfer is the same for UL and DL is the same as OTA solutions for two-sided model NW dataset/model parameter transfer.

Proposal 4: RAN2 deprioritize UE-side data transfer Solution 2 and Solution 3 via CP, as it’s not scalable and no feasibility consensus in RAN2.

## Reuse of NW-side data collection

For the same use case, data collected for NW-side model and for UE-side model may be the same. Both NW-side model and UE-side model require UE to perform measurement on the network configured CSI-ResourceConfig. For example, in beam management use case, RAN1 agreed CSI-ReportConfig can be used for configuring resources for UE-side data collection.

Agreement RAN1#120 For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B FFS: whether/how to support 'aperiodic' CSI RS

RAN2 also discussed directly/indirectly configure UE with CSI-ResourceConfig for NW-side data collection via either L1 or L3 framework. From UE point of view, UE only needs to perform corresponding measurement on the configured CSI resources, no matter it is for NW-side model or UE-side model. This also means, standardized data collection (on measurement) is use case specific. The standardized data collection can be used for any purpose, no matter where model inference is performed.

Observation 2: Data collection is use case specific, standardized data collected by UE for NW-side model and UE-side model can be the same.

During RAN2 #129bis meeting, how to support two-sided model dataset and model parameter transfer was discussed and non-OTA solutions has been concluded as feasible candidate from RAN2 point of view.

3. For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates. From RAN2 point of view it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can confirm this assumption (i.e. we can send an LS). It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

It is observed that UE-side OTT server/training entity is the common data transfer termination for both two-sided model dataset and/or model parameter and UE-side data collection. This means, any data collected for NW-side data collection can also leverage this tunnel and be transferred to UE-side OTT server for UE-side model training. Therefore, for standardized data which are collected by UE based on network measurement configuration, NW-side data collection can be further transferred to UE-side OTT server for UE-side model training.

Proposal 5: For standardized data, data collection reported by UE for NW-side data collection can be transferred to UE-side OTT server for UE-side model training. Non-OTA solution for NW-side dataset and model parameter transfer for two-sided model can be reused.

# Conclusion

In this contribution, we discussed remaining RAN impact of UE-side data collection and provide the following observations and proposals:

Visibility/Controllability

Proposal 1: NG-RAN has full visibility on UE-side data collection configuration (i.e., what data UE collects) and full visibility/controllability on whether UE starts/stops UE-side data collection.

RAN impact

Proposal 2: For UP data transfer via CN, RAN2 wait for SA2 conclusion on data transfer via UP, including initiation, termination, management of data transfer (PDU session establishment); differentiation between UE-side data collection and regular UE traffic (QoS); full visibility of standardized data (mapping between configuration and data content),etc.

Proposal 3: UP data transfer via OAM (i.e., Solution 3 via UP tunnel) is not considered as a candidate solution in 5GA Rel-19/20.

Observation 1: Restriction using CP for massive data size transfer is the same for UL and DL is the same as OTA solutions for two-sided model NW dataset/model parameter transfer.

Proposal 4: RAN2 deprioritize UE-side data transfer Solution 2 and Solution 3 via CP, as it’s not scalable and no feasibility consensus in RAN2.

Observation 2: Data collection is use case specific, standardized data collected by UE for NW-side model and UE-side model can be the same.

Reuse of NW-side data collection

Proposal 5: For standardized data, data collection reported by UE for NW-side data collection can be transferred to UE-side OTT server for UE-side model training. Non-OTA solution for NW-side dataset and model parameter transfer for two-sided model can be reused.

# Reference

[1] S2-2504235 New Key Issue for transfer of data over UP for UE data collection; Vivo(rapporteur), Samsung(rapporteur)

[2] RP-242389, LS on AIML data collection