3GPP TSG-RAN WG2 Meeting #130	R2-2503459

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on the need of cell (re)selection enhancements for DL CE

Document for: 	Discussion and Decision

Introduction

This contribution further discusses "Open issue idle/inactive-1" in [1]:

Open issue idle/inactive-1: RAN2 can further discuss whether and how to enhance cell (re)selection procedure due to DL CE Issue description: RAN2#127bis Agreements - (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE..

Discussion

For the Idle/inactive open issue-1 as cited above [1], the intention of the cell reselection enhancement to down-prioritize/prevent reselect to DL CE cells is to save UE power during measurements. We consider this cell (re)selection enhancement is an optimization which is not super critical. We provide our analyses on this issue as follows.

In past meetings, RAN2 analyzed and discussed whether extending the default SSB periodicity is backward compatible. It has been proved that extended SSB periodicity only impacts the initial cell search for the NTN cell in DL CE. Upon successful camping on such an NTN cell, the UE can work normally, as the existing configurations for SI/paging/RACH does not need to be extended as we analysed in [2] (section “2.1.2	Impact on RRC procedure”). Regarding the SMTC enhancement, it should be an optional feature for DL CE support. UEs without DL CE support could still perform measurement based on legacy smtc4list after camping on a DL CE cell. This is also aligned with the proposal in [3] to use a BC way for the SMTC extension:

Proposal 1        If a new SMTC periodicity is finally adopted and included in system information broadcast, RAN2 introduces the support in a backwards compatible manner. FFS signalling details (e.g., new field or extension of SMTC4

Additionally, in RAN2#129bis, it was agreed that introducing new barring bits from SSB extension perspective is not needed.

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

Since UE not supporting SSB extension can work well in a cell with SSB periodicity extended, it is questionable whether it is really necessary to allow these UEs to down-prioritize/prevent re-selection to cells operating with DL CE. Therefore, we propose to not pursue any cell reselection enhancement due to this DL CE feature, at least from the perspective of SSB extension (i.e. system level DL CE).

Proposal 1: At least from SSB periodicity extension point of view, RAN2 confirms to not further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Since whether link-level DL CE mechanisms in RAN1 leads to NBC issues cannot be determined by now, whether such cell (re)selection enhancement due to link-level DL CE may be further determined after the BC aspects can be decided based on further RAN1 progress.

Conclusion

In this contribution, need of cell (re)selection enhancements for DL CE is further discussed with the following proposal:

Proposal 1: At least from SSB periodicity extension point of view, RAN2 confirms to not further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Reference

- R2-2504078	Remaining 304 open issues for NR NTN

- R2-2503458	Discussion on downlink coverage enhancements	CATT

- [Post129bis][305][R19 NR NTN] RRC CR (Ericsson)	Remaining RRC open issues for NR NTN Rel-19