3GPP TSG-RAN WG2 Meeting #130	R2-2503460

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Further discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

Based on current progress and agreements [1], this contribution further discusses the service continuity aspects.

Discussion on service continuity enhancements

In the past meetings, MBS service continuity enhancements in the context of NTN were also discussed. Based on the progress of last meetings, the following further discuss possible specification impacts on aspects of service continuity.

Potential enhancements on service area for FSAI

In last meeting, it was agreed that the UE may (de-)prioritize the frequency(ies) based on whether the UE is in the associated intended service areas. While it is still FFS on whether and how the association between intended service area and frequencies.

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. - FFS whether and how the association between intended service area and frequencies is acquired.

In RAN2#129bis, there was an initial discussion regarding the FFS, but no consensus was reached [2]. Companies mainly diverge on which of the following options to choose.

- Option 1: Adding intended service area for FSAI in SIB21 [1][2][3][5][7][8][9][12] - Possible spec change is shown in Annex, from [12] - Option 2: no intended service area in SIB21[4][11][13][14][15] - UE rely on USD info combined with FSAI/freq info in SIB21

We have a further consideration on this issue but found no marginal benefits on option 1, i.e., introduce intended service area for FSAI in SIB21. Option 2 is sufficient for UE determines (de-)prioritize which frequency(ies).

- Why not add intended service area for FSAI?

Firstly, it is hard to introduce the service area for FSAI into SIB21 based on the current signalling structure. According to TS 38.331 [3], the FSAI list in SIB21 is configured in a per frequency manner.

SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 }

In addition, according to TS 29.571 [4], one MBS session is associated with a list of FSAI, and one FSAI is allowed to be associated with multiple MBS sessions.

- 5.9.4.6 Type: MbsSession Table 5.9.4.6-1: Definition of type MbsSession Attribute name; Data type; P; Cardinality; Description. mbsSessionId; MbsSessionId; C; 0..1; MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE 1). [unrelated part is omitted]. mbsFsaIdList; array(MbsFsaId); O; 1..N; List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. This attribute may be present if the "serviceType" attribute indicates a broadcast MBS session..

Above quotation implies that, if service areas were to be introduced specifically for FSAI in SIB21, the service area should be provided per FSAI and per MBS session. This brings excessive signalling overhead for the system information considering the SI size limit, making such signalling design unacceptable.

Observation 1: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable due to the SI size limitation concern.

- How it works relying on USD info and existing FSAI/freq info in SIB21

In RAN2#128, it was agreed that the intended service area may include geographic areas across the current serving cell and overlapping neighbour cell(s).

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

That is, the UE could obtain the service area info of an MBS session which covers not only the service areas of the serving cell but also those of the neighbour cells. Combining the info in USD and SIB21, the UE can determine whether the UE can obtain the service from neighbour cells [6]. Specifically:

- For a certain MBS session, the UE acquires the FSAI list of an MBS session ID via upper layer procedure [4][5], and acquires related frequencies of FSAI via SIB21. This means, the UE, can know the frequencies applicable for each MBS session based on current USD and SIB21.

- Based on the intended service areas associated with the MBS session, the UE can know the intended service area within the scope of both serving cell and neighbour cells for each MBS session.

Then, the UE could determine whether it may prioritize the frequency(ies) associated with its intended service area (as per bullet "i"), based on whether it is still located within its intended service as determined by its current GNSS location and bullet "ii". That is, the UE may further prioritize a frequency(ies) in SIB21 for its interested service, if it finds itself located in the intended service area. The information in the exiting USD and the introduced service areas info in new SIB are already sufficient, and additional ASN.1 impact is not needed.

In [2], company questions how the UE can differentiate whether f2 supports TMGI #x at intended service area #1 in the scenario as cited below. However, this issue is flawed in its scenario assumption. For a certain MBS session with TMGI#x: if its intended service area includes service area #1 (which is covered by service cell and neighbour cell in the figure), both the serving cell and the neighbour cell should broadcast the MBS session with TMGI#x in the service area #1 with their frequencies. Therefore, when UE is at the edge serving cell (in intended service area #1), it is reasonable for UE to prioritize f2, allowing it to reselect to the neighbour cell providing the MBS session in the intended service area #1 with f2. In a word, we do not think the scenario that only f1 serves intended service area #1 as in the below figure actually holds.

Figure 1: Scenario discussed in [2]

Based on the analysis on these two options, we have the proposal as follows:

Proposal 1: Do not introduce service area information specific for FSAI in SIB21.

Proposal 1a: Information in existing SIB21 and USD in the current Specs is already sufficient for the UE to determine the frequency(ies) associated with the intended service area and perform the frequency (de)prioritization operations as agreed in RAN2#129bis.

Introduction of service area for neighbour cell list in MCCH

Some companies proposed to introduce service area specific for neighbour cells in MCCH, so that the UE could determine whether to request unicast before reselecting to a neighbour cell.

On the one hand, introducing service area for the neighbour cell will lead to potentially excessive signalling overhead, especially if the service areas are contained in SIB. On the other hand, the intention raised by company can also be realized by implementation without ASN.1 impact. In detail, for a certain MBS session, its intended service area may cover geographic areas across the current serving cell and overlapping neighbour cell(s) as per previous meeting agreements. Based on this, the UE could determine whether a neighbour cell is providing this MBS session based on its GNSS location. If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE considers the neighbour cell providing this MBS session. No need to further introduce per neighbour cell service area distinction.

Observation 2: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Also, we think whether/how the UE considers the service area related configuration in MCCH in addition to the neighbour cell list in MCCH can still be left to UE implementation, when it determines whether the reselected cell providing its interested MBS session at its location.

Proposal 2: Do not introduce service area per neighbour cell in MCCH.

Proposal 2a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

Conclusion

In this contribution, support of broadcast service via NR NTN is discussed with the observations and proposals listed as follows:

Observation 1: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable due to the SI size limitation concern.

Observation 2: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Proposal 1: Do not introduce service area information specific for FSAI in SIB21.

Proposal 1a: Information in existing SIB21 and USD in the current Specs is already sufficient for the UE to determine the frequency(ies) associated with the intended service area and perform the frequency (de)prioritization operations as agreed in RAN2#129bis.

Proposal 2: Do not introduce service area per neighbour cell in MCCH.

Proposal 2a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

Reference

- RAN2_129bis_Meeting_Report

- R2-2503057	Report of [AT129bis][304][R19 NR NTN] Service continuity	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

- 3GPP TS 38.331: "NR: Radio Resource Control (RRC) protocol specification".

- 3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- 3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".