3GPP TSG-RAN WG2 Meeting #130					R2-2503464

Malta, May 19–23, 2025

Source:	Continental Automotive

Title:                       Discussion on Applicability Reporting

Agenda Item:         8.1.2.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#129-bis meeting, the following agreements were made for applicability reporting [1].

All agreements for applicability reporting made during RAN2#129-bis are:

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

In the RAN2#129-bis meeting, an agreement was made to further study how to define a simple cause related to model availability and how to capture it in the specification.

# Discussion

In the RAN2#129-bis meeting, an agreement was made to further study how to define a simple cause related to model availability and how to capture it in the specification. We would like to emphasise the importance of reporting the cause of model inapplicability to the network that it helps the network to understand why a model became inapplicable. To address this, we propose a hierarchical bitmap representation introduces a structured approach to categorize inapplicability causes into three levels: scope, entity side, and detailed cause. This method ensures a comprehensive and efficient way to communicate the reasons for model inapplicability from the UE to the network.

The scope is defined in 1-bit, indicating whether the inapplicability is temporary or permanent. This allows the network to quickly understand the duration of the issue. The entity side is defined in 2-bits, specifying whether the issue is on the UE-side, NW-side, due to metadata/configuration issues, or reserved for future use. This categorization helps in pinpointing the source of the problem. The detailed cause is defined in 4-bits, providing specific reasons for inapplicability such as low battery, buffer status full, compute core unavailable, etc. This granularity ensures that the network receives precise information about the issue.

The UE signals the bitmap using L1/L2/L3 whenever it determines the model is inapplicable. Upon receiving this indication, the network can disable or reconfigure the UE side model parameters accordingly. This signalling mechanism ensures that the network can take timely and appropriate actions based on the inapplicability causes reported by the UE.

In addition to the hierarchical bitmap, a lightweight ML model standardized on the UE is proposed to pre-empt the applicability of a model before it is configured by the network. This model considers the UE context such as CPU load, memory usage, battery level, etc., as well as network state such as AI model policy (privacy, etc.), gNB configuration, slice profile, etc. The model also considers model metadata such as size, execution time estimate, hardware dependency, etc.

The UE may report the inference before or during the model application, being proactive during model applicability reporting, or reactively after model applicability failure. The pre-empted inference can be signalled to the network via L1/L2/L3 signalling. The UE may initiate the inference upon receiving the model activation signalling from the network, periodically, or as a failure fallback.

The UE signals the bitmap using L1/L2/L3 whenever it determines the model is inapplicable. This ensures that the network is promptly informed about the inapplicability causes. Upon receiving the indication, the network can disable or reconfigure the UE side model parameters to address the issue. This mechanism ensures efficient communication and timely resolution of inapplicability issues.

The lightweight ML model on the UE can significantly improve the pre-emption of model applicability by considering various factors such as UE context, network state, and model metadata. The UE context includes parameters like CPU load, memory usage, and battery level. The network state encompasses AI model policy (privacy, etc.), gNB configuration, and slice profile. Model metadata includes size, execution time estimate, and hardware dependency.

The UE may report the inference before or during the model application, being proactive during model applicability reporting, or reactively after model applicability failure. The preempted inference can be signaled to the network via L1/L2/L3 signaling. The UE may initiate the inference upon receiving the model activation signaling from the network, periodically, or as a failure fallback.

Proposal 1: The network may configure a hierarchical bitmap representation for the UE to indicate model inapplicability causes. This bitmap will include:

- Scope: 1-bit (temporary or permanent inapplicability)

- Entity Side: 2-bits (UE-side, NW-side, metadata/config issue, reserved)

- Detailed Cause: 4-bits (low battery, buffer status full, compute core unavailable, etc.)

Proposal 2: The UE should signal the bitmap using L1/L2/L3 whenever it determines the model is inapplicable. Upon receiving the indication, the network will disable or reconfigure the UE side model parameters.

Proposal 3: A lightweight ML model standardized on the UE should be used to pre-empt the applicability of a model before it is configured by the network. This model should consider the UE context, network state, and model metadata. The UE may report the inference proactively or reactively and signal the pre-empted inference to the network via L1/L2/L3 signalling.

# Conclusions

In this document, we propose a hierarchical bitmap representation for applicability reporting and the use of a lightweight ML model for pre-empting model applicability. This method ensures efficient communication between the UE and the network regarding model inapplicability causes and pre-emptive inference. Additionally, we request RAN2 to discuss the following proposals:

Proposal 1: The network may configure a hierarchical bitmap representation for the UE to indicate model inapplicability causes. This bitmap will include:

- Scope: 1-bit (temporary or permanent inapplicability)

- Entity Side: 2-bits (UE-side, NW-side, metadata/config issue, reserved)

- Detailed Cause: 4-bits (low battery, buffer status full, compute core unavailable, etc.)

Proposal 2: The UE should signal the bitmap using L1/L2/L3 whenever it determines the model is inapplicable. Upon receiving the indication, the network will disable or reconfigure the UE side model parameters.

Proposal 3: A lightweight ML model standardized on the UE should be used to pre-empt the applicability of a model before it is configured by the network. This model should consider the UE context, network state, and model metadata. The UE may report the inference proactively or reactively and signal the pre-empted inference to the network via L1/L2/L3 signalling.

# References

[1] RAN2#129-bis meeting chair notes.