3GPP TSG-RAN WG2 Meeting #130	 R2-2503468

St Julian’s, Malta, May 19-23, 2025

Agenda item:	8.5.4 Discussion on Adaptation of common signal channel transmissions

Source:	NEC

Title:	PRACH, paging adaptation for NES

Document for:	Discussion, Decision

# Introduction

Below is mentioned regarding the agreements on RACH adaptation in RAN2#129,

1.	Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

4.	From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

6.	Will follow legacy mechanism regarding how to select RACH resource.

Below is mentioned the agreements according to RAN2#129bis

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In this contribution, we discuss the latest agreements in RAN2 #129bis and how they impact specification work going forward.

# Discussion

## PRACH adaptation

In the current configuration, legacy UEs monitor predefined PRACH occasions, which may or may not be shared with Rel-19 NES-capable UEs based on network settings. It is assumed that Rel-19 NES-capable UEs can access both NES-specific and legacy PRACH resources. However, to preserve legacy UE’s performance and improve resource efficiency, NES UEs should be configured to prioritize the use of NES-specific PRACH resources. Legacy PRACH resources may still be available for use, but their use by NES UEs can be deprioritized based on network conditions and demand levels. This prioritization mechanism allows the network to manage PRACH load effectively, particularly under a greater number of NES UEs requiring services, while avoiding contention with legacy UEs.

Proposal 1: Rel-19 NES-capable UEs should be configured to prioritize NES-specific PRACH resources, while maintaining its capability to access legacy PRACH resources by network configuration, to avoid less impact on legacy UE operations.

# Conclusion and Proposal

We have the following proposal:

Proposal 1: Rel-19 NES-capable UEs should be configured to prioritize NES-specific PRACH resources, while maintaining its capability to access legacy PRACH resources by network configuration, to avoid less impact on legacy UE operations.

# Reference

- RAN1#116 meeting minutes

- RAN1#119 meeting minutes

- RAN2#129 meeting minutes

- RAN2#129bis meeting minutes

- TS38.321

- TS38.304