3GPP TSG-RAN WG2 Meeting #130	R2-2503480

Malta, 19th – 23rd May 2025

Agenda Item:	8.2.2

Source: 	Xiaomi, Huawei, CMCC, Spreadtrum, CATT, Apple, China Telecom, Transsion Holdings, vivo, InterDigital, Philips International B.V., Qualcomm Incorporated, NTT DOCOMO, INC.

Title:  	Device behavior for parallel service requests (Joint paper: Option B - no further standard effort, issue 1-1)

Document for:	Discussion and decision

# Introduction

RAN2 has discussed the issue “Device behavior if it gets a new service request while one procedure is still ongoing” for several meetings. However, companies still have different understanding on the scenario, and the solutions [1], [2].

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request.

In the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei), A-IoT MAC CR Rapporteur also listed it as open issues for Paging:

| Issue 1-1: multi-reader paging | If a device gets a new service request while one procedure is still ongoing, whether/how to specify device behaviour or leave it to implementation, and the end of procedure if needed. - Relevant agreements: - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - FFS on end of procedure - Status in running CR: captured as Editor’s Note in 5.2. |
| --- | --- |

RAN2 shall make decision in May meeting considering only two meetings left before the completion of WI.

In this contribution, we analyse the scenario, details of option a and b, and the view on how to handle the issue.

# Discussion

## 2.1 Scenario on parallel service requests

Based on the discussion in previous RAN2 meeting, companies have different understanding on whether parallel service requests could happen, and whether it exists for single reader, multiple readers scenarios.

It would be good to align companies’ understanding from procedure perspective first.

RAN2 has agreed to introduce a new R2D message to indicate the timing of resources for MSG1 transmission, RAN1 also agreed that support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device.

We illustrate the timing relationship between R2D and D2R messages in figure 1:

Figure 1: Timing relationship for a session (T=1)

Based on RAN1 agreements, the timing relationship requirements between R2D and D2R messages are:

- The device 1 transmits Msg 1 based on Toffset1 upon receiving Paging message or New R2D message. (FFS on TDM MSG1 resource, where the second MSG1 in time domain start transmitting time is discussed in RAN1.)

- Even if there is R2D message from another Reader, the device 1 is not required to receive R2D message in Toffset1 +D2R transmission time+ TD2Rmin, considering at least the device processing time including FEC impact and A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. It will only cause the interference.

- Same as Msg 1, the device 1 transmits Msg 3 based on the timing of Msg 2. The device processing time requirement shall be considered for the scheduling of Msg 3. The device is not required to receive R2D Msg earlier than TD2R_min after the end of a D2R Msg 3 transmission.

It should be a bad reader implementation if it handles R2D Msg2 across different slots since it will increase the latency of the procedure for the devices, and it will also increase the size of Msg2. Besides, since every D2R transmission is scheduled by its corresponding R2D transmission, even for MSG3 transmission, the device turns to the Tx mode after receiving MSG2, it is not possible for device to receive another R2D message in between. Therefore, for the normal case, the reader shall complete the Inventory procedure of one A-IoT device in the same access slot (rather than across R2D trigger messages).

Observation 1: For the normal case, the reader should complete Inventory procedure for a device within the same access slot (rather than across R2D trigger messages).

Observation 2: During inventory procedure, the device is not required to receive R2D message other than the messages for this Inventory procedure if the reader follows the timing relationship of R2D and D2R message strictly.

The next question is for a device whether Command procedure can also be completed in the same Access slot, together with Inventory procedure?

According to TS23.369, the AIOTF will send Command once the Inventory response is received. That means the reader may not be able to transmit command right after D2R Msg 3 since it may not be available in the reader. It is also difficult to predict the processing time for AIOTF since the AIOTF may treat set of device IDs together. To make the procedure more efficient, the reader may transmit the command across different access slot or even different paging round.

10. NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s). 8. For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

The third question is whether the reader can transmit command for a device during the period of another session.

RAN2 already agreed that

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

During offline discussion, some companies think that the agreements means, only parallel service requests by the same reader for same device is not supported.

Figure 2: Interleaving procedure from different readers (interleaving on inventory procedure)

To our understanding, there is no additional benefit for a reader to support interleaving procedure for the different group of devices of different session/service considering so far, there is no FDMA for R2D message, the reader has to transmit them one by one even they are for different group of devices. In addition, for group ID case, the reader has no idea which device will be addressed. Therefore, the agreements shall also be applicable for different device case, i.e. the reader shall complete the current session/service including the command procedure before triggering another session/service. To achieve this purpose, one way is to have the restriction on the reader, i.e., the reader should not trigger the new session/service before completing the procedure for the previous session. Another way is to have the restriction on the AIOTF, i.e., the reader should not trigger the new session/service before a reader completes the procedure for the previous session.

Observation 3: The reader can trigger the command procedure for the same device in different access slot or even different paging round, but it should complete the command transmission of current session/service before triggering another session/service.

Proposal 1: RAN2 confirms that for CBRA, the parallel service requests by the same reader is not supported no matter to the same set of devices or different devices.

Multiple Reader scenario:

If readers with overlapping coverage transmit the R2D Msg without the coordination, the device which is in Inventory procedure either cannot decode the R2D Msg or considers it as interference. For A-IoT devices studied in R19 (i.e. device 1), they are limited by their own physical capabilities, and such interference issue cannot be overcome even in future release. RAN2 has discussed this issue at RAN2 129 and concluded to rely on implementation.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Observation 4: For overlapping among multiple readers scenario, the system does not work for both option a and b if there is no coordination between Readers on the timing of R2D transmission. However it can be left to reader’s implementation.

Figure 3 and 4 illustrates for overlapping among multiple readers scenario, Readers for some reason, decide to transmit the new trigger Message in the interleaving way, and still follows the timing relationship between R2D and D2R.

Figure 3: Interleaving procedure from different readers (interleaving on inventory procedure)

Figure 4: Interleaving procedure from different readers (no interleaving on inventory procedure but interleaving on command procedure)

If the readers can follow the timing relationship very well, the system can work. However it cannot reduce the latency since the total procedure/steps are same, and therefore the required minimum total latency is same as well considering the time relationship requirement shall be followed. It will increase the complexity and additional overhead for both reader and device.

Reader:

- Needs to define the strict rule for coordination between readers or dynamic coordination scheme between readers;

- Additional overhead for New R2D trigger message to distinguish Reader or transaction;

- No additional ID needed in R2D Msg if there is no collision on AS ID;

Device:

- Determination of access slot from different reader;

Note: as analyzed in the section 2.2, if parallel procedure happens, one of the procedure will be impacted for both option a and b. Therefore the resource utilization efficiency is really low.

Observation 5: For overlapping among multiple readers scenario, the system can work for both option a and b if readers follow the timing relationship among readers for R2D, D2R message transmission even if parallel session/service happens. However additional complexity is foreseen without significant benefit, e.g., on latency.

Proposal 2: For coverage overlapping among multiple readers scenario, the readers implementation still needs to follow the physical layer restrictions (e.g., timing relationship), i.e., readers coordination via implementation is inevitable in any case.

In the section 2.2, we analyze the impact from both reader and device side for no interleaving, interleaving on command procedure, interleaving on Inventory procedure.

## 2.2 Comparison between option a and option b

The definition of option a/b was discussed in [1], and the issues of option a/b was discussed in [2]. We copied them here for reference.

[1] The device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. Option a related issues: Discuss and decide how a device determines a procedure is ‘ongoing’ or not. Discuss and decide how a device determines whether a R2D message from Reader is associated with the ongoing procedure or not.

Issues for each option [2]:

| Issues for option a | Issues for option b |
| --- | --- |
| For option a (i.e. device ignore new messages while a procedure is ongoing), according to online and offline comments, a potential deadlock scenario can occur when, e.g. - Reader went through a power cycle/failure while devices are in the middle of a procedure. Any new paging from the reader is ignored while the device keeps waiting for the procedure to finish. - Reader sent a NACK to trigger re-access, however the device missed the NACK, hence ignores new paging messages. - Reader legitimately intends to stop the ongoing procedure and send a new service request (e.g. Reader wants to stop ongoing inventory after some time, but some devices has not finished the procedure; reader wants to perform a different service which targets the same device.) Some potential solutions to address the above were raised during the online discussion, such as: - Timer based autonomous release by the device. - Concerns: supporting timer for such low complexity devices increases complexity. Hard to find exact useful value of the timer. - Explicit release (RRC-release-like) message by the Reader to all the devices. - Concerns: new message needed to be defined; may not be possible to indicate release to specific devices and release-all-devices-within-reader-range may not be desirable. This new message may also be lost. - Turn off CW source and force the devices to drain the energy. - Concerns: this assumes Reader has control over the CW source. Forcing the device to lose all of the volatile memory and trigger power-cycle may not be ideal. This would impact all the devices under coverage. Pure NW implementation may not be sufficient. | Forward compatibility For forward compatibility, RAN2 aims to design Rel-19 AIoT messages extensible to accommodate devices and features of future release. |
| Rapp Summary: If device needs to be able to determine whether a R2D message is associated to an ongoing procedure or not, some kind of indication (e.g. transaction ID, reader ID, some other ID) would be needed in the R2D messages. | DISCUSSION point 5: With option b, does the R2D messages (other than paging message) need to include transaction ID or are they not needed? |
| In option a, the device is expected to ignore any new request while a procedure is ‘ongoing’. This means there needs to be clear understanding on what ‘end of procedure’ means from the device point of view. |  |

Based on offline discussion, for option a, companies who support option a have different view on how to define “ongoing”, and how to resolve the “deadlock”, e.g., timer based, explicit release based, implementation based, etc. Obviously, option a is not a “complete” solution. If we look into the details, there will be multiple different “option a” solutions.

Observation 6: Based on offline discussion in last meeting, option a is not a “complete” solution. Companies have different view on the details, e.g., “ongoing”, “deadlock”.

Regarding the Forward compatibility issue, RAN2 has agreed that 	RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

For Rel-20 A-IoT, so far Topology 2, device 2b/c, DO-A are under the discussion.

| Feature | Potential impact on Rel-19 devices | Forward compatibility/backward compatibility issues |
| --- | --- | --- |
| Topology 2 | Transparent to device 1 since the assumption in RAN2 is that From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. | N/A |
| Device 2b/c/DO-A | 1 device 2b/c is mainly for out door scenario which does not work for device 1 2 The timing relationship requirement defined in Rel-19 for device 1 may not be applicable for Device 2b/c. | If the reader of device 2b/c for out door scenario also wants to support device 1, backward compatibility issue is more serious than forward compatibility since there is strict timing relationship requirement for device 1 in Rel-19, i.e., Rel-20 Reader and device 2b/c shall not impact the timing of R2D/D2D transmission for Rel-19 devices. Note: backward compatibility is always the most important factor to be considered when a new feature is introduced in a system with legacy UE and network. |

Observation 7: To support Rel-20 A-IoT, message extensible based forward compatibility is sufficient in Rel-19. Backward compatibility issue shall be considered in Rel-20, e.g., not impact timing relationship requirements for Rel-19 R2D, D2R message transmission if a Rel-20 reader would like to support both device 1 and device 2b/c simultaneously.

Table I. Option a vs Option b in Multi-reader Overlapping Coverage Scenarios

|  | Option a (spec impact) (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure | Option b (spec impact) (b) terminate the ongoing procedure and respond to the latest request. | Remark |
| --- | --- | --- | --- |
| No coordination | - | - | The system does not work and does not follow RAN4 coexistence requirement strictly |
| No interleaving on Inventory and Command Procedure at all | 1 Additional solution on deadlock issue, e.g., end/release indication is needed to indicate the end of the session | N/A | No solution is needed for Option b. |
| No Interleaving on Inventory procedure, but interleaving on command procedure | 1 Additional ID would be needed in the R2D command messages if we want device to address AS ID collision issue, i.e., if two readers assign same AS ID for the same device. 2 Additional end/release mechanism is needed to indicate the end of the session 3 Additional mechanism is need to resolve the issue of end/release message loss. Consequence: 1 the paging of the second session will be dropped if the device has not completed the first session while the paging of the second session arrives; Note: it may be resolved by the reader of the second session to send more subsequent paging messages. | 1 Additional ID would be needed in the R2D command messages if we want device to address AS ID collision issue, i.e., if two readers assign same AS ID for the same device. Consequence: 1 the command will be dropped if the paging of second session is for the device and arrived before or during command procedure for the first session since AS ID of the first session will be released. Note: it may be resolved by the AIOTF to trigger an CFRA-based inventory and command procedure for the device when the session from another reader is completed. | We may not need additional ID in R2D message considering the AS ID collision ratio should be quite low. If companies want to address this issue, it can be handled by implementation, assuming there is coordination among Reader. Note1, there is no session loss issue if the sessions triggered by readers are same or the sessions are different but not for the same device since anyway the device needs to drop the second one. Note2: For both options, the resource utilization efficiency is low since the device may skip some messages, especially when many devices are involved. |
| Interleaving on Inventory procedure | 1 Additional ID is needed in new trigger message to indicate the slot boundary for each session 2 Additional ID would be needed in the R2D command messages if we want device to address AS ID collision issue, i.e., if two readers assign same AS ID for the same device. 3 Additional end/release mechanism is needed to indicate the end of the session 4 Additional mechanism is need to resolve the issue of end/release message loss. Consequence: 1 the paging of the second session will be dropped if the device has not completed the first session while the paging of the second session arrives; Note: it may be resolved by the reader of the second session to send more subsequent paging messages or handled by the AIOTF. | 1 Additional ID is needed in new trigger message to indicate the slot boundary for each session 2 Additional ID would be needed in the R2D command messages if we want device to address AS ID collision issue, i.e., if two readers assign same AS ID for the same device. Consequence: 1 the paging of the first session will be dropped if the paging of second session is for the device and arrived early than the selected access occasion for the first session; 2 the command will be dropped if the paging of second session is for the device and arrived after the selected access occasion of the first session but before or during command procedure for the first session Note: it may be resolved by the AIOTF to trigger an CFRA-based inventory and command procedure for the device when the session from another reader is completed. | We may not need additional ID in R2D message considering the AS ID collision ratio should be quite low. If companies want to address this issue, it can be handled by implementation, assuming there is coordination among Reader. Note1, there is no session loss issue if the sessions triggered by readers are same or the sessions are different but not for the same device since anyway the device needs to drop the second one. Note2: For both options, the resource utilization efficiency is low since the device may skip some messages, especially when many devices are impacted. |

Based on above analysis, we have following observations:

Observation 8: For coverage overlapping among multiple readers scenario, the resource utilization efficiency is really low for both option a and option b if interleaving service is allowed, especially when many devices are impacted.

Observation 9: For coverage overlapping among multiple readers scenario, session loss issue does not exist if the sessions triggered by readers are same or the sessions are different but not for the same device even if the interleaving is allowed.

Observation 10: For coverage overlapping among multiple readers scenario, the differences between option a and b are whether to define solution for “ongoing” and “deadlock” if the interleaving is allowed.

Based on the above table, Option a brings more spec impacts and Option b is simpler.

Note that there will be error case(s) no matter whether option a or option b is supported. Resource utilization efficiency issue is applied for both option a and option b if parallel session/services are triggered for the same device.

For the error case caused by the use of option b, it only affects devices in overlapping coverage which is a minority portion of the devices. For interleaving case, the worst scenario for option b is that a device in overlapping coverage will be driven by the most recent paging message from another reader, but this only adds some latencies. As long as the device choose an AO slot earlier than the alternative paging message, then the inventory can succeed with this device. For the skipped session/service, we can rely on AIOTF to re-trigger the procedure.

For option a, the new session/service will be skipped, and has to be re-triggered by the reader or AIOTF. To make it work, we have to resolve the deadlock issue and define “ongoing”, otherwise the device would stick with a reader with poor UL performance and never been successfully inventoried. Further enhancements are needed, e.g., end indication, which affects all devices (non-overlapping device and overlapping devices). In addition, it is difficult to reach consensus among companies who support solution a, since there are too many options on how to determine “ongoing”, how to resolve the deadlock. We do not think it can be completed in two meetings within Rel-19.

Observation 11: For deadlock issue of option a, it affects all devices (non-overlapping device and overlapping devices).

Considering the discussion and observations above, we should go for a solution with less specification impact and less implementation efforts. Therefore, we prefer the option B.

Proposal 3: The device always respond to the new service indicated by the received paging message, regardless whether there is any procedure ongoing, i.e., no specification impact, same as the description in the current section “Upon receiving the A-IoT Paging message” in the MAC running CR.

# Conclusions

Based on the above discussion, we propose following proposals:

Observation 1: For the normal case, the reader should complete Inventory procedure for a device within the same access slot (rather than across R2D trigger messages).

Observation 2: During inventory procedure, the device is not required to receive R2D message other than the messages for this Inventory procedure if the reader follows the timing relationship of R2D and D2R message strictly.

Observation 3: The reader can trigger the command procedure for the same device in different access slot or even different paging round, but it should complete the command transmission of current session/service before triggering another session/service.

Observation 4: For overlapping among multiple readers scenario, the system does not work for both option a and b if there is no coordination between Readers on the timing of R2D transmission. However it can be left to reader’s implementation.

Observation 5: For overlapping among multiple readers scenario, the system can work for both option a and b if readers follow the timing relationship among readers for R2D, D2R message transmission even if parallel session/service happens. However additional complexity is foreseen without significant benefit, e.g. on latency.

Observation 6: Based on offline discussion in last meeting, option a is not a “complete” solution. Companies have different views on the details, e.g., “ongoing”, “deadlock”.

Observation 7: To support Rel-20 A-IoT, message extensible based forward compatibility is sufficient in Rel-19. Backward compatibility issue shall be considered in Rel-20, e.g., not impact timing relationship requirements for Rel-19 R2D, D2R message transmission if a Rel-20 reader would like to support both device 1 and device 2b/c simultaneously.

Observation 8: For coverage overlapping among multiple readers scenario, the resource utilization efficiency is really low for both option a and option b if interleaving service is allowed, especially when many devices are impacted.

Observation 9: For coverage overlapping among multiple readers scenario, session loss issue does not exist if the sessions triggered by readers are same or the sessions are different but not for the same device even if the interleaving is allowed.

Observation 10: For coverage overlapping among multiple readers scenario, the differences between option a and b are whether to define solution for “ongoing” and “deadlock” if the interleaving is allowed.

Observation 11: For deadlock issue of option a, it affects all devices (non-overlapping device and overlapping devices).

Proposal 1: RAN2 confirms that for CBRA, the parallel service requests by the same reader is not supported no matter to the same set of devices or different devices.

Proposal 2: For coverage overlapping among multiple readers scenario, the readers implementation still needs to follow the physical layer restrictions (e.g., timing relationship), i.e., readers coordination via implementation is inevitable in any case.

Proposal 3: The device always respond to the new service indicated by the received paging message, regardless whether there is any procedure ongoing, i.e., no specification impact, same as the description in the current section “Upon receiving the A-IoT Paging message” in the MAC running CR.

# Reference

[1] R2-2502211	Email discussion report: [POST129][035][AIoT] Paging	Qualcomm Incorporated

[2] R2-2503155	Offline discussion report: [AT129bis][010][AIoT] Paging	Qualcomm Incorporated