3GPP TSG-RAN WG2 Meeting #130	R2-2503492

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.13.3

Source:	OPPO

Title:	Control plane procedures of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the control plane procedures of multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

SRAP-1: SRAP configuration of indirectly connected Remote UE

For how to configure the Remote UE ID and BEARER ID, in RAN2 #129bis meeting, it was agreed that SRAP configuration at least includes the configuration of the directly connected child UE, based on the L2 ID of the directly connected child UEs and FFS on whether to include the configurations of the indirectly connected child UEs.

SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17. For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs. For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination. We will review next meeting if this explicit mapping functionality is needed. FFS whether to introduce reflective bearer mapping as an optional feature.

The current specifications (both SRAP and RRC) are drafted based on the baseline mechanism, i.e., the SRAP configuration of each indirect child UE is included, and it works well with very minor specification impact.

The baseline mechanism works well and is implemented in the running CR with very minor specification impact.

in R17 U2N Relay, the network configures the SRAP configuration for one or more remote UEs at the U2N Relay as follows:

-- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ... } <…> SL-SRAP-Config-r17 ::= SEQUENCE { sl-LocalIdentity-r17 INTEGER (0..255) OPTIONAL, -- Need M sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17 OPTIONAL, -- Need N sl-MappingToReleaseList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17 OPTIONAL, -- Need N ... } SL-MappingToAddMod-r17 ::= SEQUENCE { sl-RemoteUE-RB-Identity-r17 SL-RemoteUE-RB-Identity-r17, sl-EgressRLC-ChannelUu-r17 Uu-RelayRLC-ChannelID-r17 OPTIONAL, -- Cond L2RelayUE sl-EgressRLC-ChannelPC5-r17 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N ..., } -- TAG-SL-L2RELAYUE-CONFIG-STOP -- ASN1STOP

As shown in the above specification, the network configures the SRAP configuration for each remote UE at the U2N Relay UE based on the L2 ID of the remote UE via the ToAddModList structure. The L2 ID is used for both link and SRAP configuration identification.

In R17, at the U2N Relay UE, the SRAP configuration for each remote UE is configured based on the L2 ID of the remote UE, the L2 ID is mandatory present and can uniquely identify a link and SRAP configuration for the remote UE.

UE ID/BEARER ID and RLC channel configuration are used for bearer mapping, i.e., the mapping from a radio bearer identified by UE ID field and BEARER ID field to egress RLC channel, which means both UE ID and BEARER ID are required to identify a radio bearer.

The configuration of the SRAP entity for U2N Relay UE includes: - The local identity for each U2N Remote UE via RRC; - Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress Uu Relay RLC channel for each U2N Remote UE via RRC; - Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress PC5 Relay RLC channel for each U2N Remote UE via RRC.

For the proposed reflective bearer mapping to save the explicit local ID configuration, it is not clear how to identify the radio bearer without UE ID or Bearer ID.

In legacy, radio bearer is identified by UE ID and BEARER ID, it is not clear how to identify the radio bearer without local ID or bearer ID configuration.

In legacy, egress RLC channel is configured by the network based on the BEARER ID for each remote UE, it is not clear how to know the RLC channel without RLC channel configuration.

Another issue is in legacy, the remote UE is added by the network to the relay UE, and relay UE upon SRAP PDU reception, will check whether the Local ID and BEARER ID in SRAP header match the SRAP configuration, to see whether the received data is an unknown, unforeseen, and erroneous protocol data. Without local ID and/or BEARER ID configuration, it means the relay UE cannot check whether the received PDU is an unknown, erroneous DU or not, it forwards any received data blindly, which is not a reasonable UE behavior.

In legacy, Relay UE checks whether the received SRAP PDU is unknown, unforeseen, and erroneous by matching the UE ID and BEARER ID with the SRAP configuration, without local ID configuration, the handling of unknown, unforeseen, and erroneous protocol data cannot work.

The argument of using reflective bearer mapping is to save the explicit mapping configuration. The current SRAP configuration of a Remote UE includes the following information:

Table 1. SRAP configuration of L2 U2N Relay

| IE |  | Can be saved by using reflective mapping |
| --- | --- | --- |
| L2 ID of the directly connected child UE: sl-L2IdentityRemote |  | Cannot be further saved since in the current running CR, the SRAP configuration list of the indirectly connected child UE is already associated with the single L2 ID entry |
| sl-SRAP-ConfigRelay | sl-LocalIdentity | Cannot be saved since Local ID + Bearer ID can uniquely identify the bearer of the remote UE |
|  | sl-RemoteUE-RB-Identity | Cannot be saved since Local ID + Bearer ID can uniquely identify the bearer of the remote UE |
|  | sl-EgressRLC-Channel | Cannot be saved since relay needs to know the mapped RLC channel configuration. |

As shown in the above table, it is not clear what signaling can be saved by reflective bearer mapping.

Therefore, reflective bearer mapping as an optimization should not be supported considering the above issues and unclear gain.

(SRAP-1) RAN2 not pursue reflective bearer mapping enhancement and confirm the current running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE) as it is.

SI/Paging

In the POST RAN2 #129bis discussion, majority companies think the legacy specification is enough to allow direct SIB monitoring at the intermediate relay when the SIB requested by the Child UE is also the concerned SIB of the Intermediate Relay UE. However, there are different views on whether additional clarification is needed for the case when the requested SIB by the Child UE is not concerned by the Intermediate relay UE.

- For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly, RAN2 is suggested to discuss whether this can be left to UE implementation[5/11] or capture it in stage2 specification[5/11].

Based on the current stage-2 specification, it says the remote UE can acquire any necessary SIB (s), and based on the discussion, all the companies agree this sentence also applies to the intermediate Relay UE, and the following TP is agreed upon by all the companies.

The in-coverage L2 U2N Remote UE/intermediate relay UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE.

With the above unanimously agreed TP, our understanding is that the necessary SIB(s) for the intermediate relay UE include any SIB requested by the child UE no matter concerned or not concerned by the intermediate relay itself, since otherwise, its behavior aligns functionally with the L2 U2N Remote UE under existing specifications. Consequently, the legacy Stage 2 text (which addresses only the L2 U2N Remote UE) remains sufficient without modification.

The necessary SIB(s) for Intermediate U2N relay UE includes the SIB(s) requested by the Child UE already.

For any SIB(s) requested by child UE, the Intermediate U2N Relay UE is allowed to acquire them over Uu interface as necessary SIB(s) without any further specification impact.

For Paging message forwarding. It is agreed that the Intermediate U2N Relay UE can monitor paging for a child UE on Uu interface, and the duplicated paging delivery to the Remote UE should be avoided.

Agreements:

Support the child UE relying on the parent relay UE for paging monitoring.

Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

The reasonable Intermediate U2N Relay UE behavior should be either to send the paging request to the parent UE or acquire directly from the network.

Upon reception of paging request from the Child UE, the intermediate U2N Relay UE should either send the paging request to the parent UE or acquire paging directly from the network.

Based on the current specification, the RemoteUEInformationSidelink is sent for Paging requesting if the UE has paging-related information to provide, which allows the UE implementation to decide whether the paging request to the parent is needed or it can acquire paging directly from the network. This is also aligned with the discussion of SIB request.

1> if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE as follows: 2> if the L2 U2N Remote UE is in RRC_IDLE: 3> include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer; 2> else if the L2 U2N Remote UE is in RRC_INACTIVE: 3> include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, 4> set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC); 3> else: 4> set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

Similar to SIB request, legacy condition “if the UE has paging related information to provide” is sufficient to let the intermediate U2N Relay UE forward paging request for the child UE when necessary (i.e., no direct acquisition from the network).

Rely on legacy condition “if the UE has paging related information to provide” to allow intermediate U2N Relay UE to either forward paging request for the child UE when necessary or to directly acquire from the network (but not both) without additional spec impact.

The related procedure should be captured to support the RemoteUEInformationSidelink transmission at the Intermediate U2N Relay UE for paging request. Similar to SIB request, a paging information list can be included to request Paging for one or multiple remote UEs. The TP can be as follows:

1> if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows: 2> if the L2 U2N Remote UE is in RRC_IDLE: 3> include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer; 2> else if the L2 U2N Remote UE is in RRC_INACTIVE: 3> include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, 4> set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC); 3> else: 4> set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC; 2> if any paging information is received from the Child UE: 3> include the received paging information in the sl-PagingInfo-RemoteUE-List; <…> RemoteUEInformationSidelink-r17-IEs ::= SEQUENCE { sl-RequestedSIB-List-r17 SetupRelease { SL-RequestedSIB-List-r17} OPTIONAL, -- Need M sl-PagingInfo-RemoteUE-r17 SetupRelease { SL-PagingInfo-RemoteUE-r17} OPTIONAL, -- Need M lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RemoteUEInformationSidelink-v1800-IEs OPTIONAL } RemoteUEInformationSidelink-v1800-IEs ::= SEQUENCE { sl-RequestedPosSIB-List-r18 SetupRelease { SL-RequestedPosSIB-List-r18 } OPTIONAL, -- Need M sl-SFN-DFN-OffsetRequested-r18 ENUMERATED { true } OPTIONAL, -- Need R connectionForMP-r18 ENUMERATED {true} OPTIONAL, -- Need N sl-DestinationIdentityRemoteUE-r18 SL-DestinationIdentity-r16 OPTIONAL, -- Need N nonCriticalExtension RemoteUEInformationSidelink-v1900-IEs OPTIONAL } RemoteUEInformationSidelink-v1900-IEs ::= SEQUENCE { sl-PagingInfo-RemoteUE-List-r19 SetupRelease { SL-PagingInfo-RemoteUE-List-r19} OPTIONAL, -- Need M SL-PagingInfo-RemoteUE-List-r19 ::= SEQUENCE (SIZE (max)) OF SL-PagingInfo-RemoteUE-r17

Similar to SIB request, a paging information list should be introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs as above TP.

Another FFS point is whether the intermediate relay in RRC connected can release the Paging information in the request to its parent relay for the child node.

In R17, for a Relay UE in RRC connected state, it requests and acquires paging for the remote UE via dedicated RRC signalling.

##### 5.8.9.8.3 Reception of RemoteUEInformationSidelink message by the L2 U2N/U2U Relay UE The L2 U2N Relay UE shall: 1> if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE: 2> if the UE is in RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace; or 2> if the UE is in RRC_IDLE or RRC_INACTIVE: 3> if the sl-PagingInfo-RemoteUE is set to setup: 4> monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE; 3> else (the sl-PagingInfo-RemoteUE is set to release): 4> stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion; 4> release the received paging information in sl-PagingInfo-RemoteUE; 2> else (the UE is in RRC_CONNECTED on an active BWP without pagingSearchSpace configured): 3> if the sl-PagingInfo-RemoteUE is set to setup: 4> include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3; 3> else (the sl-PagingInfo-RemoteUE is set to release): 4> initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3; 4> release the received paging information in sl-PagingInfo-RemoteUE; RRCReconfiguration-v1700-IEs ::= SEQUENCE { otherConfig-v1700 OtherConfig-v1700 OPTIONAL, -- Need M sl-L2RelayUE-Config-r17 SetupRelease { SL-L2RelayUE-Config-r17 } OPTIONAL, -- Need M sl-L2RemoteUE-Config-r17 SetupRelease { SL-L2RemoteUE-Config-r17 } OPTIONAL, -- Need M dedicatedPagingDelivery-r17 OCTET STRING (CONTAINING Paging) OPTIONAL, -- Cond PagingRelay needForGapNCSG-ConfigNR-r17 SetupRelease {NeedForGapNCSG-ConfigNR-r17} OPTIONAL, -- Need M needForGapNCSG-ConfigEUTRA-r17 SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17} OPTIONAL, -- Need M musim-GapConfig-r17 SetupRelease {MUSIM-GapConfig-r17} OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M scg-State-r17 ENUMERATED { deactivated } OPTIONAL, -- Need S appLayerMeasConfig-r17 AppLayerMeasConfig-r17 OPTIONAL, -- Need M ue-TxTEG-RequestUL-TDOA-Config-r17 SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17} OPTIONAL, -- Need M nonCriticalExtension RRCReconfiguration-v1800-IEs OPTIONAL } dedicatedPagingDelivery This field is used to transfer Paging message for the associated L2 U2N Remote UE to the L2 U2N Relay UE in RRC_CONNECTED.

The RRC_CONNECTED relay UE on an active BWP without pagingSearchSpace configured acquires paging message of the RRC_CONNECTED remote UE via dedicatedPagingDelivery.

Therefore, the same mechanism can be reused for the intermediate relay UE in multi-hop U2N Relay, i.e., the intermediate relay relies on the dedicated Uu RRC message (i.e., SUI to report and RRCReconfiguration to acquire) to acquire the paging message for its child UEs.

As in R17, the RRC_CONNECTED L2 intermediate relay UE reports paging-related information of the connected child UE via SUI and acquires paging messages for its child UEs via dedicatedPagingDelivery.

Therefore, there is no need to keep the paging information request for the child node towards the parent relay UE.

The intermediate relay UE in RRC_CONNECTED releases the paging-related information for the child UEs in the parent UE.

Conclusion

We have the following observations:

Observation 1	The baseline mechanism works well and is implemented in the running CR with very minor specification impact.

Observation 2	In R17, at the U2N Relay UE, the SRAP configuration for each remote UE is configured based on the L2 ID of the remote UE, the L2 ID is mandatory present and can uniquely identify a link and SRAP configuration for the remote UE.

Observation 3	In legacy, radio bearer is identified by UE ID and BEARER ID, it is not clear how to identify the radio bearer without local ID or bearer ID configuration.

Observation 4	In legacy, egress RLC channel is configured by the network based on the BEARER ID for each remote UE, it is not clear how to know the RLC channel without RLC channel configuration.

Observation 5	In legacy, Relay UE checks whether the received SRAP PDU is unknown, unforeseen, and erroneous by matching the UE ID and BEARER ID with the SRAP configuration, without local ID configuration, the handling of unknown, unforeseen, and erroneous protocol data cannot work.

Observation 6	As shown in the above table, it is not clear what signaling can be saved by reflective bearer mapping.

Observation 7	The necessary SIB(s) for Intermediate U2N relay UE includes the SIB(s) requested by the Child UE already.

Observation 8	Upon reception of paging request from the Child UE, the intermediate U2N Relay UE should either send the paging request to the parent UE or acquire paging directly from the network.

Observation 9	Similar to SIB request, legacy condition “if the UE has paging related information to provide” is sufficient to let the intermediate U2N Relay UE forward paging request for the child UE when necessary (i.e., no direct acquisition from the network).

Observation 10	The RRC_CONNECTED relay UE on an active BWP without pagingSearchSpace configured acquires paging message of the RRC_CONNECTED remote UE via dedicatedPagingDelivery.

We have the following proposals:

Proposal 1	(SRAP-1) RAN2 not pursue reflective bearer mapping enhancement and confirm the current running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE) as it is.

Proposal 2	For any SIB(s) requested by child UE, the Intermediate U2N Relay UE is allowed to acquire them over Uu interface as necessary SIB(s) without any further specification impact.

Proposal 3	Rely on legacy condition “if the UE has paging related information to provide” to allow intermediate U2N Relay UE to either forward paging request for the child UE when necessary or to directly acquire from the network (but not both) without additional spec impact.

Proposal 4	Similar to SIB request, a paging information list should be introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs as above TP.

Proposal 5	As in R17, the RRC_CONNECTED L2 intermediate relay UE reports paging-related information of the connected child UE via SUI and acquires paging messages for its child UEs via dedicatedPagingDelivery.

Proposal 6	The intermediate relay UE in RRC_CONNECTED releases the paging-related information for the child UEs in the parent UE.