3GPP TSG-RAN WG2 Meeting #130	R2-2503515

St. Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.3.3

Source:	Qualcomm Incorporated

Title:	Configuration and reporting for inference for network-side model

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #129-bis meeting [1], the final set of conclusions were drawn from the simulation studies on AI/ML for mobility. Also, RAN2 initiated the study of LCM aspects and specification impacts for AI/ML for mobility, with a view towards converting the SI into a WI and determining the scope of the WI in Rel-20. A set of basic agreements were reached on the LCM aspects and specification impacts for AI/ML for mobility [1].

In this short contribution, we discuss configuration and reporting for inference for network-side model for AI/ML for mobility.

# Configuration and reporting for inference for network-side model

Network-side AI/ML model was proposed for RRM measurement prediction in the SI description of Rel-19 AI/ML for mobility [2].

It is possible to have network-side AI/ML model for Measurement Event prediction as well. The following observation indicates that no additional information or reporting is needed from the UE to support network-side Measurement Event prediction than is needed to support network-side RRM measurement prediction.

Observation 1. If UE provides RRM measurement reports to support network-side RRM measurement prediction, then no additional information or reports are needed by the source gNB from the UE to support network-side Measurement Event prediction.

For network-side RRM measurement prediction, the source gNB configures the UE with the cells and the set of beams on each cell to measure and a measurement reporting configuration, similar as for UE-side RRM measurement prediction. The exact configuration depends on the use case – e.g., intra-frequency, inter-frequency, intra-cell, or inter-cell – and it should be left up to the WI phase to specify. The configuration can use the legacy RRM measurement configuration and reporting framework.

Proposal 1: For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. Any additional impact to the specification is left up to the WI phase discussion.

# 3 Conclusions

Based on the discussion above, we recommend RAN2 to discuss the proposals and observations below.

Observation 1. If UE provides RRM measurement reports to support network-side RRM measurement prediction, then no additional information or reports are needed by the source gNB from the UE to support network-side Measurement Event prediction.

Proposal 1: For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. Any additional impact to the specification is left up to the WI phase discussion.

# References

[1] Chair notes, RAN2 #129-bis meeting.