3GPP TSG-RAN WG2 Meeting #130	R2-2503528

St.Julians, Malta, 19th – 23rd May, 2025

Agenda Item:	8.9.2

Source: 	OPPO

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

# Introduction

In Rel-19 IoT NTN WID [1], Store & Forward satellite operation is one of the features to be supported.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

This contribution would like to provide our considerations on this objective of Rel-19 IoT NTN.

# Discussion on Store & Forward satellite operation

Below are the agreements made in the RAN2#129bis meeting, here we discuss the leftover issues based on them.

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

Transition time between S&F mode and Normal mode

In RAN2#129bis meeting, on top of the transition time from S&F mode to normal mode, it was further agreed to introduce the indication for the normal mode to S&F mode transition, while the details are FFS.

The issue has already discussed in the post-meeting email discussion [Post129bis][310], with the conclusion as shown below.

Proposal 1: (RRC-1) RAN2 to further discuss which of the following options to choose for indicating the transition time from normal mode to S&F mode: Option 1: (5/9) It is up to NW implementation to set the legacy t-Service as the transition time from normal mode to S&F mode. Option 2: (5/9) Using the agreed time information in SIB31 for both directions of transition. UE determines which direction it is based on whether the S&F indication is present.

In our understanding, Option 2 is preferred, since using the same field of time information to indicate both directions of transition could be supported without additional effort. UE can know the transition direction based on the “S&F operation” indication broadcasted in SIB1. If time information and “S&F operation” indication are both present, UE can realize the time information is for the transition from “S&F operation” mode to real-time/normal mode. And if time information is present but “S&F operation” indication is not present, UE can realize the time information is for the transition from real-time/normal mode to “S&F operation” mode. Thus, it is no harm to further support indicating the time information for the opposite transition.

- [RRC-1] Use the agreed time information in SIB31 for both directions of transition time between normal mode and S&F mode, and UE determines which direction it is based on whether the S&F indication is present.

Considering the details on the solution have already been covered by post-meeting email discussion [Post129bis][310], if RAN2 would like to progress more, here we further discuss the leftover issues on the opposite transition.

We understand the time information for the opposite transition can also be used for AS layer. At least for Rel-19 UEs not supporting S&F operation, it can allow these UEs to start neighbour cell measurement for reselection before transiting to “S&F operation” mode.

- Rel-19 UEs not supporting S&F operation allow to start neighbour cell measurement for reselection before transiting to S&F operation mode, like the behaviours adopted for t-Service.

S&F operation indication

For “S&F operation” indication broadcasted in SIB1, there are three states, i.e., “1”, “0”, and “not present”. One issue is whether the change of “S&F operation” indication leads to system information change notifications or not. There are three cases for the changes.

- Case-1) In case that the current satellite operation mode transits from “S&F operation” mode to real-time/normal mode, “S&F operation” indication in SIB1 changes to be not present.

- Case-2) In case that the current satellite operation mode transits from real-time/normal mode to “S&F operation” mode, “S&F operation” indication in SIB1 changes to be present.

- Case-3) “S&F operation” indication in SIB1 changes from 0 to 1, or vice versa.

For Case-1, thanks to the time information agreed in the last meeting, RRC_IDLE UE does not have to rely on system information change notifications to determine the changes. UE can be aware of the changes according to time information.

For Case-2, since it is still FFS on whether the time information on transition from real-time/normal mode to “S&F operation” mode is introduced or not, it is questionable whether UE can know the changes if not relying on system information change notifications.

For Case-3, system information change notifications are essential, since no other information is provided to UE.

Considering that system information change notifications are required at least for Case-3, but it is weird to only apply system information change notifications to Case-3, we think for all cases above it is necessary to still rely on system information change notifications to estimate the change of “S&F operation” indication as legacy, i.e., changes of “S&F operation” indication should result in system information change notifications.

- Changes of S&F operation indication should result in system information change notifications.

Furthermore, it is agreed that the “S&F operation” indication setting as “0” means the cell is operating in “S&F operation” mode for UE in RRC_CONNECTED mode, while NB-IoT/eMTC UEs in RRC_CONNECTED mode are not required to detect system information changes. Thus, it is questionable how UE in RRC_CONNECTED mode behaves in the following scenarios:

- Scenario 1: the cell is switching from “S&F operation” mode to normal mode

- Scenario 2: the cell is switching from normal mode to “S&F operation” mode

Due to the difference in latency between “S&F operation” mode and normal mode, for both Scenario 1 and 2, Rel-19 UEs supporting S&F operation may be updated with some NAS parameters, such as QoS parameters, in NAS layer, and then some AS configurations may need to be updated accordingly. Especially for UEs in Scenario 2, i.e., the cell that is switching from normal mode to “S&F operation” mode, the latency requirement of ongoing services may not be fulfilled due to lake of the feeder link connection.

Besides, for a cell in normal mode, except for Rel-19 UEs supporting S&F operation, it is possible to also allow legacy UEs and Rel-19 UEs not supporting S&F operation to access. Therefore, if this cell is switching from normal mode to “S&F operation” mode, these legacy UEs should be considered during NW handling.

In general, from RAN2 perspective, there are three ways of NW handling in AS layer for the two scenarios above:

- Release UE to RRC_IDLE mode,

- Trigger handover to switch the UE to a correct cell,

- Update the radio bearer configuration to adapt the operation mode of the current cell, for Rel-19 UEs supporting S&F operation.

We understand all these three ways are fully up to NW implementation, and no special UE behaviour is needed in AS layer from RAN2 perspective. But considering for the two scenarios above maybe some NAS procedures are necessary which is up to SA2, it is not sure whether there is any AS impact caused by NAS layer. It would be safer to further check it with SA2.

- Rely on NW implementation to handle UE in RRC_CONNECTED mode if NW switches between S&F operation mode and normal mode, and no special AS impact is foreseen. FFS whether there is any AS impact caused by NAS layer.

MME-configured satellite list

For Satellite ID-related issue, the reply LS from SA2 [2] on MME-configured satellite list is received, as shown below.

1. Overall Description: SA2 thanks RAN2 for the LS on Satellite IDs for store-and-forward operation and would like to provide the following feedback. - RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. [SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9. Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s): - the satellite has the UE context but does not necessarily support store-and-forward operation; or - the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)? [SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

For the Question, SA2 confirms the option 2 as the correct understanding. However, for the RAN2 understanding, SA2 does not provide a direct answer, and just refer to TS 23.401 clause 4.13.9.

In the latest SA2 CR S2-2502513 [3] for TS 23.401, based on NOTE B in clause 4.13.9 as shown below, it is fully up to UE implementation to use the MME-configured satellite list, which means the RAN2 understanding is correct, i.e., the UE configured with a satellite ID list by MME may camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. But at the same time, it also points out that in this case there is increased probability that it will not be able to complete the NAS procedure.

NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In our understanding, we need to confirm the SA2 response of the RAN2 understanding, considering no direct answer from SA2. Besides, as mentioned by companies in post-meeting email discussion [Post129bis][310], RAN2 needs to clarify that it is up to UE to find better normal mode cell at any time.

- RAN2 clarifies that the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. i.e., it is up to UE to find better normal mode cell at any time.

Furthermore, we need to further discuss whether there is any AS impact on their concern, e.g., whether to prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list during cell reselection procedure or not.

In RAN2#129bis meeting, it was discussed but without consensus, as captured by the meeting minutes:

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list. - Google supports this. CMCC supports this but thinks this would apply to UEs supporting S&F - IDC thinks we don’t have cell specific priorities but only frequency priorities. Also what does it means that the UE prioritizes. IDC also thinks that NB-IoT does not support priority based cell reselection. - Nokia does not support this and thinks the observations do not refer to cell reselection - QC thinks p1 and p1a are in contradiction (p1 relates to MME configured satellite list) - Novamint supports the principle of p1 and p1a - Samsung wonders if there will be many satellites the UE can prioritize - Xiaomi thinks p1 makes sense if there are only S&F mode cells around. - Come back in the next meeting (the proponents could show the possible spec impact if we decide to go in this direction)

Note that, based on the reply LS from SA2[2], SA2 thinks the UE behavior on MME configured satellite list is up to UE implementation, we understand that it is proper to also up to UE implementation whether to prioritize the cell in MME-configured satellite list in cell reselection.

- Up to UE implementation whether to prioritize the cell in MME-configured satellite list in cell reselection.

Open issue on UE capability

In the open issue list of 36306, one open issue on S&F is whether to introduce a UE AS capability with signalling for support of S&F mode or not, as captured in the 36306 running CR below.

#### 4.3.38.xx ntn-StoreAndForward-r19 This field defines whether the UE supports the store and forward operation when feeder link is not available as defined in TS 36.300 [xx]. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17. Editor’s note: eNB may also need this information to decide whether to keep the UEs or release the UEs when feeder link connection is lost or will be lost soon. Given UE capability is exchanged between UE and MME, this may not be needed. FFS whether to keep this UE capability signaling over radio.

As discussed in our Proposal 3, for UE in RRC_CONNECTED mode, rely on NW implementation for UE handling if eNB switches between S&F operation mode and normal mode, and the different ways of NW handling in AS layer includes: 1) release UE to RRC_IDLE mode, 2) trigger handover to switch the UE to a correct cell, or 3) update the radio bearer configuration to adapt the operation mode of the current cell, for Rel-19 UEs supporting S&F operation. Therefore, to assist eNB to make the proper implementation, eNB also needs to know the information of whether the UE supports S&F operation or not.

Considering that NAS layer capability for UE supporting S&F operation has already been introduced, we understand eNB could know this information of UE based on the network-side interaction between eNB and MME by network implementation. For example, when MME detaches the UE by MME-initiated Detach Procedure in NAS layer, at the same time MME also sends S1 Release Command message to eNB to release the S1-MME signalling connection for the UE. Therefore, eNB can simply follow the indication of MME to perform AS handling accordingly. It is redundant for UE to indicate the capability in both AS layer and NAS layer.

- [UE Cap-1] Not introduce AS capability signalling for UE supporting of S&F mode, given the introduced NSA capability for this.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Transition time between S&F mode and Normal mode

- [RRC-1] Use the agreed time information in SIB31 for both directions of transition time between normal mode and S&F mode, and UE determines which direction it is based on whether the S&F indication is present.

- Rel-19 UEs not supporting S&F operation allow to start neighbour cell measurement for reselection before transiting to S&F operation mode, like the behaviours adopted for t-Service.

S&F operation indication

- Changes of S&F operation indication should result in system information change notifications.

- Rely on NW implementation to handle UE in RRC_CONNECTED mode if NW switches between S&F operation mode and normal mode, and no special AS impact is foreseen. FFS whether there is any AS impact caused by NAS layer.

MME-configured satellite list

- RAN2 clarifies that the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. i.e., it is up to UE to find better normal mode cell at any time.

- Up to UE implementation whether to prioritize the cell in MME-configured satellite list in cell reselection.

Open issue on UE capability

- [UE Cap-1] Not introduce AS capability signalling for UE supporting of S&F mode, given the introduced NSA capability for this.

# 4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek

[2] R2-2501753, Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT)

[3] S2-2502513, Support of S&F Monitoring List in S&F Mode, Huawei, HiSilicon, Samsung