3GPP TSG-RAN WG2 Meeting #130	R2-2503545

St. Julians, Malta, May 19th– 23rd, 2025

Agenda Item:	8.3.4

Source:	OPPO

Title:	Discussion on data collection

Document for:	Discussion, Decision

Introduction

In RAN2#129bis, we agreed that for data collection to take AI/ML PHY related procedures as baseline for further discussion.

4  As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

5  Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

In this contribution, we would like to look at the different specification impact of both NW-side model and UE-side models on data collection for AI mobility.

Data collection

Common part

Data collection configuration

Figure 2.1.1-1. NR measurement configuration and report procedure

Figure 2.1.1-1 illustrates the procedure of NR measurement configuration and report. NW sends measurement configuration to UE to give the details about measurement objects, report configuration, measurement identities, quantity configurations, and measurement gaps. An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the NW. The NW may also configure the UE to report L3 measurement information per beam. Once the report condition is met, UE will transfer measured results to the NW.

Observation 1: NR measurement configuration includes measurement objects, report configuration, measurement identities, quantity configurations, and measurement gaps.

Observation 2: NR measurement report includes L3 cell and L3 beam measurement results.

For data collection for RRM and measurement event prediction, the measurement is to some extend similar to NR measurement. The contents of those configuration can be reused as much as possible.

Proposal 1: Measurement configuration and report framework is taken as baseline for data collection.

During the simulation phase, two different filtering approaches were evaluated, i.e., sliding filtering and non-sliding filtering. These approaches feature different measurement periods. At least in simulation, we have already tested measurement periods of 40ms, 80ms, 200ms, and 400ms.

Observation 3: Measurement period for data collection tasks can vary.

In NR, only L3 cell or beam level measurement results are reported to the NW. During the simulation phase, we have discussed different types of measurement results that could serve as inputs and outputs for AI/ML models.

Figure 2.1.3-2. measurement model in TS 38.300

Referring to Figure 2.1.3-2 from TS 38.300, data points A, A1, B, C, E, and F can all be utilized. That means during data collection procedure, the report of all types of measurement result types should be supported. Therefore, it is proposed that

Proposal 2: Additionally, data collection configuration includes at least the following components:

- Measurement result type indication: Specify the type of measurement results, such as L1 beam results, L3 beam-level results and L3 cell-level results.

- Logging time interval: Set the time interval for measurement logging, e.g., 200ms.

In simulation, we assume that the measurement results of two different frequencies can be obtained simultaneously. However, measuring two different frequencies introduces delays due to the need to switch between frequencies. Currently, it is up to UE to determine the exact timing for measure within a measurement period. If the time gap between the logged measurements of two different frequencies is large. The correlation between the two measurement results will decrease. This will impair the performance of AI/ML.

Observation 4: Inter-frequency prediction performance will be impacted by the logging time of measurements of different frequencies.

To make sure the correlation between measured results of different frequencies does not degrade much, it is proposed that

Proposal 3: The logging timing for different frequencies in inter-frequency prediction should be aligned.

NW-side model specific

For NW-side model data collection, as per agreement at RAN2#129bis, only RRM prediction is considered. NW-side model for measurement event prediction is up to NW implementation.

Observation 5: NW-side model data collection is for RRM prediction only.

Data collection initiation

In AI/ML for PHY, the following sentences are agreed

RAN2#125bis

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

and

RAN2#126

Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

We can reuse those conclusions for AI mobility.

Proposal 4: RAN2 confirms the following BM agreements can be used for AI mobility:

For NW-side model, model training, data collection can be initiated by OAM (via immediate MDT) or by gNB. The same measurement framework is applied to both gNB-centric and OAM-centric data collection.

Data logging

Regarding data logging, AI/ML for PHY states

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

and

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

The principle applies to both BM and AI mobility, we do not see any obstacles that prevent us from reaching to the following agreement:

Proposal 5: Reuse the wording of data logging in BM for AI mobility NW-side model, i.e., both periodic and radio condition-based event-triggered data logging are supported. UE can use L3 measurement event-triggered to determine whether to perform data logging.

Proposal 6: Reuse the wording of data logging in BM for AI mobility NW-side model, i.e., periodic reporting of logged data is not supported, and on-demand reporting of the logged measurements will be specified

Availability of logged data

For availability of logged data, AI for PHY confirms that

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

In RAN2#129bis, additional agreements were reached regarding the availability indication:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

We propose extending these solid conclusions into AI mobility while setting aside the FFS items for future study.

Proposal 7: RAN2 to confirm the agreements from AI PHY on availability of logged data that:

- The UE can indicate the availability of logged data to the network. Upon reception of the availability indication, the network can trigger the UE information procedure.

- During handover the UE can retain the logged data and the UE can indicate the availability of logged data to network.

- Availability indication can be triggered by full buffer being reached (if configured), buffer threshold being reached (if configured), or low power (if configured)

- UAI is used to indicate the availability of logged data, reason (full buffer, exceed threshold) to trigger, and low power state.

In RAN2#129bis, AI for PHY also agreed that

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

For AI mobility, we do not see the necessity to do the same. Releasing the logged data is a waste of UE power and resources. Once UE re-enters RRC_CONNECTED, the logged data can still be transferred to the NW for model training.

Proposal 8: UE should be allowed to retain logged data when transitioning to RRC_IDLE, RLF, or RRC_INACTIVE, and report those data to the NW upon returning to RRC_CONNECTED, for purpose of NW-side model training.

Low power state

In the AI/ML PHY, it is stated that

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected.

- The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

- Low power bit indication is supported

Again, it can be easily applied to AI mobility. Therefore, we propose that

Proposal 9: RAN2 confirms the following contents from AI PHY also apply to AI mobility:

To reduce UE power consumption, when low power state is detected, the UE can indicate the low power state to the network. Upon reception of the low power status indication, the network should de-configure UE to release the data collection configuration. How the UE determines low power state is up to UE implementation.

Buffer limitation

For buffer limitation, AI PHY agreed that

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

AI mobility can reach similar conclusions as follow

Proposal 10: RAN2 to confirm the agreements from AI PHY on buffer limitation of logged data that:

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

UE-side model specific

Data collection procedure

For UE-side model, RAN2#129 agreed that

- For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

It can be reused by AI mobility that

Proposal 11: RAN2 confirms the agreement in BM applies for AI mobility:

For UE-side model, the network can configure whether UE is allowed to initiate a request about data collection configuration. The network can also provide UE with data collection configuration or release the data collection configuration (at any point in time), with or without UE request.

Data collection configuration

For UE-side model, we agreed to study both RRM prediction and measurement event prediction.

For indirect measurement event prediction, although the model used is identical to that for RRM prediction, the associated data collection tasks can differ. For example, data collection for RRM prediction can configure the UE to measure frequency#1. In contrast, data collection for measurement event prediction may require measurement of frequency#2. Besides, at least NW shall inform UE whether a given data collection task is intended for RRM or measurement event prediction, e.g., via including event type indicator. For measurement event prediction, data collection requirements also differ from those of RRM prediction, particularly in terms of the logging of occurring time of measurement events.

Observation 6: Data collection configuration for RRM prediction and measurement event prediction for UE-side model training are distinct, e.g., they may involve measurement on different cells/frequencies and may or may not include measurement event type.

This distinction allows a UE to execute two separate data collection tasks for RRM prediction and measurement event prediction simultaneously.

Proposal 12: Data collection configuration for training of measurement event prediction and RRM prediction should be treated as two separate configurations, enabling the simultaneous execution of different data collection tasks.

For UE-side model, the data collection configuration, e.g., filtering parameters, is typically set by the gNB. However, the logged data report can be transferred to other network nodes (e.g., NWDAF) and may become transparent to the gNB. In such cases, the receiving node would require access to the data collection configuration to perform model training effectively.

Even when the logged data report is not transparent to the gNB, the time interval between the configuration setup and the actual reporting of logged data can be significant. The network may not retain the corresponding data collection configuration over such an extended period. Therefore, it would be beneficial for the UE to include relevant configuration details in its logged data report to ensure consistency and enable accurate data usage.

Proposal 13: For UE-side model, the logged data report includes related data collection configuration(s), such as measurement configurations.

Conclusion

We have following observations and proposals:

Common part: data collection configuration

Observation 1: NR measurement configuration includes measurement objects, report configuration, measurement identities, quantity configurations, and measurement gaps.

Observation 2: NR measurement report includes L3 cell and L3 beam measurement results.

Observation 3: Measurement period for data collection tasks can vary.

Observation 4: Inter-frequency prediction performance will be impacted by the logging time of measurements of different frequencies.

Proposal 1: Measurement configuration and report framework is taken as baseline for data collection.

Proposal 2: Additionally, data collection configuration includes at least the following components:

- Measurement result type indication: Specify the type of measurement results, such as L1 beam results, L3 beam-level results and L3 cell-level results.

- Logging time interval: Set the time interval for measurement logging, e.g., 200ms.

Proposal 3: The logging timing for different frequencies in inter-frequency prediction should be aligned.

NW-side mode general

Observation 5: NW-side model data collection is for RRM prediction only.

NW-side model: Data collection initiation

Proposal 4: RAN2 confirms the following BM agreements can be used for AI mobility:

For NW-side model, model training, data collection can be initiated by OAM (via immediate MDT) or by gNB. The same measurement framework is applied to both gNB-centric and OAM-centric data collection.

NW-side model: Data logging

Proposal 5: Reuse the wording of data logging in BM for AI mobility NW-side model, i.e., both periodic and radio condition-based event-triggered data logging are supported. UE can use L3 measurement event-triggered to determine whether to perform data logging.

Proposal 6: Reuse the wording of data logging in BM for AI mobility NW-side model, i.e., periodic reporting of logged data is not supported and on-demand reporting of the logged measurements will be specified

NW-side model: Availability of logged data

Proposal 7: RAN2 to confirm the agreements from AI PHY on availability of logged data that:

- The UE can indicate the availability of logged data to the network. Upon reception of the availability indication, the network can trigger the UE information procedure.

- During handover the UE can retain the logged data and the UE can indicate the availability of logged data to network.

- Availability indication can be triggered by full buffer being reached (if configured), buffer threshold being reached (if configured), or low power (if configured)

- UAI is used to indicate the availability of logged data, reason (full buffer, exceed threshold) to trigger, and low power state.

Proposal 8: UE should be allowed to retain logged data when transitioning to RRC_IDLE, RLF, or RRC_INACTIVE, and report those data to the NW upon returning to RRC_CONNECTED, for purpose of NW-side model training.

NW-side model: Low power state

Proposal 9: RAN2 confirms the following contents from AI PHY also apply to AI mobility:

To reduce UE power consumption, when low power state is detected, the UE can indicate the low power state to the network. Upon reception of the low power status indication, the network should de-configure UE to release the data collection configuration. How the UE determines low power state is up to UE implementation.

NW-side model: Buffer limitation

Proposal 10: RAN2 to confirm the agreements from AI PHY on buffer limitation of logged data that:

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

UE-side model: Data collection procedure

Proposal 11: RAN2 confirms the agreement in BM applies for AI mobility:

For UE-side model, the network can configure whether UE is allowed to initiate a request about data collection configuration. The network can also provide UE with data collection configuration or release the data collection configuration (at any point in time), with or without UE request.

UE-side model: Data collection configuration

Observation 6: Data collection configuration for RRM prediction and measurement event prediction for UE-side model training are distinct, e.g., they may involve measurement on different cells/frequencies, and may or may not include measurement event type.

Proposal 12: Data collection configuration for training of measurement event prediction and RRM prediction should be treated as two separate configurations, enabling the simultaneous execution of different data collection tasks.

Proposal 13: For UE-side model, the logged data report includes related data collection configuration(s), such as measurement configurations.