3GPP TSG-RAN WG2 Meeting #130	R2-2503575

# St. Julians, Malta,  May 19th – 23rd , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5339 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on SPR configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the field description, thresholdPercentageT304-SCG is “configured by the target PSCell”, but this point has not been reflected in current procedural text. Furthermore, in case of intra-SN PSCell change, when sn-initiatedPSCellChange and thresholdPercentageT304-SCG are configured, according to the existing spec, UE considers the SPR configuration is configured by source PSCell, but not target PSCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifity the procedural text for UE to clarify that if the T304 configuration is configured then it is the target SN configuring it, and if it is absent it is the source SN. Impact analysis Architecture options NR-DC Impacted functionality: Rel-18 SON/MDT SPR Inter-operability: If only the network is implemented according to the CR and the UE is not, no interoperability problems are foreseen. If only the UE is implemented according to the CR and the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may incorrectly determine which node provides the corresponding configuration for evaluating whether the SPR should be logged. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxBW-PreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes minSchedulingOffsetPreferenceConfigExt:

4>	consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation, successful handover report, and successful PSCell change or addition report (if received for the associated cell group);

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if successHO-Config is set to setup:

2>	consider itself to be configured to provide the successful handover information in accordance with 5.7.10.6;

1>	else:

2>	consider itself not to be configured to provide the successful handover information.

1>	if sn-initiatedPSCellChange is not included in the received otherConfig and if the successPSCell-Config in the received otherConfig is set to setup:

2>	consider itself to be configured by the corresponding cell group to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

1>	else:

2>	consider itself not to be configured by the corresponding cell group to provide the successful PSCell change or addition information.

1>	if sn-initiatedPSCellChange is included in the received otherConfig and if the received otherConfig includes successPSCell-Config set to setup and thresholdPercentageT304-SCG is not included; or

1>	if sn-initiatedPSCellChange is included in received otherConfig and successPSCell-Config is already configured for the SCG:

2>	consider itself to be configured by the source PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

- if successPSCell-Config in the received otherConfig is set to setup and thresholdPercentageT304-SCG is included:

2>	consider itself to be configured by the target PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7

1>	if the successPSCell-Config received in otherConfig is set to release:

2>	consider itself not to be configured by the corresponding cell group to provide the successful PSCell change or addition information.

1>	if the received otherConfig includes the ul-GapFR2-PreferenceConfig:

2>	consider itself to be configured to provide its preference on FR2 UL gap in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide its preference on FR2 UL gap;

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

1>	if the received otherConfig includes the musim-GapPriorityAssistanceConfig:

2>	consider itself to be configured to provide MUSIM assistance information for gap(s) priority in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide MUSIM assistance information for gap(s) priority;

1>	if the received otherConfig includes the musim-CapabilityRestrictionConfig:

2>	if musim-CapabilityRestrictionConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for capability restriction in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for capability restriction and stop timer T348 and T346n, if running;

1>	if the received otherConfig includes the rlm-RelaxationReportingConfig:

2>	if rlm-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of RLM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of RLM measurements and stop timer T346j associated with the cell group, if running;

1>	if the received otherConfig includes the bfd-RelaxationReportingConfig:

2>	if bfd-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of BFD measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of BFD measurements and stop timer T346k associated with the cell group, if running;

1>	if the received otherConfig includes the scg-DeactivationPreferenceConfig:

2>	if the scg-DeactivationPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its SCG deactivation preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its SCG deactivation preference and stop timer T346i, if running.

1>	if the received otherConfig includes the propDelayDiffReportConfig:

2>	if the propDelayDiffReportConfig is set to setup:

3>	consider itself to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s) in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s).

1>	if the received otherConfig includes the rrm-MeasRelaxationReportingConfig:

2>	if the rrm-MeasRelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the fulfilment of the criterion for relaxing RRM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the fulfilment of the criterion for relaxing RRM measurements.

1>	if the received otherConfig includes the multiRx-PreferenceReportingConfigFR2:

2>	if the multiRx-PreferenceReportingConfigFR2 is set to setup:

3>	consider itself to be configured to provide its preference on multi-Rx operation for FR2 in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on multi-Rx operation for FR2 and stop timer T346m, if running.

1>	if the received otherConfig includes the aerial-FlightPathAvailabilityConfig:

2>	consider itself to be configured to indicate the availability of flight path information in accordance with 5.7.4;

1>	if the received otherConfig includes the ul-TrafficInfoReportingConfig:

2>	if ul-TrafficInfoReportingConfig is set to setup:

3>	consider itself to be configured to provide UL traffic information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide UL traffic information and stop all instances of timer T346l, if running;

1>	if the received otherConfig includes n3c-RelayUE-InfoReportConfig:

2>	consider itself to be configured to report relay UE information with non-3GPP connection(s).

CHANGE END