3GPP TSG RAN WG2 #130          	                                                                R2-2503590

St Julian’s, Malta, May. 19th – 23rd, 2025

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], many agreements have been achieved for NW side data collection in the aspect of BM use case:

Handling during mobility/state transition:

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Availability indication (Triggering)

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Data collection configuration (Framework):

Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Content of collected data:

For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

SRB to be used:

Agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection  (with lower priority)

In this contribution, we further discuss the data collection for NW-side model training for BM use cases.

- Discussion

  - Mechanism related to NW side data collection

- Configuration framework

For immediate MDT, the UE measurement and report mechanism is entirely under NW control. The NW could configure a set of MeasObjectNR(s) and a set of ReportConfigNR(s), only the MeasObjectNR and ReportConfigNR associated by a measID could be actually performed for measurement. Multiple measId(s) could be configured to the UE for different measurements. Therefore based on the immediate MDT, it is easy for the NW to configure multiple AI/ML purposed measurements, and the activation/deactivation of measurements can be controlled by the presence/absence of measId in RRCReconfiguration.

The gNB should configure UE to perform data logging and reporting based on the potential AI/ML training requirements, such as:

- trigger conditions of measurement result logging;

- trigger conditions of availability indication reporting.

Based on the legacy immediate MDT, the reference signal related information in the serving/neighboring cell(s) to be measured could be included in the MeasObjectNR IE, and the logging trigger conditions and reporting (availability indication) trigger conditions could be introduced in the ReportConfigNR IE which may include logging type, reporting type, logging/reporting quantity, logging interval/amount .etc.

Figure1 AI/ML data collection configuration structure

Furthermore, to consider future AI mobility use cases which are RAN2 specific use cases, to enhance legacy RRM framework is more appropriate. For example, the measurement event prediction use case is to predict the triggering of measurement event based on the UE measured result. AI/ML method is an alternative solution for reducing measurement efforts or enhancing HO performance. Therefore the measId is also needed for identify the measurement context, and the predicted/measured frequency and the triggered event parameters are also need in the measObject and reportConfig related to the measId.

Observation 1: For future AI mobility use cases consideration, the legacy RRM framework should be used as baseline for specification enhancement.

Proposal 1: (RRC-13) Use RRM framework for AI/ML NW side training data collection. The network configures data logging trigger conditions and availability indication reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

- Configuration of data logging

It is agreed in previous RAN2 meeting that both periodic logging and event-triggered logging will be supported.

For periodic logging, it is more like the current periodic CSI measurement in legacy CSI-ResourceConfig.

For event-triggered logging, the UE can use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. Therefore the parameters of l3-Threshold, hysteresis and timeToTrigger can be configured, similar as eventL1 in logged MDT configuration:

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

Proposal 2: Parameters of l3-Threshold, hysteresis and timeToTrigger can be configured for event-triggered training data logging, similar as eventL1 in logged MDT configuration.

- Configuration of data reporting

Besides the event triggered logging parameters, the buffer threshold, and buffer full and/or low power indication have been agreed to be configured by the network:

- For buffer threshold, multiple percentages could be configured, e.g. {p70, p80, p90, p95, spare4, spare3, spare2, spare1};

- For buffer full and/or low power, 2 separate indications for each condition is enough.

Proposal 3: (RRC-8) Network configures multiple percentages for buffer threshold and configures separate indications for buffer full and low power.

- Enhanced data logging in UE

It is agreed in previous RAN2 meeting that both periodic logging and event-triggered logging will be supported.

For periodic recording, it is more akin to a combination of immediate MDT and logged MDT. The UE can record one or multiple entries related to AL/ML of UE measurement results within one reporting period according to the network-configured interval parameters. Moreover, the recording will commence immediately after the UE receives the training data collection configuration.

For event-based recording, the UE can record only one entry if the pre-configured event is triggered. For example, when a handover failure (HOF) occurs or the UE measurement fulfills the event A3 threshold. Or, it can periodically record the measurement results in the UE variable according to the network-configured recording trigger conditions and interval parameters after the recording trigger conditions are met.

To compare among these recording mechanism, different triggering conditions may apply to different data collection content requirements, and the event-triggered periodic data logging could make a trade-off between the one-shot event based logging and the pure periodic logging.

Proposal 4: Support event-triggered periodic data logging to make a trade-off between the one-shot event based logging and the pure periodic logging.

- Data content report by UE

It is agreed in last RAN2 meeting that on-demand reporting principles as for logged MDT will be used for AI/ML data collection of BM use case, and the UEInformationRequest/UEInformationResponse is used. The format could also follow the logged MDT report. Besides the agreed availability indication for further data available for transmission, how to record and report the physical instances of samples in chronological order should be discussed first. In temporal domain, the network could be made aware of the sample measured time by the absolute time and the relative time. The absolute time could be the time when UE received the data collection configuration, or the first entry measured time. The relative time could be the time between the corresponding entry measured time and the absolute time, or represented by the entry number * the fixed gap between two consecutive entries. For simple, the timestamp method for entries recorded for logged MDT could be reused.

Observation 2: The method for entries recorded for logged MDT could be reused for the timestamp of training data collection.

Therefore, to make the network be aware of the timestamp of each sample and to clarify the identity of the training data and the sending termination point, following contents should be included in the reporting:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index similar as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

Proposal 5: The report of collected training data includes:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index similar as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

For the cell ID recorded in each entry as open issue-12, CGI should be used similar as the cell index in logged MDT. Since the training data logging takes a long time, the unique identifier makes the cell attribution of the collected data unambiguous.

Proposal 6: (RRC-12) Use CGI to identify the cell ID in each entry of training data collection.

  - Memory for NW side training data

It is agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. But it is FFS on the memory size. For logged MDT, the minimum memory size of MDT logged measurements is 64KB. Therefore 64KB could also be assumed as the minimum memory size of AI/ML training data.

Proposal 7: Set the minimum memory size of AI/ML training data logging to be 64KB.

  - Transmission priority of NW side training data

It is agreed in last RAN2 meeting that New SRB can be configured with lower priority. So a new SRB6 could be used:

SRB6 is for AI/ML NW side data collection reporting for Beam Management use case, all using DCCH logical channel. SRB6 has a lower priority than SRB1 and may be configured by the network after AS security activation.

Proposal 8: SRB6 is configured with lower priority than SRB1 for AI/ML NW side data collection reporting for Beam Management use case.

Since the UEInformationResponse message will be carried on SRB2 when logged measurement information is included, for training data reporting the similar rule will be used either. If the UEInformationResponse message includes the logged AI/ML training data, SRB6 will be used irrespective whether other information (e.g. SON/MDT report) is included in the same message. And since the logged MDT measurement and the logged AI/ML training data are both large amount of data, the network could control to not configure these two logging in the same UE at the same time period.

Observation 3: Network to control not to configure logged MDT measurement and the logged AI/ML training data in the same UE at the same time period.

Proposal 9: (RRC-14) If the UEInformationResponse message includes the logged AI/ML training data, SRB6 will be used irrespective whether other information (e.g. SON/MDT report) is included in the same message.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the observation and proposals are summarized as follows:

- Mechanism related to NW side data collection

Observation 1: For future AI mobility use cases consideration, the legacy RRM framework should be used as baseline for specification enhancement.

Proposal 1: (RRC-13) Use RRM framework for AI/ML NW side training data collection. The network configures data logging trigger conditions and availability indication reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

Proposal 2: Parameters of l3-Threshold, hysteresis and timeToTrigger can be configured for event-triggered training data logging, similar as eventL1 in logged MDT configuration.

Proposal 3: (RRC-8) Network configures multiple percentages for buffer threshold and configures separate indications for buffer full and low power.

Proposal 4: Support event-triggered periodic data logging to make a trade-off between the one-shot event based logging and the pure periodic logging.

Observation 2: The method for entries recorded for logged MDT could be reused for the timestamp of training data collection.

Proposal 5: The report of collected training data includes:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index similar as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

Proposal 6: (RRC-12) Use CGI to identify the cell ID in each entry of training data collection.

- Memory

Proposal 7: Set the minimum memory size of AI/ML training data logging to be 64KB.

- Priority

Proposal 8: SRB6 is configured with lower priority than SRB1 for AI/ML NW side data collection reporting for Beam Management use case.

Observation 3: Network to control not to configure logged MDT measurement and the logged AI/ML training data in the same UE at the same time period.

Proposal 9: (RRC-14) If the UEInformationResponse message includes the logged AI/ML training data, SRB6 will be used irrespective whether other information (e.g. SON/MDT report) is included in the same message.

- References

- R2-2503302, RAN2#129bis Meeting Report