3GPP TSG RAN WG2 #130                                                                                   R2-2503593

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT, Turkcell

Title:	Discussion on data collection for AIML mobility

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis meeting, LCM and spec impact for AI/ML mobility was discussed, and the following agreements about data collection were achieved [1].

Agreement 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management

Agreements 4 As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework. 5 Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

In this contribution, we mainly focus on the discussion on data collection for AI mobility, including data collection on RRM measurement prediction for both NW-sided model and UE-sided model and data collection on measurement event prediction for UE-sided model.

- Discussion

  - RRM measurement prediction

In this section, we discuss the data collection on RRM measurement prediction for both NW-sided model and UE-sided model respectively.

NW-side data collection

In RAN2#129bis meeting, it was agreed that NW-side data collection procedures, configuration and reporting framework of AI/ML PHY can be used as the baseline for NW-side data collection of AI mobility. We discuss NW-side data collection for RRM measurement prediction from following aspects.

- Data collection configuration

In AI/ML PHY, the data collection configuration for NW-side data collection is under discussion, i.e., whether legacy L1 CSI framework or legacy L3 RRM measurement framework is used for NW-side data collection for beam management use case. For RRM measurement prediction, it is nature to use legacy L3 RRM measurement framework for NW-side data collection configuration. The measurement resources configuration for NW-side data collection can be provided in e.g., MeasObjectNR, and the data logging and reporting configuration can be configured in e.g., reportConfigNR associated with a MeasId that is linked to the corresponding MeasObjectNR without RRM measurement reporting.

Proposal 1: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide NW-side data collection configuration from network to UE.

- Logging conditions/events

In AI/ML PHY, the logging conditions/events for NW-side data collection was discussed and the following agreements were achieved.

Agreements on logging conditions/events - Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported. - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

These agreements above about logging conditions/events are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 2: The following agreements on logging conditions/events for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- Periodic logging is supported;

- Event-triggered logging is supported. Support the use of L3 measurement event trigged (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE perform logging or not.

- Content of collected data

In AI/ML PHY, the following agreement was achieved for the content of collected data for beam management use case.

Agreements on content of collected data 1 For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress.

For RRM measurement prediction, the following sub-use cases are captured in the latest TR.

3 sub-use cases are considered for cell-level RRM measurement prediction: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s). 3 sub-use cases are considered for beam-level RRM measurement prediction: - Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated; - Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s); - Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

For the content of collected data for RRM measurement prediction, it is up to the sub-use case, e.g.,

- for sub-use case 1/4, at least includes L1 beam level measurement result(s) and/or beam-IDs;

- for sub-use case 2, at least includes L3 cell-level measurement result(s) and/or cell-IDs;

- for sub-use case 3, at least includes L1 beam level measurement result(s) and/or beam-IDs and/or L3 cell-level measurement result(s) and/or cell-IDs;

- for sub-use case 5, at least includes L3 beam level measurement result(s) and/or beam-IDs;

- for sub-use case 6, at least includes L1 beam level measurement result(s) and/or beam-IDs and/or L3 beam level measurement result(s).

Proposal 3: For RRM measurement prediction, for NW-side data collection, at least cell/beam level measurement result(s) and/or cell/beam-IDs needed to be collected by UE, the detailed contents are up to the specific sub-use case.

- Starting/Stopping of logging

In AI/ML PHY, it was agreed that data collection is controlled by the network, the UE will not autonomously stop when low power state is detected. Also, when UE reaches its buffer limitation, the UE stops measurement for data collection purposes and logging.

Agreements on starting/stopping of logging - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

The agreements above about starting/stopping of logging are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 4: The following agreements on starting/stopping of logging for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Availability indication

In AI/ML PHY, about availability indication from UE to network, the following agreements were achieved.

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

The agreements above about availability indication are also appropriate for NW-side data collection for RRM measurement prediction use case, an issue needs to be considered is whether extra indication information is needed separate from AI/ML PHY.

Proposal 5: The following agreements on availability indication for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction. FFS whether extra indication information is needed separate from AI/ML PHY.

- Availability indication can be triggered due to:

- Full buffer being reached (if configured)

- Buffer threshold being reached (if configured).

- Low power (if configured)

- The UE send a UAI that indicates:

- Data is available

- Reason for trigger (full buffer, threshold)

- Low power indication

- Sending of the collected data

In AI/ML PHY, it was agreed that on-demand reporting of the logged measurements will be specified, and the following agreements were achieved.

Agreements on sending of the collected data - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - New SRB can be configured for NW-side data collection (with lower priority)

The agreements above about sending of the collected data are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 6: The following agreements on sending of the collected data for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- New SRB can be configured for NW-side data collection (with lower priority)

- Handling of configuration/data during mobility/state transition

In AI/ML PHY, how to handle the configuration/data during mobility/state transition was discussed and the following agreements were achieved.

Agreements on configuration/data handling during mobility/state transition - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

The agreements above about configuration/data handling during mobility/state transition are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 7: The following agreements on configuration/data handling during mobility/state transition for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).

- Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data.

UE-side data collection

- UE requests data collection

In AI/ML PHY, for UE-side data collection, the following agreements were achieved for UE requests data collection in RAN2.

Agreements on UE requests data collection - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

The agreements above about UE requests data collection are also appropriate for UE-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 8: The following agreements on UE requests data collection for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- The UE can request measurement configuration for data collection. The request can contain one or more of the following:

- An indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

- Data collection configuration

In AI/ML PHY, for UE-side data collection, the following agreements were achieved for data collection configuration in RAN2.

Agreements on data collection configuration - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

The agreements above about data collection configuration are also appropriate for UE-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 9: The following agreements on data collection configuration for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

In RAN1, it has been agreed that for UE sided model, the CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report i.e., L1 CSI framework is used for UE-side data collection configuration for AI/ML based beam management.

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Similarly, legacy L3 RRM measurement framework can be used for UE-side data collection configuration without measurement reporting.

Proposal 10: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide UE-side data collection configuration from network to UE.

  - Measurement event prediction

In this section, we discuss UE-side data collection for measurement event prediction involving both indirect measurement event prediction and direct measurement event prediction.

Indirect measurement event prediction

In indirect measurement event prediction, considering that the RRM measurement prediction (temporal, frequency, and spatial domain RRM prediction) model is used, the UE-side data collection for training including e.g., UE requests data collection and data collection configuration etc. for indirect measurement event prediction are same as RRM measurement prediction.

Proposal 11: For indirect measurement event prediction, UE-side data collection, which including e.g., UE requests data collection and data collection configuration etc. is the same as that for RRM measurement prediction.

Direct measurement event prediction

Similar as UE-side data collection in RRM measurement prediction, UE requests data collection and data collection configuration were agreed in AI/ML PHY are also applied to direct measurement event prediction.

Proposal 12: The UE requests data collection and data collection configuration mechanism were agreed in AI/ML PHY are also applied to direct measurement event prediction.

Similar as RRM measurement prediction, for direct measurement event prediction, legacy L3 RRM measurement framework can be used for UE-side data collection configuration without measurement reporting.

Proposal 13: Legacy L3 RRM measurement framework can be used for UE-side data collection configuration for direct measurement event prediction.

For the collected contents, we understand at least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, whether other information (e.g., UE location) is needed can be further discussed.

Proposal 14: For UE-side data collection on direct measurement event prediction, at least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, other information is FFS.

- Conclusion

According to the discussion in section 2, we propose:

RRM measurement prediction

NW-side data collection

Proposal 1: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide NW-side data collection configuration from network to UE.

Proposal 2: The following agreements on logging conditions/events for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- Periodic logging is supported;

- Event-triggered logging is supported. Support the use of L3 measurement event trigged (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE perform logging or not.

Proposal 3: For RRM measurement prediction, for NW-side data collection, at least cell/beam level measurement result(s) and/or cell/beam-IDs needed to be collected by UE, the detailed contents are up to the specific sub-use case.

Proposal 4: The following agreements on starting/stopping of logging for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

Proposal 5: The following agreements on availability indication for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction. FFS whether extra indication information is needed separate from AI/ML PHY.

- Availability indication can be triggered due to:

- Full buffer being reached (if configured)

- Buffer threshold being reached (if configured).

- Low power (if configured)

- The UE send a UAI that indicates:

- Data is available

- Reason for trigger (full buffer, threshold)

- Low power indication

Proposal 6: The following agreements on sending of the collected data for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- New SRB can be configured for NW-side data collection (with lower priority)

Proposal 7: The following agreements on configuration/data handling during mobility/state transition for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).

- Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data.

UE-side data collection

Proposal 8: The following agreements on UE requests data collection for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- The UE can request measurement configuration for data collection. The request can contain one or more of the following:

- An indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Proposal 9: The following agreements on data collection configuration for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

Proposal 10: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide UE-side data collection configuration from network to UE.

Measurement event prediction

Indirect measurement event prediction

Proposal 11: For indirect measurement event prediction, UE-side data collection, which including e.g., UE requests data collection and data collection configuration etc. is the same as that for RRM measurement prediction.

Direct measurement event prediction

Proposal 12: The UE requests data collection and data collection configuration mechanism were agreed in AI/ML PHY are also applied to direct measurement event prediction.

Proposal 13: Legacy L3 RRM measurement framework can be used for UE-side data collection configuration for direct measurement event prediction.

Proposal 14: For UE-side data collection on direct measurement event prediction, at least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, other information is FFS.

- References

- R2-2503302, RAN2_129bis_Meeting_Report