3GPP TSG RAN2 Meeting #130                                                                R2-2503604

19th May – 23rd May 2025, St.Julians, Malta

Agenda item:	8.4.3

Source:	Samsung

Title:	RRM measurement relaxation and offloading in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

1 Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

And during the previous meeting #127bis~129bis, with respect to RRM measurement relaxation and offloading agenda, RAN2 agreed as follows:

#127bis - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. #128 The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. #129 The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds. #129bis RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. How to define LP-RSRP and LP-RSRQ is up to RAN1. The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

In this contribution, we would like to discuss the detailed RRM measurement relaxation and offloading methods to support LP-WUS in RRC IDLE/INACTIVE mode.

2 Discussion

  - Working Assumption to be confirmed

Building upon the discussions from the previous RAN2 #127bis meeting, RAN2 has established the working assumption that require clarification and confirmation. Below, we outline these assumptions and propose refinements for further deliberation.

#127bis - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Here we propose to reaffirm and refine the existing working assumption regarding neighbor cell measurement relaxation for UEs capable of LP-WUS, especially with FFS on UE with low mobility.

While the original WA effectively addresses key aspects, it does not account for scenarios where rapid movement within a cell could lead to suboptimal behavior. Specifically, reliance on measurement-based thresholds alone may result in inefficient operation for high-speed UEs, such as those on vehicles or trains. These UEs frequently toggle between serving cell measurement offloading/relaxation and regular MR paging, leading to ping-pong effects and increased energy consumption.

To mitigate this issue and enhance efficiency, we propose integrating a "low mobility" criterion as an optional feature that networks can configure based on specific deployment requirements. This refinement aims to optimize LP-WUS operations by reducing unnecessary power consumption and improving overall system performance.

Proposal 1: Discuss and confirm the existing ‘working assumption’ of RAN2#127bis on neighbour cell RRM measurement relaxation, as follows:

- Original WA: For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’

- Proposal: For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

By explicitly including the "low mobility" criterion, networks can better tailor their configurations to suit specific deployment scenarios, thereby improving efficiency and reducing unnecessary power consumption during LP-WUS operations.

And for the FFS on enhancement of LR measurement, we would like to extend the criteria as follows:

Proposal 2: For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, the LR-based UE not at cell edge and UE low mobility are supported.

Furthermore, we may extend the discussion of including ‘low-mobility’ to ‘serving cell measurement offloading’.

Proposal 3: For Rel19 serving cell measurement (full) offloading for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

  - Reducing the # of Thresholds

According to the previous discussion of merging serving cell and neighbour cell measurement offloading in RAN2, now the LP-WUS capable UE has the below three functionalities:

- (1) Serving cell measurement offloading,

- (2) LP-WUS monitoring,

- (3) RRM measurement relaxation.

And there are further proposals as follows:

- Opt. A) to merge (1) and (2),

- Opt. B) to merge (2) and (3),

- Opt. C) to merge (1), (2), and (3) all together,

- Opt. D) not to merge.

Firstly, we believe the use case and coverage of (1) full offloading and (3) RRM relaxation should be different. So we suggest to rule out Opt. C).

Then, now we would like to discuss the use case of merging (2) LP-WUS monitoring, with other options

Considering option A), If (2) LP-WUS monitoring is combined with (1) full offloading, paging enhancements would only occur alongside serving cell offloading within the central area of the cell, confined to a limited small coverage. Logically, there is no issue if a UE performing full offloading of MR serving cell measurements can also conduct LP-WUS-based paging monitoring. However, such the UE would not perform paging enhancements (LP-WUS monitoring) in areas where serving cell offloading is not feasible. This limitation could result in reduced power-saving benefits due to the restricted ability to receive LP-WUS-based paging notifications.

Considering option B), If (2) LP-WUS monitoring is combined with (3) RRM relaxation, paging enhancements would occur alongside RRM relaxation, resulting in a significantly larger coverage area compared to (1) full offloading. This would allow the UE to benefit from both RRM relaxation and paging enhancements across a broader region. However, it remains uncertain whether a UE capable of RRM relaxation would also be able to perform LP-WUS reception-based paging enhancements, as these are distinct operations.

The LP-WUS monitoring is a power-saving mechanism specifically designed for paging. Therefore, it differs from RRM measurement relaxation or offloading, which involve skipping or stopping MR measurements to conserve power. Now, the question becomes:

- Can the paging enhancement mechanism be merged with the measurement relaxation/ offloading mechanism?

If the answer is simply "no," we should support Option D), which advocates against merging. The primary advantage of merging (2) and (1) or (3) would merely be the reduction in the number of thresholds, with no additional benefits. Based on this reasoning, we currently prefer the option D.

Proposal 4: For Rel19 LP-WUS, no additional merging of thresholds or functions is required.

3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: Discuss and confirm the existing ‘working assumption’ of RAN2#127bis, on neighbour cell RRM measurement relaxation, as follows:

- Original WA: For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’

- Proposal: For Rel19 serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Proposal 2: For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, the LR-based UE not at cell edge and UE low mobility are supported.

Proposal 3: For Rel19 serving cell measurement (full) offloading for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Proposal 4: For Rel19 LP-WUS, no additional merging of thresholds or functions is required.

References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC