3GPP TSG-RAN WG2 Meeting #130		R2-2503615

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on LP-WUS/WUR in RRC_CONNECTED for RAN2 as below listed from [1] and [2].

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-3 | whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group Editor’s NOTE: FFS on whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-4 | value range for offset UAI for LP-WUS monitoring for option 1-1 and option 1-2 Editor’s NOTE: FFS on the value range for offset UAI for LP-WUS monitoring for option 1-1 and option 1-2. | Issue Type: essential How to address it: can be easily addressed based on RAN1 inputs |
| RRC-5 | whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2 Editor’s NOTE: There is no conclusion on whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| Editor’s NOTE: FFS whether the maintenance of lpwus_PDCCHMonitoringTimer is per DRX group or per MAC entity. Editor’s NOTE: FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. Editor’s NOTE: FFS whether lpwus_PDCCHMonitoringTimer is configured per DRX group or common to DRX groups. |  |  |  |

- Discussion

  - LP-WUS operation for UE configured with dual DRX(RRC-3)

In Rel-16, secondary DRX group configuration is introduced for one cell group. RAN2 concluded that the DCP could not be configured when the secondary DRX group is configured for simplicity since there is not enough time in Rel-16 to discuss the coexistence issue [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

While for LP-WUS with dual DRX group, RAN1 also discussed this issue and concluded the coexistence between dual DRX with LP-WUS, while there is no consensus on the detailed implementation shown as the following conclusion in RAN1#120bis [4].

Conclusion For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

With this, we understand RAN2 could confirm LP-WUS could coexist with dual DRX group for both Option 1-1 and Option 1-2. Furthermore, RAN1 has converged to the above two alternatives. In our view, RAN2 could further discuss the above two implementations alternatives based on what have been concluded from RAN1.

It should be noted that the dual DRX is introduced due to the serving cells in FR2 have different data requirements from the serving cells in FR1 of the same CG, to match the different data requirements within the dual DRX group. Finally, the length of drx-onDurationTimer and drx-InactivityTimer are agreed to be different for the two DRX groups.

Observation 1: Data requirements may be different between primary DRX group and secondary DRX group.

Both alternatives above support configuring two set of LP-WUS configurations for primary DRX group and secondary DRX group in one CG: one LP-WUS is configured on PCell for primary DRX group to control the PDCCH monitoring in primary DRX group, and the other LP-WUS is configured on an activated SCell for secondary DRX group to control the PDCCH monitoring in secondary DRX group. The only difference between these two alternatives is that Alt1 defines the basic capability which requires one set of LP-WUS to control the PDCCH monitoring of the two DRX groups, additionally. In our view, there is no need to introduce the basic capability since generally the date requirements are different between two DRX groups. Besides it requires more design and impacts to support the basic capability. While only Alt2 could also achieve the same PDCCH monitoring behaviour as the basic capability to send the LP-WUS of primary DRX group and secondary DRX group simultaneously. Thus, we propose:

Proposal 1: (RRC-3) For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX.

  - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group.

  - LP-WUS indication for PDCCH monitoring is applicable to all serving cells in the DRX group where LP-WUS is monitored.

Besides, since the length of drx-onDurationTimer and drx-InactivityTimer are different between two DRX groups, which means the data transmission duration between the two DRX groups are different, thus separate lpwus-PDCCHMonitoringTimer should also be configured, with the same logic.

Proposal 2: (RRC-3) lpwus-PDCCHMonitoringTimer is configured per DRX group.

  - Empty UAI on offset for option 1-1 and option 1- (RRC-5)

RAN2 agreed in RAN2#129bis meeting that UE could provide the preferred time offset via UAI signalling [5].

If configured, the UE can signal a preferred time offset via UAI signalling.

Regarding the issue whether it is allowed to report an empty UAI offset for LP-WUS monitoring for both option 1-1 and option 1-2. In our understanding, there is no need to restrict on UE couldn’t report an empty UAI offset, similar as legacy UAI report for power saving. In this case, it means UE has no preference for the time offset between LP-WUS and PDCCH monitoring.

Proposal 3: (RRC-5) It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2, which means UE has no preference on the time offset between LP-WUS and PDCCH monitoring.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality, as below.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID, current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 4: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, we want to clarify the following terms firstly:

- Enable for LP-WUS monitoring: Which is to enable the LP-WUS functionality while UE could use LP-WUS only if the LP-WUS functionality is enabled and is activated if there is some activation mechanisms for LP-WUS;

- Activation for LP-WUS monitoring: When LP-WUS function is enabled, it is possible for network to activate the LP-WUS monitoring, and in this case UE could monitor LP-WUS; otherwise if the LP-WUS is not enabled, the activation procedure for LP-WUS is not expected to happed;

- Deactivation for LP-WUS monitoring: After LP-WUS monitoring is activated, upon the corresponding deactivation mechanism is performed, UE stops using the LP-WUS functionality.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, and the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Besides, according to the following RAN1 agreement, it could be observed LP-WUS could be used if it is enabled, thus there is no other additional activation/deactivation mechanism for LP-WUS.

Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1

Proposal 5: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

  - CSI report behaviour within the Active time triggered by LP-WUS

For LP-WUS Option 1-2, in RAN2#128 meeting, we agreed that the drx-ondurationTimer is not started if Option 1-2 LP-WUS is configured and in this case, during the non-Active time configured by drx-onDurationTimer UE doesn’t report the semi-periodic CSI and report the periodic CSI/L1-RSRP based on network configuration [6], for example:

- If lpwus-TransmitOtherPeriodicCSI is configured with value true, UE reports periodic CSI;

- If lpwus-TransmitPeriodicL1-RSRP is configured with value true, UE reports periodic L1-RSRP.

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Observation 2: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

However, we have not discussed the UE behaviour on CSI report during the Active Time triggered by LP-WUS Option 1-2. According to the current MAC running CR [7], UE behaviors on CSI report is the same as other Active Time when UE in the C-DRX Active time, i.e., lpwus-PDCCHMonitoringTimer triggered by LP-WUS is running.

The current CSI report behavior within the drx-onDurationTimer and other C-DRX Active Time is as follows:

- In the Active time triggered by drx-onDurationTimer: UE report the periodic CSI on PUCCH, semi-periodic CSI on PUCCH and PUSCH;

- In the Active time triggered not but others than drx-onDurationTimer: UE report semi-periodic CSI on PUCSCH, and:

  - if CSI-mask is set to true, UE doesn’t report periodic CSI and semi-periodic CSI on PUCCH;

  - if CSI-mask isn’t set to true, UE report periodic CSI and semi-periodic CSI on PUCCH.

Since when LP-WUS Option 1-2 is configured, the drx-onDurationTimer is not started, then the Active Time triggered by LP-WUS is most frequently compared with Active Time caused by other triggers, thus we think the LP-WUS triggered C-DRX Active Time is more similar to the legacy drx-onduraionTimer rather than C-DRX Active Time caused by other triggers. And the only difference of CSI report between the drx-onduraionTimer and other C-DRX Active Time is in case CSI-mask is set to true.   Accordingly, the CSI/L1-RSRP reporting behaviour in LP-WUS triggered C-DRX Active Time should be same as the legacy drx-ondurationTimer. Specifically, UE reports the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer triggered by LP-WUS is running. Otherwise, insufficient CSI reporting may cause the failure of data transmission during Active Time when CSI-mask is set to true.

Proposal 6: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

  - LP-WUS Option3

RAN1 has agreed to support LP-WUS Option 1-1 and Option 1-2, however, for Option 1-3, RAN1 didn’t have any conclusion on it. Since Option 1-3 is transparent to MAC, which only impacts the physical layers design, we think it is better to wait for RAN1 progress, if any. RAN2 should focus on Option 1-1 and Option 1-2 and won’t discuss Option 1-3 unless RAN1 triggers.

Proposal 7: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

- Conclusion

In this contribution, we discuss the remaining issues on LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observation 1: Data requirements may be different between primary DRX group and secondary DRX group.

Observation 2: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

LP-WUS operation for UE configured with dual DRX(RRC-3)

Proposal 1: (RRC-3) For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX.

  - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group.

  - LP-WUS indication for PDCCH monitoring is applicable to all serving cells in the DRX group where LP-WUS is monitored.

Proposal 2: (RRC-3) lpwus-PDCCHMonitoringTimer is configured per DRX group.

Empty UAI on offset for option 1-1 and option 1- (RRC-5)

Proposal 3: (RRC-5) It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2, which means UE has no preference on the time offset between LP-WUS and PDCCH monitoring.

Enable and disable for LP-WUS monitoring

Proposal 4: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 5: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

CSI report behaviour within the Active time triggered by LP-WUS

Proposal 6: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

LP-WUS Option3

Proposal 7: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

- References

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR

- R2-2503808, Report of [Post129bis][210][LPWUS] Running CR for TS 38.321 (Apple)

- 3GPP TS 38.331:"NR; Radio Resource Control (RRC) protocol specification".

- 3GPP RAN1#120bis meeting, Chairman notes

- R2_129b R18 MIMO R19 MIMO LPWUS SBFD NR_Others

- R2_128 R18 MIMO R19 MIMO LPWUS SBFD NR_Others