3GPP TSG-RAN WG2 Meeting #130		R2-2503619

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:		Discussion on conditional LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 bis meeting[1], a number of agreements related to conditional LTM were reached:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this contribution, we would like to discuss some remaining open issues, including:

- For L3 based RACH-less LTM, how to select cell/beam/CG resource;

- Raise the issue that what if the max number of maintained TA values exceeds the UE capability;

- Co-existence of LTM and CLTM;

- RACH-less procedure when no CG at CLTM execution;

- Co-existence of CLTM and (e)RedCap/ CovEn.

- Discussion

  - L3 based RACH-less CLTM (RRC-3/9)

For L3 based RACH-less CLTM, there are a lot of remaining issues to be solved. In L3 based CLTM, the CLTM execution is triggered when the execution condition is satisfied for at least one candidate cell, but after that, how to select the cell (in case multiple cells satisfied the execution conditions), the beam, and the valid CG resource to perform L3-based RACH-Less LTM?

According to the post email discussion on list of RRC FFSs for mobility[2], we have the following open issue:

RRC-3: Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

First, we would like to have the simple proposal on cell selection in L3 based CLTM, which we prefer to leave it to UE implementation.

Proposal 1: (RRC-3) For L3 based CLTM, when multiple candidate cells satisfy the CLTM execution conditions, it is up to UE implementation to select one of them to be the target cell.

After the cell selection, the next step is to select the proper beam. Unlike L1-based CLTM, the beam selection in L3-based CLTM may be more challenging. Shown as the MAC open issue in [4]:

| MAC-9 | In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM? Editor’s NOTE: FFS in case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

In Rel-18 L3 RACH-less handover, the mechanism is to select SSB based on configured threshold, as follows[3]:

### 5.8.2 Uplink <omitted…> For the uplink grant configured for configured grant Type 1 for RACH-less handover, if the configured uplink grant is valid according to TS 38.214 [7] for which the above formula is satisfied and RACH-less handover is not successfully completed, the MAC entity shall: - if the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33: <omitted…> 1> else if at least one SSB corresponding to the configured uplink grant with SS-RSRP above cg-RRC-RSRP-ThresholdSSB is available: 2> select an SSB with SS-RSRP above cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant; 2> indicate the selected SSB index to the lower layer; 2> consider this configured uplink grant as valid.

In our understanding, the similar mechanism can be reused in L3-based RACH-less CLTM, and the UE can select good beams based on threshold with valid configured grant, thus being able to perform RACH-less CLTM. If there is no such beam, then the UE may select a beam below configured RSRP threshold, then only RACH-based procedure can be initiated.

Proposal 2: (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the concerned beam is selected and CG-based RACH-less CLTM is performed.

Proposal 3: (MAC-9) If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

  - Max number of maintained TA values exceeds the UE capability

As agreed in the previous meeting[1], the max number of TA values a UE can maintain for CLTM is defined as a UE capability, with a value range of 1~8.

- Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

As we know, up to 8 LTM candidate cells can be configured for a UE. Thus, the max number of TA values that a UE can maintain may be lower than the number of configured LTM candidate cells.

If the UE has already reached its maximum capacity for maintaining TA values but receives an early TA MAC CE for a CLTM candidate cell whose CLTM TAT is not running, it will be unable to maintain this new TA value. RAN2 needs to discuss and resolve this issue to ensure that there is no mismatch between the UE and the network.

Proposal 4: RAN2 discusses how to handle the case where the maintained TA value number has reached the upper capability limit but an LTM Candidate Timing Advance Command MAC CE for a candidate cell whose TAT is not running is received from the network.

  - Co-existence of LTM and CLTM (MAC-14)

According to the current MAC running CR[4], there is an open issue about the coexistence of LTM and CLTM.

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

For this issue, we propose discussing the following two cases:

- Case 1: Reception of an LTM Cell switch Command MAC CE without valid TA

- Case 2: Reception of an LTM Cell switch Command MAC CE with valid TA

Discussion on case 1

According to the current MAC specification, if a UE receives an LTM Cell Switch Command MAC CE without valid TA, it performs RACH-based LTM. However, if the CLTM candidate TAT timer for the target cell is running, the UE should leverage its maintained TA to execute RACH-less LTM instead. This avoids unnecessary handover latency.

Proposal 5: (MAC-14) If a UE receives an (Enhanced) LTM Cell Switch Command MAC CE without valid TA and CFRA resource while the CLTM candidate TAT timer for the target cell is running, it shall perform RACH-less LTM.

Discussion on case 2

As agreed in the previous meeting[1], the C-LTM TAT timer for the target cell will not stop upon C-LTM cell switch execution (if running).

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

In our view, the C-LTM TAT for target cell should also not be stopped upon LTM cell switch execution triggered by the LTM Cell Switch Command MAC CE while it is running.

Furthermore, if the network sends an LTM Cell switch Command MAC CE with valid TA for the target cell, the UE should store the TA value for the target cell and the C-LTM TAT for the target cell should be started or restarted. This ensures that the target cell maintains an up-to-date TA value for the subsequent CLTM procedures.

Proposal 6: (MAC-14) If an LTM Cell switch Command MAC CE with valid TA is received, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

  - RACH-Less procedure with no CG

In RAN2 #129 meeting[5], the following agreement clarifies the UE’s behaviour to use RACH-Less procedure for CLTM at execution:

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

However, there was some discussions about what should be the UE’s behaviour when there is no CG occasion at the time of CLTM execution, considering now only CG-based RACH-Less is supported for CLTM.

In our understanding, the right condition for the UE to perform RACH-less C-LTM is to judge whether the TA timer is still running in the first available CG occasion for initial CG transmission. i.e., even the UE has a valid TA at the time of CLTM execution, if the TA timer is going to expire at the first available CG occasion, the UE should not perform RACH-less CLTM. This is already captured in running CR[6]:

2> if the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the Target Configuration ID for the TAG associated with the selected SSB or selected CSI-RS is running in the first available CG occasion for initial CG transmission according to clause 5.8.2 in case in case two TAGs are configured for the CLTM candidate cell: 3> process the stored Timing Advance Command associated with the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (see clause 5.2); 3> consider the RACH-less CLTM cell switch to be ongoing;

However, a question would be raised that, if the TA timer is running at the first available CG occasion, and the RACH-less CLTM begins, what the UE’s behaviour should be if the transmission on first available CG fails.

Observation 1: The UE’s behaviour is not clear when RACH-less CLTM begins but the transmission on CG occasion fails.

In that case, the UE should wait for the next available CG, and the UE should again to evaluate whether the TA timer is going to expire at the next available CG occasion. If that happens (i.e., TA timer is going to expire at the next available CG occasion), then the UE can:

- Option-1: to consider the RACH-less procedure failed and perform re-establishment;

- Option-2: to transfer to RACH-based CLTM.

In this case, as RACH-less procedure already started, we think option-1 should be easier. Moreover, with option-1, the UE may perform the fast recovery, so it may also end up with RACH-based LTM but without spec impact. Therefore,

Proposal 7: If UE performs RACH-LESS CLTM but transmission on following CG fails, the UE continue to perform RACH-LESS procedure if the TAT is still running at the next available CG occasion, otherwise the ongoing CLTM is considered failure.

  - Co-existence of CLTM and (e)RedCap/ CovEn (MAC-13)

According to the current MAC running CR[3], there is an open issue about the coexistence of LTM and CLTM.

| MAC-13 | FFS the coexistence between CLTM and (e)RedCap, CovEnh? Editor’s NOTE: Whether/How CLTM could co-exist with (e)RedCap is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. Editor’s NOTE: Whether/How CLTM could co-exist with CovEnh is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

In our understanding, the co-existence between CLTM and (e)RedCap/ CovEn has already been supported for both R18 intra-CU LTM and R19 inter-CU LTM. Considering the CLTM is just one kind of LTM configuration with extra execution condition configuration, if we are going to exclude the co-existence between CLTM and (e)RedCap/ CovEn, the spec impact would be foreseen. On the other hand, we don’t see the real concern to support the scenarios. Therefore, we propose:

Proposal 8: (MAC-13) Co-existence of Rel-19 CLTM and (e)RedCap is supported.

Proposal 9: (MAC-13) Co-existence of Rel-19 CLTM and coverage enhancement is supported.

- Conclusion

In this contribution, we discussed remaining issues on CLTM, including both RRC and MAC open issues.

We have the following Observations and Proposals:

For L3-based RACH-less CLTM:

Proposal 1: (RRC-3) For L3 based CLTM, when multiple candidate cells satisfy the CLTM execution conditions, it is up to UE implementation to select one of them to be the target cell.

Proposal 2: (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the concerned beam is selected and CG-based RACH-less CLTM is performed.

Proposal 3: (MAC-9) If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

For Max number of maintained TA values

Proposal 4: RAN2 discusses how to handle the case where the maintained TA value number has reached the upper capability limit but an LTM Candidate Timing Advance Command MAC CE for a candidate cell whose TAT is not running is received from the network.

For Co-existence of LTM and CLTM

Proposal 5: (MAC-14) If a UE receives an (Enhanced) LTM Cell Switch Command MAC CE without valid TA and CFRA resource while the CLTM candidate TAT timer for the target cell is running, it shall perform RACH-less LTM.

Proposal 6: (MAC-14) If an LTM Cell switch Command MAC CE with valid TA is received, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

For RACH-Less procedure with no CG

Observation 1: The UE’s behaviour is not clear when RACH-less CLTM begins but the transmission on CG occasion fails.

Proposal 7: If UE performs RACH-LESS CLTM but transmission on following CG fails, the UE continue to perform RACH-LESS procedure if the TAT is still running at the next available CG occasion, otherwise the ongoing CLTM is considered failure.

For Co-existence of CLTM and (e)RedCap/ CovEn

Proposal 8: (MAC-13) Co-existence of Rel-19 CLTM and (e)RedCap is supported.

Proposal 9: (MAC-13) Co-existence of Rel-19 CLTM and coverage enhancement is supported.

- References

- RAN2 #129bis meeting chair notes;

- R2-250xxxx- List of RRC FFSs for mobility_v00_Ericsson;

- 3GPP TS 38.321 V18.5.0 (2025-03);

- R2-253766, Discussion summary and list of MAC open issues for mobility Enhancements, vivo(rapporteur);

- RAN2 #129 meeting chair notes;

- R2-2503616_Running MAC CR for Mob Ph4_v19_Rapp2;