3GPP TSG-RAN WG2 #130	R2-2503636

Malta, May 19th– 23rd, 2025

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [4]. The core part objective relevant to OD-SIB1 based on the revised WID is:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify the corresponding core requirements, for the above features [RAN4].

During the RAN WG2 Meeting #129bis, the following agreements were reached for the on-demand SIB1:

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In Appendix A we also provide the complete list of all RAN2 agreements reached for on-demand SIB1 so far.

Additionally, we recall that according to the agreement from the latest WID RAN#105 [4], RAN2 should:

- Specify procedures and signaling method(s) for Case 2 [RAN1/2]:

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Furthermore, during RAN2#127bis meeting the following general agreement concerning the WI phase was made:

RAN WG2 Meeting #127bis

- We will inherit all agreements made during SI phase to WI phase.

With these two general guidelines in mind, in this paper, we discuss the remaining open questions concerning the following aspects:

- The open issue list identified in 38.331 and 38.304 specifications,

- The UL WUS configuration,

- On-demand SIB1 request,

- Impact on Random Access report,

- On-demand SIB1 and Automatic neighbour relation.

# 2	On the open issue lists

In this section, we discuss the questions identified as open issues by Rapporteurs of 38.331 and 38.304 specifications, respectively.

Discussion on 38.331:

The following issue has been identified in 38.331:

- Issue 1: Co-existence of SBFD and OD-SIB1

At this stage, we do not see any co-existence issues between SBFD and OD-SIB1.

Discussion on 38.304:

Based on post-meeting email discussion, Rapporteur identified the following two open issues in 38.304:

- Issue 1: FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

[Issue description] Some companies proposed a new UE behavior on handling excluded cell list. Their view is that NW may not configure Rel-19 excluded cell list if there is no excluded cell for NES UE in real deployment. In this case, they think that the NES UE also needs to ignore the legacy excluded cell list because NES Cell is included in legacy IE to bar the legacy UE. Thus, they propose that the NES UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

[Rapporteur view] Rapporteur is not convinced by this new UE behavior.

This case was never discussed in Rel-19. If capture it, we need RAN2 agreement.

From technique perspective, there are two issues:

It actually assumes NW will only include NES cells in legacy excluded cell list. We don’t think it is correct because it restricts NW implementation. We assume that legacy excluded cell list may include both NES cells and/or some normal cells (e.g. blacklist cells for all UE access). When legacy excluded cell list includes only blacklist cells for all UE, if NES UE ignores legacy excluded cell list, it will wrongly access blacklist cells.

As RAN2 agreed that legacy UE is barred via “no-SIB1” indication in MIB, the dedicated excluded cell list itself is an optimization. We don’t prefer to do optimization over optimization.

- Issue 2: FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

[Issue description] Some companies proposed to explicitly capture the failure case of OD-SIB1 window expiry in 38.304 according to RAN2#129b agreement. And they proposed to add the failure condition in 38.331.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

[Rapporteur view] After coordination with RRC CR rapporteur and MAC CR rapporteur, current conclusion is to make it FFS for further discussion due to below concern:

  - It seems to be obvious that the failure is after the window.

  - If we specify the window, then we may need to capture separately all the other cases why UE doesn’t receive SIB1.

Regarding Issue 1, our view is that there is no need to always ignore the excluded cell list. In the described case, the NES excluded cell list would simply include those regular cells and not the NES cells, which aligns with the original intent of introducing this list.

A UE supporting OID-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present.

Regarding Issue 2, we concluded in the 38.331 discussion that there is no need to separately mention the end of the window. Hence, we support the view summarized above by the Rapporteur of 38.304, i.e., that it is not needed to capture this in the specification.

Do not capture separately that the cell is barred if SIB1 is not received by the end of the window since the procedure already captures it in more general way.

# 3	UL WUS configuration

Different aspects related to the UL WUS configuration have been discussed in previous RAN2 meetings. However, there are several open questions, which we will discuss in this section.

The first question concerns the agreement from the RAN2#127bis meeting according to which a UE should check if SIB1 is currently broadcasted by the NES cell before requesting it:

RAN WG2 Meeting #127bis

Broadcast status for OD-SIB1

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

It is important to observe that in certain scenarios checking if SIB1 is being broadcasted or provided on-demand before requesting SIB1 may lead to unnecessary energy consumption on the UE side because a UE may wait unnecessarily long before concluding that it needs to request SIB1. Since the broadcasting is periodical, this issue can be addressed by informing a UE about the timing of the next tentative SIB1 occasion. This information can be provided in the UL WUS configuration, and the UE can use this information to determine during which period it needs to check for SIB1 before requesting on-demand SIB1, i.e., before transmitting the UL WUS. The advantage of this solution is twofold. First, it enables the UE to know when to monitor for a potential SIB1 transmission, which is beneficial in terms of UE energy savings since it creates an opportunity for the UE to sleep during periods when potential SIB1 transmissions are not expected. Second, it ensures that the UE only monitors for as long as necessary to comply with the RAN2#127bis agreement on the “broadcast status for OD-SIB1,” thereby minimizing on-demand SIB1 acquisition delay.

The UL WUS configuration includes the information on SIB1 timing according to which SIB1 is tentatively transmitted.

The second question concerns the validity of the UL WUS configuration. In what follows, we address this question in different cases, i.e., when the UE is in Cell A and when the UE is NES Cell.

Validity of the UL WUS configuration within Cell A:

In RAN2#127bis meeting it was agreed that the UL WUS configuration is provided in a new SIBxx:

RAN WG2 Meeting #127bis

NES Cell’s WUS configuration via existing SIB or new SIB

⦁	NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

While the UE is camping on Cell A that provides SIB1 according to legacy, the UE can rely on the legacy SI modification procedures to maintain a valid version of the UL WUS configuration. This means that a UE while camped on Cell A needs to ensure that it maintains a valid version of UL WUS configuration(s) by maintaining a valid version of Cell A’s SIBxx.

Cell A provides SIB1 according to legacy and allows legacy UEs to camp on it.

Validity of the UL WUS configuration obtained from Cell A within NES Cell:

The following agreement that is relevant to the question of the UL WUS configuration validity within the NES Cell was reached during the RAN WG2 Meeting #128:

RAN WG2 Meeting #128

Validity of UL WUS configuration

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

Regarding the above agreement, we observe that “directly succeeding cell reselection” does not have a temporal meaning, i.e., it does not have any meaning for how long the UL WUS configuration is valid. Our understanding of  “directly succeeding cell reselection” is that the UL WUS configuration that was obtained in Cell A is valid within a NES cell only if that NES cell is the cell that was reselected right after Cell A. Furthermore, our understanding is that the above agreement implies that if a UE leaves Cell A and later returns to it, the UE cannot take for granted that the stored UL WUS configuration is valid. Instead, upon the return to Cell A, the UE needs to ensure (e.g., via legacy SI procedures) that it has a valid UL WUS configuration before attempting to reselect a NES Cell.

In the agreement from RAN2#128 meeting on the “validity of UL WUS configuration”, “directly succeeding cell reselection” does not have a temporal meaning, e.g., it does not specify for how long the UL WUS configuration obtained in Cell A is valid after the UE performs the reselection from Cell A to NES Cell.

Furthermore, according to the following agreement from the RAN2#127bis, RAN2 prefers not to start the discussion on the question of how to ensure the synchronization of the UL WUS configuration updates between Cell A and NES Cell:

RAN WG2 Meeting #127bis

WUS Configuration update between cell A and NES cell:

⦁	RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Therefore, according to the above agreement,  if the validity of the UL WUS configuration obtained from Cell A is tied to the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A, then the UE would need upon the expiration of the SIBxx validity timer to reselect Cell A just to obtain an updated version of SIBxx from Cell A, and then reselect the NES Cell again. Clearly, this would lead to unnecessary cell reselections, and hence to unnecessary signaling, and energy consumption both on the UE and the network side.

If the validity of the UL WUS configuration obtained from Cell A is tied to the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A, then the UE would need upon the expiration of the SIBxx validity timer to reselect Cell A just to obtain an updated version of SIBxx from Cell A, and then reselect the NES Cell again.

Coupling the validity of the UL WUS configuration of a NES Cell obtained from Cell A to the Cell A’s SIBxx validity while the UE is camping in that NES Cell, introduces unnecessary cell reselections, and hence unnecessary signalling and energy consumption both on the UE and the network side.

Considering the existing RAN2 agreements and the above observations, we think that the validity of the UL WUS configuration obtained from Cell A should be decoupled from the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A.

The UE considers that the UL WUS configuration it obtained from Cell A is valid within the NES Cell that the UE reselected right after Cell A until the NES Cell itself provides an update concerning the UL WUS configuration, i.e., irrespective of the validity of the Cell A’s SIBxx that contains the UL WUS configuration.

Agree to adapt the TP1.

-----------------------------------TP1 start---------------------------------------------------------------------

5.2.2.4.2x	Actions upon reception of SIBxx

Editors notes: FFS depending SIBxx/UL WUS validity discussion details

Upon receiving SIBxx, the UE shall:

1>	if the UE has reselected to a cell providing OD-SIB1, UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell until receiving SIBxx from this cell:

-----------------------------------TP1 end-------------------------------------------------------------------------

UL WUS configuration in the case of multiple NES Cells:

In what follows we discuss the aspects related to the UL WUS configuration in the case of multiple NES Cells. The versions of Scenario 1a with multiple Cells A and NES Cells were discussed during the WG2 Meeting #126, and the following agreements were made:

RAN WG2 Meeting #126

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with NES Cell.

As highlighted in the agreement on “Further clarification”, the same Cell A may assist multiple NES Cells.  One of the remaining questions that needs to be addressed is whether the same UL WUS configuration can be used for multiple NES Cells. The benefit of using the same UL WUS configuration for multiple NES Cells is that it allows for minimizing the size of the message that carries the UL WUS configuration(s). However, using different UL WUS configurations for different NES Cells may be needed if Cell A assists multiple NES Cells that are in different bands (e.g., NES Cells in FR1 and FR2) or have different deployment characteristics such as different cell sizes. The benefit of using different UL WUS configurations is that it allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells. Furthermore, in order to avoid waking up multiple NES cells by the same UL WUS configuration/transmission, distinguished UL WUS configurations for various NES cells are favourable. Therefore, in order to achieve the balance between the message size, unnecessary invocations in multi-NES-Cell deployments, and configuration flexibility, the NW should be able to decide depending on a concreate scenario whether to use the same or different UL WUS configurations for multiple NES Cells.

Furthermore, the following agreement was made during the WG2 Meeting #129bis meeting:

RAN WG2 Meeting #129bis

Agreements on OD-SIB1

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups and allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

It is beneficial to support configuration flexibility in indicating whether the same UL WUS configuration applies to a single NES Cell or multiple NES Cells, as needed.

Related to this topic, RAN1 made the following agreements in RAN#117 and RAN#118bis meetings, respectively:

RAN1#117

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

RAN1#118bis

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

RAN1#120bis

Agreement

In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion.

Given the above RAN1 and RAN2 agreements, we observe that the configuration flexibility in indicating whether the same UL WUS configuration applies to a single NES Cell or multiple NES Cells can be supported by associating the UL WUS configuration with a list of NES-CellId parameters. Note that according to RAN1, NES-CellId is defined by PhysCellId and ARFCN-ValueNR. Therefore, to address the “FFS Whether the UL-WUS configuration applies to one cell or several cells” that was raised in [6], we propose the following:

The NES-CellId list is included in the UL WUS configuration and can be used to support configuration flexibility in indicating whether the same UL WUS configuration applies to a single NES Cell or multiple NES Cells.

Cell access and barring related information and UL WUS configuration

One of the remaining issues that were left to discuss after the RAN2#129bis meeting was whether to include the cell access related information in the UL-WUS configuration. 	In the RAN1#120 meeting, it was agreed that cell barring information, among other parameters (see the Appendix B for details), is not to be included in the UL-WUS configuration. In our view, including cell access related information in the UL-WUS configuration is not desirable, as it would lead to two unfavourable consequences. First, it would increase inter-node information exchange during updates, thereby adding unnecessary complexity to the feature. Second, due to the nature of cell access-related information, it would result in more frequent SIBxx updates. Both outcomes are counterproductive to network energy savings—particularly considering the agreement from RAN2#127 meeting according to which the NES cell itself is expected to provide UL WUS configuration updates, once Rel-19 NES UE camps on the NES cell.

Another important aspect to consider is that to know and decide, what exact parameters should be included in the UL-WUS configuration and how feasible these additions are, requires more effort and discussion that a negligible RAN2 online and offline discussion. These additional discussions and studies require additional time, and in consideration with the Release 19 coming to an end, are not worth the effort at this point. Therefore, considering all the above discussions, the cell access related information should not be included in the UL-WUS configuration.

If the cell access related parameters are to be added to the UL-WUS configuration, SIBxx would need more frequent updates which is suboptimal in many deployments.

The more parameters are included in the configuration, the more complicated becomes the inter-node coordination/information exchange.

Do not support including the cell access related information into the UL-WUS configuration.

# 4	On-demand SIB1 request

In the RAN1#120bis, the following question was raised as a note in [8]:

“Note: whether/how to address SIB1 request for connected UEs including the need of capability signaling is up to RAN2”

In our opinion, there is no need to consider the capability signalling for connected UEs, as the OD-SIB1 is considered for the UEs in the connected mode only in the RLF case as agreed in the RAN plenary.

No need to consider the capability signalling for connected UEs.

# 5	Impact on Random Access report

Another important feature that may be affected by on-demand SIB1 is Random Access (RA) report, which comprises vital information for RACH optimization. Since on-demand SIB1 procedure uses RA procedure, this case should be supported. One of the mandatory IEs of RA report is RA purpose, e.g., “access related”. One issue is that none of the existing values for RA purpose covers the case where RA procedure was used for request for SIB1. Note that the existing value “requestForOtherSI” specifies that RA procedure was used to request other SI, i.e., SIB2, SIB3, etc. SIB1, however, is a part of the so-called minimum SI (the MIB in the SSB is the other). Hence the value does not qualify for being reused.

None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

With this issue, the introduction of on-demand SIB1 leaves RA report ill-defined, and the UE behaviour unclear. Hence, the issue must be resolved in Rel-19.

The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Fortunately, there are many spare values for RA purpose. A simple solution could be to add a new value, e.g., “requestForSIB1” (the name is FFS). We propose RAN2 to discuss the impact of on-demand SIB1 on RA report and consider the addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

# 6 On-demand SIB1 and Automatic Neighbour Relation

One of the important features that may be affected by on-demand SIB1 is Automatic Neighbour Relation (ANR). Introduction of on-demand SIB1 affects the ANR in a way that NES Cell with on-demand SIB1 cannot be discovered by the ANR. According to the current specification (c.f., Section 15.3.3 of 38.300 [3]), if a cell is not broadcasting SIB1, the UE aborts the Cell Global Identity (CGI) reporting procedure and reports "noSIB1":

“In case the detected NR cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331 [2].”

Therefore, if a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally. Furthermore, if the coexistence of on-demand SIB1 and ANR is not addressed, operators may need to manually provision and manage NES cells with on-demand SIB1 in the neighbour relation tables. Since this is not a practical and desirable solution, there is a need for new means that would allow for a functional coexistence of on-demand SIB1 and ANR.

If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

One solution to support the coexistence of on-demand SIB1 and ANR could be that the NW configures the UE whether CGI-Info is to be captured for NES Cells with on-demand SIB1. Another solution could be that CGI-Info (or parts of it) of the NES cell is included in the UL WUS configuration such that the UE already knows or only needs to capture the UL WUS configuration and hence avoids asking for SIB1 for the sake of ANR. The later may be possible if the UE can obtain the UL WUS configuration. In the following, we discuss how to address the problem if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell. We refer to this problem as the problem of a missing UL WUS.

The problem of a missing UL WUS configuration is illustrated in Figure 1, and it occurs in a scenario where a part of the coverage area of the NES Cell is not covered by a Cell A, but it is covered by another cell, here denoted Cell X. The scenario depicted in Figure 1 can occur in reality, for various reasons. For example, if the coverage area of a cell has a very irregular shape which is typical e.g., in cities, due to various obstructions or if the coverage area of a cell changes over time, e.g., due to Coverage and Capacity Optimization (CCO). In these cases, the UE can find the NES Cell, but it cannot find (any) Cell A (i.e., the UE cannot camp on the NES Cell). Therefore, the problem of a missing UL WUS configuration may lead to additional issues in the context of the coexistence of on-demand SIB1 and ANR.

Figure 1: An example of the scenario in which the missing UL WUS configuration problem occurs.

The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

The above observations imply that there is a need to support the detection of problems with missing UL WUS configuration and other similar problems with on-demand SIB1 provisioning. On the one hand, there is a need to support the means that would allow the NES Cell to identify itself as a NES Cell without having to rely on a Cell A. For example, the means that would allow a UE to detect that a cell is a NES Cell (i.e., that is using on-demand SIB1 transmission) from the SSB/MIB of the cell itself. On the other hand, there is a need for means that would allow a UE to inform the NW about problems with provisioning the on-demand SIB1 and the UL WUS configuration.

A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Related to this aspect, RAN1 agreed the following during the RAN1#118bis meeting:

RAN1#118bis

Agreement

UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A

According to the above agreement from RAN1, a UE can use at least the UL WUS configuration from Cell A to determine a cell with on-demand SIB1. It is important to observe that if a UE relies only on Cell A when determining a cell supporting on-demand SIB1 (as captured in the RAN1 agreement) a UE may not be able to detect faulty configuration if a Cell A has not been visited (as illustrated in Figure 1). Since the RAN1 agreement does not exclude the other means that would allow a UE to determine a cell supporting on-demand SIB1 (i.e., the agreement states that “at least UL WUS configuration from Cell A” is used), we propose the following.

RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

# 7	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	At this stage, we do not see any co-existence issues between SBFD and OD-SIB1.

Observation 2	Cell A provides SIB1 according to legacy and allows legacy UEs to camp on it.

Observation 3	In the agreement from RAN2#128 meeting on the “validity of UL WUS configuration”, “directly succeeding cell reselection” does not have a temporal meaning, e.g., it does not specify for how long the UL WUS configuration obtained in Cell A is valid after the UE performs the reselection from Cell A to NES Cell.

Observation 4	If the validity of the UL WUS configuration obtained from Cell A is tied to the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A, then the UE would need upon the expiration of the SIBxx validity timer to reselect Cell A just to obtain an updated version of SIBxx from Cell A, and then reselect the NES Cell again.

Observation 5	Coupling the validity of the UL WUS configuration of a NES Cell obtained from Cell A to the Cell A’s SIBxx validity while the UE is camping in that NES Cell, introduces unnecessary cell reselections, and hence unnecessary signalling and energy consumption both on the UE and the network side.

Observation 6	Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Observation 7	Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups and allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both ES cells in FR1 and FR2, different cell sizes, etc.).

Observation 8	It is beneficial to support configuration flexibility in indicating whether the same UL WUS configuration applies to a single NES Cell or multiple NES Cells, as needed.

Observation 9	If the cell access related parameters are to be added to the UL-WUS configuration, SIBxx would need more frequent updates which is suboptimal in many deployments.

Observation 10	The more parameters are included in the configuration, the more complicated becomes the inter-node coordination/information exchange.

Observation 11	None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

Observation 12	The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Observation 13	If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Observation 14	The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Observation 15	Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

Observation 16	A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

Observation 17	There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Based on the discussion in the previous section, we propose the following:

Proposal 1	A UE supporting OID-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present.

Proposal 2	Do not capture separately that the cell is barred if SIB1 is not received by the end of the window since the procedure already captures it in more general way.

Proposal 3	The UL WUS configuration includes the information on SIB1 timing according to which SIB1 is tentatively transmitted.

Proposal 4	The UE considers that the UL WUS configuration it obtained from Cell A is valid within the NES Cell that the UE reselected right after Cell A until the NES Cell itself provides an update concerning the UL WUS configuration, i.e., irrespective of the validity of the Cell A’s SIBxx that contains the UL WUS configuration.

Proposal 5	Agree to adapt the TP1.

Proposal 6	The NES-CellId list is included in the UL WUS configuration and can be used to support configuration flexibility in indicating whether the same UL WUS configuration applies to a single NES Cell or multiple NES Cells.

Proposal 7	Do not support including the cell access related information into the UL-WUS configuration.

Proposal 8	No need to consider the capability signalling for connected UEs.

Proposal 9	RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

Proposal 10	Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

Proposal 11	RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Proposal 12	Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

RAN1#120 agreements:

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TS 38.331 V18.1.0, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification, Release 18.

3GPP TS 38.300 V18.1.0, Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description, Release 18.

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN105, Melbourne, Australia, September 2024.

RP-243297, “Way forward on NES”, RAN106, December 2024.

R2-2502150, “Comments to 38.331 CR for NES”

R2-2502173, “Running RRC CR for enhancements for network energy efficiency”

R1-2502961, “Session Notes of AI 9.15.4”, Ad-Hoc Chair (NTT DOCOMO)Wuhan, China, April 7th-11th, 2025.

# Appendix A

### RAN WG2 Meeting #125bis

1.	At least RAN2 starts scenario 1a. Other scenarios are not excluded.

a. 	Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts:

i.	Add WUS configuration in SIB of cell A.

ii.	Cell reselection from cell A to NES cell, including trigger condition and cell barring changes.

iii.	Whether allow camping, paging and SIB update in NES cell.

iv.	Cell reselection from NES cell to cell A or normal cell.

2.	RAN2 assumes that RACH procedure is reused for UE to request on-demand SIB1.

3.	UL WUS configuration includes at least below information:

a.	RACH configuration

4.	A UE needs to know a UL WUS configuration to request SIB1 of which cell.

5.	Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

6.	The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

### RAN WG2 Meeting #126

1.	Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

a.	More than one Cell A may provide configuration for the same NES cell.

b. 	The same Cell A may assist more than one NES Cells.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

3.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

4.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

5.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

6.	If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

7.	After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

8.	RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

9.	NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

10.	How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

11.	If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

12.	After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

13.	RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

14.	As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

15.	Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

16.	RAN2 to wait for RAN1’s progress whether to support scenario 3.

### RAN WG2 Meeting #127

1.	No consensus for RAN1 case 3 in RAN2.

2.	We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3.	Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

5.	Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

a.	Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

b.	Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

c.	Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

7.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

8.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

### RAN WG2 Meeting #127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

### RAN WG2 Meeting #128

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

### RAN WG2 Meeting #129

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

### RAN WG2 Meeting #129bis

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

# Appendix B

- RAN1#120 agreements:

R1-2501380	FL summary 3 for on-demand SIB1 in idle/inactive mode        Moderator (MediaTek)

Conclusion

It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

Conclusion

- The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system.

- RA-RNTI is not included in the UL-WUS configuration.

- Cell barring information is not included in the UL-WUS configuration.

- Cell selection information is not included in the UL-WUS configuration.

- Accessible UE types is not included in the UL-WUS configuration.

- CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration.

- InitialUplinkBWP is not included in the UL-WUS configuration.

- TotalNumberOfRA-Preambles is not included in the UL-WUS configuration.

- SIB1-RequestResourceRedCap is not included in the UL-WUS configuration.

- SupplementaryUL is not included in the UL-WUS configuration