3GPP TSG-RAN WG2 Meeting #130	R2-2503637

Malta, 19 - 23 May, 2025

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect data to train AI model. In this contribution, we discuss open issues of NW-side data collection.

Discussion

RRC Open issues

Issue 18

This issue is about how to indicate UE to retain or release logged data during HO. Two options were proposed:

o	the source gNB sends the 1-bit indication to the UE before HO.

o	the source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB.

Even in intra-vendor case option 2 may still have RAN3 impact, since source gNB may need to indicate the data release indication to target gNB. Option 2 doesn’t work in inter-vendor handover, while the main concern of data exposure is on inter-vendor handover case. In inter-RAT handover, option 2 would result in impact to LTE, since the target LTE gNB has to generate release indication. Therefore, we prefer source gNB sends the 1-bit indication to UE before HO, i.e. option 1.

Observation 1: Option 2, i.e. target generates the logged data release indication, may introduce impact to RAN3 and LTE spec.

Proposal 1: (RRC-18) Source gNB sends retain/release logged data indication to UE before HO.

Issue 19

This issue is about what the otherConfig should cation to enable UE to report assistance information. We understand full buffer and low power don’t require explicit configuration. It’s essential to support NW side data collection. Full buffer is important for NW to fetch the logged data. Without full buffer indication, data collection would stop. Low power is important from UE point of view to save power. Without low power indication, UE has to continue the data collection even when it’s power is low, which is unreasonable.

Proposal 2: (RRC-19) Full buffer and low power indication can be provided without network configuration (i.e., network does not need to enable UE reporting full buffer/low power in otherConfig).

However, buffer threshold triggered report needs explicit configuration, which at least includes buffer threshold. Furthermore, UE may not support the buffer threshold report, since such report may require additional UE capability.

Proposal 3: (RRC-19) Buffer threshold triggered report requires explicit configuration in otherConfig.

Proposal 4: (RRC-19) Buffer threshold triggered report is optionally supported by UE indicated by capability.

Issue 20

This issue is about whether UE needs to report more assistance information, e.g., buffer not full or power not low.

According to the running CR, UE would discard the logged data after reporting the logged data via UEInformationResponse. Therefore, NW is awared of when the buffer becomes not full, i.e., after logged data report. There is no need to further introduce duplicated indication of ‘buffer is not full’.

It’s agreed NW is expected to release the data collection configuration upon receiving the power low indication. After data collection is released, UE is not expected to remember it was configured to perform data collection before. We understand the UE, which is not configured with data collection, is not expected to evaluate its power state. There’s no need for UE to evaluate and report its power status after data collection configuration is released. Furthermore, NW may not want to wait for the same UE to recover its power. Instead, network may want to select other suitable UEs to perform NW-side data collection. ‘Power not low’ indication is not so useful from NW point of view. Therefore, the power not low indication is also not needed.

Proposal 5: (RRC-20) No need to introduce ‘buffer not full’ and ‘power state not low’ indication.

With above proposal, prohibit timer is not needed either, since the power state low and buffer full indication doesn’t trigger frequently.

Proposal 6: (RRC-20) Prohibit timer for UE assistance information report related to logging (i.e., power state low and buffer full indication) is not needed.

Issue 21

This issue is about how to indicate the logged data is consecutive or not. NW shall be able to identify the gap in the logged data for model training. If there is gap during data logging due to logging radio event, UE shall indicate the gap to NW. Two options are observed to support the gap indication,

Option 1: UE explicit indicate the gap along with logged data after logging gap is detected.

Option 2: UE creates another list when logging gap is detected. The logged data within the same list is consecutive.

Theoretically, both options can work. However, we understand the maximum number of lists supported by UE may be a fixed number. In option 2, if gap occurs frequently, the lists may be exhausted, but each list only contains small number of entries with logged data. Option 2 may be low efficient compared with option 1.

In option 1, it’s not clear whether absolute time or 1bit indication is used to indicate the gap. We understand UE can indicate the absolute time along with the first entry with logged data after gap. Timing information is already supported in MDT and is more useful for NW to decide how to train the AI model, compared with 1bit indication. Furthermore, NW may change the CSI resource measurement periodicity. Therefore, UE shall also indicate the corresponding CSI resource periodicity along with the logged data.

Proposal 7: (RRC-21) UE explicit indicates the absolute time along with first logged data after logging gap is detected. Additionally, for consecutive logged data, UE also indicates the corresponding CSI resource measurement periodicity along with the logged data.

Issue 23

This issue is about which cell ID is used during data collection. Considering cells on different frequencies may have overlapped PCI, and data collected by NW-side data collection might also be further transmitted to OAM, We support to store CGI as cell ID.

Proposal 8: (RRC-23) UE stores CGI of serving cell along with the logged data.

Issue 25

This issue is about whether multiple configurations can be provided to UE and whether dynamic activation/deactivation is needed.

We understand the data would be use cases specific, e.g. BM and CSI, or even sub use case specific. The required CSI resource pattern or periodicity may be different for different use cases. For example, one data sample of spatial BM prediction includes measurement results of set A and B beam at the same time with the periodicity of 40 ms. While, another data sample of temporal BM prediction includes historic measurement results of set B beam and one measurement result of set A beam with the periodicity of 60 ms. It’s difficult to merge the required CSI resource into one data collection configuration. Furthermore, the logging event may not be the same for different use cases. For example, spatial BM prediction model may require data at cell edge, while the temporal BM prediction may require data at cell centre. If only one data collection configuration is allowed, NW can’t configure appropriate logging event. UE has to always collect data, which results in unnecessary power consumption.

Furthermore, more AI use cases are expected to be supported in future, e.g. AI mobility. It’s straightforward for NW to configure multiple data collection configuration for BM and mobility.

Therefore, NW should be allowed to provide multiple data collection configurations for different (sub) use cases. The support of multiple data collection can be indicated by UE capability.

Proposal 9: (RRC-25) Multiple NW-side training data collection configurations can be configured to UE.

Proposal 10: (RRC-25) Multiple NW-side training data collection configurations can be optionally supported by UE indicated by capability.

A subsequent question is whether dynamic activation/deactivation of data collection is supported. It’s always possible for NW to setup/release the data collection configuration by RRC. According to LS from RAN1 [2], it is observed that the latency requirement of training data collection is rather relaxed, which means the data collection configuration(s) will be valid for a very long time from network point of view. There is no need to introduce dynamic control.

Proposal 11: (RRC-25) L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Issue 26

This issue is about whether new SRB can be used to deliver legacy SON/MDT report. We don’t see the motivation to use new SRB to deliver legacy SON/MDT report. The reason we introduce new SRB is to separate the logged data report from other legacy uplink messages. Furthermore, both SON/MDT and logged data report has relaxed latency requirement. It can be up to UE implementation to trigger UEInformationResponse messages on legacy SRB or new SRB based on the content of UEInformationResponse. If NW requests both SON/MDT and logged data, UE can trigger UEInformationResponse messages on legacy SRB and new SRB to report SON/MDT and logged data separately.

Proposal 12: (RRC-26) New SRB is only used to report logged data.

Issue 27

This issue is about whether to introduce logging interval setting. We understand the logging interval shall be aligned with the RS transmission periodicity. Otherwise, some RS would be missed, which results in radio resource waste.

Proposal 13: (RRC-27) Logging periodicity is aligned with RS transmission periodicity. No need to introduce separate logging interval setting.

Issue 28

This issue is about how to handle the data collection configuration during RRCReestablishment, transition to RRC_INACTIVE. RAN2 has already agreed logged data is released in this case. There is less concern on exposure of data collection configuration. The data collection configuration can be handled as legacy RRC configuration. UE kept data collection configuration. After RRC reestablishment or resume, the serving cell can decide to do delta or full reconfiguration.

Proposal 14: (RRC-28) Follow legacy RRC handling. UE keeps data collection configuration during RRCReestablishment or transition to RRC_INACTIVE. After RRC reestablishment or resume, serving cell can decide to do delta or full reconfiguration.

Issue 29

This issue is about the relationship between data availability report and low power state. NW is expected to release the data collection configuration upon reception of low power indication from UE. The logged data would also be released if data collection is released. NW may want to retrieve the available logged data before releasing the data collection configuration to avoid data lost. Therefore, UE shall report the data availability if there is any logged data and low power state is detected. The cause value can be set to low power.

Proposal 15: (RRC-29) UE reports data availability if there is any logged data and low power state is detected. The cause value of data availability report can be set to low power.

Issue 33

NW side data collection would consume UE storage and power. Similar as MDT, user consent shall be considered when NW decides to configure UE to perform NW side data collection. Currently, user consent is only about whether MDT is allowed as in SA5 spec [1]. It’s not clear whether user consent regrading NW side data collection can reuse the same user consent indication of MDT or separate user consent is introduced. We understand user may have different consents regarding MDT and NW side data collection. Because MDT and NW side data collection are two separate features. Furthermore, compared with MDT, NW side AI may have benefit on UE side from power saving point of view. We propose to introduce separate user consent regarding NW side data collection.

Proposal 16: (RRC-33) User consent regarding NW side data collection is separate from legacy MDT user consent.

Even if the user consent is separate, the framework of existing user consent regarding MDT can still be reused. But SA5 needs to define new user consent bit for NW data collection. Therefore, RAN2 should send LS to ask SA5 to introduce NW side data collection specific user consent.

Proposal 17: (RRC-33) Sent LS to ask SA5 to introduce NW side data collection specific user consent.

Other Open issues

If NW has collected enough data, NW may release the data collection configuration. In this scenario, UE shall also release the logged data. However, if NW modifies training data collection configuration, e.g. modify the CSI-ReportConfig for set A of beams, it’s not clear whether UE shall release or keep the logged data.

Observation 2: If NW modifies training data collection configuration, it’s not clear whether UE shall release or keep the logged data.

It could be beneficial for UE to keep the logged data, considering NW may still need the old data to train a generic model. On the other hand, network can directly release configuration if network sees no need for UE keeps logging the data. In this case, NW has the flexibility to control whether UE can keep the data implicitly.

Proposal 18: If NW releases the data collection configuration, UE releases the logged data. Otherwise, UE keeps the logged data.

Even if UE supports the data logging, it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different types of training data may require different storage sizes. Also, the data reporting configuration may be associated with the storage size, e.g. longer report period would result in UE storing more data. UE can report the supported data logging storage size to NW. NW can decide which data is to be logged by UE and the appropriate configuration, e.g. report period. The reported logging storage size can be used across all use cases.

Proposal 19: UE can support different data logging AS buffer size across all use cases, which can be indicated by UE capability. FFS on the size of reported AS buffer size.

In legacy, NW may page UE in RRC_IDLE/INACTIVE due to traffic arrival or other reasons. It’s mainly up to NW implementation. However, we believe NW shall not page UE to collect data only. Because UE may consume more power in RRC_CONNECTED. NW shall prioritize the UEs in RRC_CONNETED to perform data collection.

Proposal 20: NW shall not page UE for NW-side data collection purpose only.

Conclusion

Based on the discussion in section 2, we have following proposals:

Observation 1: Option 2, i.e. target generates the logged data release indication, may introduce impact to RAN3 and LTE spec.

Proposal 1: (RRC-18) Source gNB sends retain/release logged data indication to UE before HO.

Proposal 2: (RRC-19) Full buffer and low power indication can be provided without network configuration (i.e., network does not need to enable UE reporting full buffer/low power in otherConfig).

Proposal 3: (RRC-19) Buffer threshold triggered report requires explicit configuration in otherConfig.

Proposal 4: (RRC-19) Buffer threshold triggered report is optionally supported by UE indicated by capability.

Proposal 5: (RRC-20) No need to introduce ‘buffer not full’ and ‘power state not low’ indication.

Proposal 6: (RRC-20) Prohibit timer for UE assistance information report related to logging (i.e., power state low and buffer full indication) is not needed.

Proposal 7: (RRC-21) UE explicit indicates the absolute time along with first logged data after logging gap is detected. Additionally, for consecutive logged data, UE also indicates the corresponding CSI resource measurement periodicity along with the logged data.

Proposal 8: (RRC-23) UE stores CGI of serving cell along with the logged data.

Proposal 9: (RRC-25) Multiple NW-side training data collection configurations can be configured to UE.

Proposal 10: (RRC-25) Multiple NW-side training data collection configurations can be optionally supported by UE indicated by capability.

Proposal 11: (RRC-25) L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Proposal 12: (RRC-26) New SRB is only used to report logged data.

Proposal 13: (RRC-27) Logging periodicity is aligned with RS transmission periodicity. No need to introduce separate logging interval setting.

Proposal 14: (RRC-28) Follow legacy RRC handling. UE keeps data collection configuration during RRCReestablishment or transition to RRC_INACTIVE. After RRC reestablishment or resume, serving cell can decide to do delta or full reconfiguration.

Proposal 15: (RRC-29) UE reports data availability if there is any logged data and low power state is detected. The cause value of data availability report can be set to low power.

Proposal 16: (RRC-33) User consent regarding NW side data collection is separate from legacy MDT user consent.

Proposal 17: (RRC-33) Sent LS to ask SA5 to introduce NW side data collection specific user consent.

Observation 2: If NW modifies training data collection configuration, it’s not clear whether UE shall release or keep the logged data.

Proposal 18: If NW releases the data collection configuration, UE releases the logged data. Otherwise, UE keeps the logged data.

Proposal 19: UE can support different data logging AS buffer size across all use cases, which can be indicated by UE capability. FFS on the size of reported AS buffer size.

Proposal 20: NW shall not page UE for NW-side data collection purpose only.

Reference

[1] 32.422

[2] R2-2311720	Reply LS on Data Collection Requirements and Assumptions