3GPP TSG-RAN WG2 Meeting #130	                	                  R2-2503644

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.12.2

Source: 	OPPO

Title:  	Remaining issues of asymmetric DL sTRP and UL mTRP

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion regarding the asymmetric DL sTRP and UL mTRP, RAN2 discussed several issues, including pathloss offset, TCI state activation/deactivation, coexistence with other features and so on. In this contribution, we discuss the remaining issues for the asymmetric DL sTRP and UL mTRP.

# Discussion

## Pathloss offset stored in RRC

RAN2#129bis Agreements: - From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

In general, when the pathlossOffset is configured/updated via RRC or MAC CE, the UE stores the pathlossOffset configuration and use the latest pathlossOffset value for the uplink-only TRP. This means that the UE should change its stored RRC configuration of pathlossOffset, when MAC CE provides a new value of pathlossOffset. However, according to the TS 38.331 guidance of using “need code”, which can be also found in the Annex A, the description of the meaning of “need code” only refers to the configuration provided by the RRC message, not the MAC CE. Then, we could have the following issues of using the “need code” for pathlossOffset updated via both RRC message and MAC CE.

| Need Code | Meaning in TS 38.331 | Issues |
| --- | --- | --- |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). | Issue 1: It is unclear what UE behaviours need to be specified regarding the configuration, e.g. whether/how to store the field and the UE behaviour when field is not absent. |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. | Issue 1: If Need M is used, an explicit release for the pathlossOffset is required. Issue 2: If the pathlossOffset is absent, it is unclear whether the UE should apply the last value configured by RRC or the latest value updated by MAC CE. The meaning of “the current value” used to refer to only the RRC configuration (i.e. not MAC CE). |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. | Issue 1: Need N is not applicable, as pathlossOffset should be stored for the use of many cases, e.g. power control and RACH. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. | Issue 1: The pathlossOffset should be always provided in TCI-state when TCI-state is configured, as the absence of pathlossOffset requires the UE to release it. Issue 2: If the pathlossOffset is absent, it is unclear whether the UE should release the last value configured by RRC or the latest value updated by MAC CE. The meaning of “the current value” used to refer to only the RRC configuration (i.e. not MAC CE). |

Observation 1: In 38.331, it can be understood that the “current” pathlossOffset configuration stored at the RRC only refers to the last configuration provided by RRC message.

If the pathlossOffset updated by MAC is not stored in RRC, the UE implementation would need two separate internal variables of pathloss offset, i.e. one variable for RRC and one variable for MAC. The UE needs to compare the two values and use the latest one. This will complicate the UE implementation. If the pathlossOffset updated by MAC is stored/updated in RRC variable, the UE implementation can be simplified that each time when a new value of pathlossOffset is provided by either RRC or MAC CE, the UE updates the RRC pathlossOffset variable.

Observation 2: If the pathlossOffset updated by MAC does not update the value stored in RRC, a complex UE implementation is needed to have two separate internal variables of pathloss offset, i.e. one variable for RRC message configuration and one variable for MAC CE configuration, which are compared with each other to determine the latest configuration.

TS 38.331: For (parent) fields without need codes in downlink RRC messages or sidelink PC5 RRC message, if the parent field is absent, UE shall follow the need codes of the child fields. Thus, if parent field is absent, the need code of each child field is followed (i.e. Need R child fields are released, Need M child fields are not modified and the actions for Need S child fields depend on the specified conditions of each field). Examples of (parent) fields in downlink RRC messages and sidelink PC5 RRC message without need codes where this rule applies are: - nonCriticalExtension fields at the end of a message using empty SEQUENCE extension mechanism, - groups of non-critical extensions using double brackets (referred to as extension groups), and - non-critical extensions at the end of a message or at the end of a structure, contained in a BIT STRING or OCTET STRING (referred to as parent extension fields).

Running CR of TS 38.331:

According to the running RRC CR, pathlossOffset is configured via the unified TCI-state, i.e. TCI-State and TCI-UL-State. According to the TS 38.331 as quoted above, ul-TCI-ToAddModList (including TCI-UL-State) and tci-StatesToAddModList (including TCI-State) which is “Need N” is the parent field for pathlossOffset. If TCI-State and TCI-UL-State is absent, the UE keeps using the old pathlossOffset as configured by RRC if the pathlossOffset provided by MAC CE is not stored in RRC. Then the out-of-date pathlossOffset as configured by RRC will be considered as the latest configuration.

Observation 3: If the pathlossOffset updated by MAC does not update the value stored in RRC, the absence of the TCI-State and TCI-UL-State (as the parent field for pathlossOffset) due to delta configuration forces the UE to reuse the out-of-date pathlossOffset configured by RRC message, which will be considered as the latest configuration.

In conclusion, we think that the pathlossOffset provided by the MAC CE should update the pathlossOffset configured by RRC message.

Proposal 1: RAN2 is kindly requested to clarify that the pathlossOffset configuration stored in RRC refers to the latest configuration provided by RRC message or MAC CE.

Running CR of TS 38.321:

According to the running MAC CR, the path offset provided by the Pathloss Offset Update MAC CE is not indicated to the RRC, which causes that the RRC layer is not able to update the pathlossOffset stored in the RRC layer. Thus, we think that the running MAC CR should be updated to indicated the path offset provided by the Pathloss Offset Update MAC CE to the RRC (i.e. upper layer).

Proposal 2: To clarify in TS 38.321 that the path offset provided by the Pathloss Offset Update MAC CE is indicated to RRC (i.e. upper layer).

## Processing delay of MAC CE

According to the processing delay of RRC configuration as quoted in Annex B from TS 38.331, in addition to the 10ms of normal RRC processing delay, extra 6ms are added for scell addition/release and SCG establishment/ modification/ release due to the change of the PHY configuration. The RRC processing delay include the decoding time of the RRC message and the time required for applying the RRC configuration. When the pathlossOffset is updated via MAC CE, it is unclear what the processing delay requirement for updating pathlossOffset configuration is. From our understanding, the expected delay should be less than 16ms, as the decoding time required for MAC CE is lower than the decoding time required for RRC message. We think that to allow sufficient processing time in the UE, we could choose 10ms or MAC CE decoding time or “leave to UE implementation” for the processing delay requirement for updating pathlossOffset configuration via MAC CE.

Proposal 3: RAN2 is kindly requested to confirm one of the following options for the maximum processing delay requirement of updating pathlossOffset configuration via MAC CE.

- Option 1: 10ms, as RRC reconfiguration

- Option 2: MAC CE decoding time

- Option 2: up to UE implementation

## Handover procedure

During the handover, the pathlossOffset will be forwarded from the source node to the target node, and the target node can provide a delta configuration by keeping/updating the pathlossOffset configuration. According to the above analysis, the pathlossOffset stored at the RRC should be the latest configuration provided by RRC message or MAC CE. Thus, the source node should forward the latest pathlossOffset to the target node, so that the UE is able to use the latest available pathlossOffset and keep the asymmetric UL/DL configuration for mTRP.

Proposal 4: During handover, the pathlossOffset forwarded from the source node to the target node is the latest configuration provided by RRC message or MAC CE.

## Resume procedure

TS 38.331: 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; - aerial-Config, if configured; - cellDTX-DRX-Config, if configured;

According to the RRC INACTIVE UE behaviours, when the UE is released from RRC_CONNECTED to RRC_INACTIVE, the latest RRC configuration of TCI-state/ TCI-UL-State is suspended and stored at the UE, and will be resumed when the UE receives the RRCResume message from the gNB. According to the TS 38.331, the current RRCResume message is able to provide the latest pathlossOffset configuration (via TCI-state configuration included in the CellGroupConfig) to the UE. As explained above, the latest pathlossOffset stored at the RRC should be the latest configuration provided by RRC or MAC CE. Then during the RRC resume procedure, the RRCResume message does not have to include the pathlossOffset configuration provided in TCI-state field, and the MAC CE can be used to update the pathlossOffset value.

Proposal 5: The INACTIVE UE stores the latest pathlossOffset provided by RRC message or MAC CE.

Proposal 6: The gNB can provide the latest pathlossOffset via the CellGroupConfig of the RRCResume message. No specification change is needed.

## LTM

WID of Rel-19 MIMO: - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

According to the Rel-19 MIMO WID as quoted above, the asymmetric DL sTRP/UL mTRP is only for intra-band intra-DU case, this means that during the inter-gNB or inter-DU handover, the target node is not able to provide the configuration for the asymmetric DL sTRP/UL mTRP, including the pathloss offset configuration.

Observation 4: According to the WID of Rel-19 MIMO, the Rel-19 asymmetric DL sTRP/UL mTRP deployment is only applicable to intra-band intra-DU non-co-located mTRP scenarios.

For the LTM procedure, we think that for the intra-DU LTM, the pathloss offset configuration can still be provided via the candidate LTM configuration. As the pathloss offset can be changed when the UE moves around, the UE should be provided with the latest pathloss offset if the UL only TRP is to be used during the cell switching procedure triggered by LTM Cell Switch Command MAC CE. However, as the LTM procedure is used to reduce the handover latency, and the UL-only TRP is used to improve the uplink throughput, the gNB can still indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE, and the latest pathloss offset for the uplink-only TRP can be indicated after the completion of the handover.

Observation 5: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE, and the latest pathloss offset can be provided after LTM completion.

Thus, we consider that the asymmetric DL sTRP/UL mTRP can still be configured for the LTM at least for the intra-DU case, and the gNB by implementation can provide the latest pathloss offset after handover.

Proposal 7: Asymmetric DL sTRP/UL mTRP can be configured for LTM without extra enhancements.

## TCI state activation MAC CE

RAN1#117 meeting agreements: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

According to the RAN1#117 meeting agreements, for the asymmetric DL sTRP/UL mTRP deployment scenario, the Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework are reused.

TS 38.214: The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State(s) and/or TCI-UL-State(s) to only one TCI codepoint, the UE shall apply the indicated TCI-State(s) and/or TCI-UL-State(s) to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

According to TS38.214, one code point of Transmission Configuration Indication field in DCI format 1_2/1_3/1_4 can indicate the UE to apply the “TCI-State(s) and/or TCI-UL-State(s)” associated with this code point, and MAC CE is used to map (i.e. activate) the “TCI-State(s) and/or TCI-UL-State(s)” associated with each code point of Transmission Configuration Indication field in DCI format 1_2/1_3/1_4.

TS 38.321: (Rel-17 unified TCI/ICBM) - Pi: This field indicates whether each TCI codepoint has multiple TCI states or single TCI state. If Pi field is set to 1, it indicates that ith TCI codepoint includes the DL TCI state and the UL TCI state. If Pi field is set to 0, it indicates that ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields; - D/U: This field indicate whether the TCI state ID in the same octet is for joint/downlink or uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink. If this field is set to 0, the TCI state ID in the same octet is for uplink; - TCI state ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remainder 6 bits indicate the TCI-UL-State-Id as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16;

As quoted above, “Unified TCI state activation/deactivation MAC CE” can be reused to map “one joint TCI state” or “one DL TCI state + one UL TCI state” to a code point of Transmission Configuration Indication field in DCI format 1_2/1_3/1_4, for the asymmetric DL sTRP/UL mTRP.

Observation 6: When the UE is configured with Rel-17 unified TCI state, Unified TCI state activation/deactivation MAC CE of Rel-17 can be reused to map “one joint TCI state” or “one DL TCI state + one UL TCI state” to one code point.

TS 38.321: (Rel-18 unified TCI) - Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th joint TCI state is present or not, where j=1, 2. If Fi,j field is set to 1, it indicates the j-th joint TCI state for codepoint i is present. If Fi,j field is set to 0, it indicates the j-th joint TCI state for codepoint i is absent. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields; - Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th DL TCI state is present or not, where j=1, 2. If Fi,j field is set to 1, it indicates the j-th DL TCI state for codepoint i is present. If Fi,j field is set to 0, it indicates the j-th DL TCI state for codepoint i is absent; - Si,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th UL TCI state is present or not, where j=1, 2. If Si,j field is set to 1, it indicates the j-th UL TCI state for codepoint i is present. If Si,j field is set to 0, it indicates the j-th UL TCI state for codepoint i is absent;

When the UE is configured with Rel-18 unified TCI state, for joint TCI state, “Enhanced unified TCI states activation/deactivation MAC CE for Joint TCI States” can be reused to map “up to two joint TCI states” to a code point of Transmission Configuration Indication field in DCI format 1_2/1_3/1_4, for the asymmetric DL sTRP/UL mTRP.

For separate TCI state, to map “one DL TCI state + up to two UL TCI state” to one code point of TCI field, “Enhanced unified TCI states activation/deactivation MAC CE for Separate TCI States” can be reused by setting one Fi,j field to “0” (i.e. one Fi,j field set to “1” + two Si,j field set to “1”).

Observation 7: When the UE is configured with Rel-18 unified TCI state, Enhanced unified TCI states activation/deactivation MAC CE for Joint TCI States can be reused to map “up to two joint TCI states” to one code point, and Enhanced unified TCI states activation/deactivation MAC CE for Separate TCI States can be reused to map “one DL TCI state + up to two UL TCI state” to one code point by setting one Fi,j field to “0”.

TS 38.214: When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.

However, according to TS 38.214 of Rel-18, when the UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, it seems that the subsequent DCI can only update the two TCI states, and cannot change the number of indicated TCI-state. This could be problematic when the number of indicated TCI-states need to be changed, e.g. changing from “2 DL TRP + 2 UL TRP” to “1 DL TRP + 2 UL TRP” as illustrated below. However, we think that the issue is due to the restriction in the RAN1 specification, and the Rel-18 MAC CE is flexible enough to map different combinations of TCI states to each code point. RAN1 should be the responsible working group to handle their specification issues.

Figure 1: Issue of indicated TCI states

Observation 8: According to TS 38.214, it seems that the number of indicated TCI-state cannot be changed.

Proposal 8: When the UE is configured with Rel-17 unified TCI state, Unified TCI state activation/deactivation MAC CE of Rel-17 is reused, unless the new design of MAC CE is required by RAN1.

Proposal 9: When the UE is configured with Rel-18 unified TCI state, Enhanced unified TCI states activation/deactivation MAC CE for Joint TCI States and Enhanced unified TCI states activation/deactivation MAC CE for Separate TCI States are reused, unless the new design of MAC CE is required by RAN1.

Proposal 10: The PHY specification issue that the number of indicated TCI states cannot be changed should be discussed in RAN1.

# Conclusion

According to the analysis provided above, we have the following observations and proposals:

Observation 1: In 38.331, it can be understood that the “current” pathlossOffset configuration stored at the RRC only refers to the last configuration provided by RRC message.

Observation 2: If the pathlossOffset updated by MAC does not update the value stored in RRC, a complex UE implementation is needed to have two separate internal variables of pathloss offset, i.e. one variable for RRC message configuration and one variable for MAC CE configuration, which are compared with each other to determine the latest configuration.

Observation 3: If the pathlossOffset updated by MAC does not update the value stored in RRC, the absence of the TCI-State and TCI-UL-State (as the parent field for pathlossOffset) due to delta configuration forces the UE to reuse the out-of-date pathlossOffset configured by RRC message, which will be considered as the latest configuration.

Observation 4: According to the WID of Rel-19 MIMO, the Rel-19 asymmetric DL sTRP/UL mTRP deployment is only applicable to intra-band intra-DU non-co-located mTRP scenarios.

Observation 5: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE, and the latest pathloss offset can be provided after LTM completion.

Observation 6: When the UE is configured with Rel-17 unified TCI state, Unified TCI state activation/deactivation MAC CE of Rel-17 can be reused to map “one joint TCI state” or “one DL TCI state + one UL TCI state” to one code point.

Observation 7: When the UE is configured with Rel-18 unified TCI state, Enhanced unified TCI states activation/deactivation MAC CE for Joint TCI States can be reused to map “up to two joint TCI states” to one code point, and Enhanced unified TCI states activation/deactivation MAC CE for Separate TCI States can be reused to map “one DL TCI state + up to two UL TCI state” to one code point by setting one Fi,j field to “0”.

Observation 8: According to TS 38.214, it seems that the number of indicated TCI-state cannot be changed.

Proposal 1: RAN2 is kindly requested to clarify that the pathlossOffset configuration stored in RRC refers to the latest configuration provided by RRC message or MAC CE.

Proposal 2: To clarify in TS 38.321 that the path offset provided by the Pathloss Offset Update MAC CE is indicated to RRC (i.e. upper layer).

Proposal 3: RAN2 is kindly requested to confirm one of the following options for the maximum processing delay requirement of updating pathlossOffset configuration via MAC CE.

- Option 1: 10ms, as RRC reconfiguration

- Option 2: MAC CE decoding time

- Option 2: up to UE implementation

Proposal 4: During handover, the pathlossOffset forwarded from the source node to the target node is the latest configuration provided by RRC message or MAC CE.

Proposal 5: The INACTIVE UE stores the latest pathlossOffset provided by RRC message or MAC CE.

Proposal 6: The gNB can provide the latest pathlossOffset via the CellGroupConfig of the RRCResume message. No specification change is needed.

Proposal 7: Asymmetric DL sTRP/UL mTRP can be configured for LTM without extra enhancements.

Proposal 8: When the UE is configured with Rel-17 unified TCI state, Unified TCI state activation/deactivation MAC CE of Rel-17 is reused, unless the new design of MAC CE is required by RAN1.

Proposal 9: When the UE is configured with Rel-18 unified TCI state, Enhanced unified TCI states activation/deactivation MAC CE for Joint TCI States and Enhanced unified TCI states activation/deactivation MAC CE for Separate TCI States are reused, unless the new design of MAC CE is required by RAN1.

Proposal 10: The PHY specification issue that the number of indicated TCI states cannot be changed should be discussed in RAN1.

# Reference

[1] 3GPP TS38.331, “NR; Radio Resource Control (RRC) protocol specification”

[2] 3GPP TS 38.214, “NR; Physical layer procedures for data”

# Annex A

The follows are quoted from TS 38.331:

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

# Annex B

The followings are quoted from TS 38.331:

12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (Intra-NR mobility with LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration | DLDedicatedMessageSegment | RRCReconfigurationComplete | 16+( Nseg -1)*10 | Nseg is number of RRC segments |