3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503660

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations on open issues of RRM relaxation and offloading in RRC_IDLE/INACTIVE.

Discussion

(RRC-7/38304-3)The exit condition of MR RRM measurement relaxation

In RAN2#129 meeting, agreements on MR serving cell measurement relaxation included:

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In RAN2#129bis meeting, it was agreed to merge entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). But it is still FFS on exit condition for MR RRM measurement relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition includes MR and/or LR-based measurements.

The possible options for exit condition of MR measurement relaxation include:

- Option 1: No separate exit condition.

- Option 2: Only based on LR serving cell quality.

- Option 3: Only based on MR serving cell quality.

During MR measurement relaxation period, MR serving cell quality is relaxed with longer measurement cycle. MR serving cell quality cannot be updated timely. Hence, it is not reasonable to consider MR serving cell quality only for exit condition of MR serving cell measurement relaxation. Option 3 is not preferred. Option 1 is similar as R16 RRM relaxation criteria. But different from R16 RRM relaxation for which only neighbor cell quality is relaxed, for LP-WUS MR serving cell quality is relaxed as well. It is questionable to reuse the principle of R16 RRM relaxation to guarantee the measurement accuracy. While, Option 2 is aligned with both LP-WUS monitoring and serving cell measurement offloading, which is preferred.

Proposal 1: (RRC-7/38304-3) The exit condition of MR measurement relaxation is only based on LR serving cell quality.

(RRC-8/38304-4) Whether/how to reduce the threshold number for LP-WUS

- After on-line discussion in RAN2#129bis, the possible directions to reduce the threshold number for LP-WUS for further discussion are:

- Direction 3a. merging LPWUS monitoring and serving cell offloading

- Direction 3b. merging LPWUS monitoring and Rel-19 RRM relaxation

Considering majority think there is almost no power saving gain if UE performs LP-WUS monitoring with normal MR RRM measurements, we are ok to merge LP-WUS monitoring with RRM relaxation or offloading. In our understanding, criteria for serving cell offloading should be better than those for Rel-19 RRM relaxation if both are configured in a cell. In that case, in order to perform LP-WUS monitoring as much as possible, it is reasonable to merge LP-WUS monitoring and Rel-19 RRM relaxation. But regarding it is possible that Rel-19 RRM relaxation is not configured, direction 3a needs to be supported as well. Therefore, if Rel-19 RRM relaxation is configured in a cell, LP-WUS monitoring criteria are the same as Rel-19 RRM relaxation criteria. If only serving cell offloading is configured in a cell, LP-WUS monitoring criteria are the same as that of serving cell offloading.

Proposal 2: (RRC-8/38304-4) If Rel-19 RRM relaxation is configured in a cell, LP-WUS monitoring criteria are the same as Rel-19 RRM relaxation criteria. If only serving cell offloading is configured in a cell, LP-WUS monitoring criteria are the same as the criteria of serving cell offloading.

(38304-1) FFS on the terminology LP-WUS UE

It is FFS on the terminology LP-WUS UE in TS 38.304 running CR for LP-WUS. The terminology LP-WUS UE is related with UE capabilities for LP-WUS. According to the discussion on UE capabilities for LP-WUS [2], use cases can be:

Proposal 3: Way forward on the supported use cases regarding UE supporting LP-WUS reception, and RRM measurement relaxation and fully offloading: - Agree to support “Case 1” (7/9 support Case 1): UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading - Discuss on supporting “Case 2” (2/9 support Case 2): UE supports LP-WUS reception, but does not support RRM measurement relaxation and RRM measurement fully offloading - Preclude “Case 3” (1/9 support Case 3): UE supports RRM measurement relaxation and RRM measurement fully offloading, but does not support LP-WUS reception

If only Case 1 is supported, UE supporting LP-WUS can be used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS. And UE supporting LP-WUS can refer to the definition in TS 38.306. If Case 2 is supported, UE supporting MR RRM relaxation and full offloading via LR is used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS. And UE supporting MR RRM relaxation and full offloading can refer to the definition in TS 38.306 as well.

Proposal 3: (38304-1)

- If only Case 1 (i.e., UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading) is supported, UE supporting LP-WUS is used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS;

- If Case 2 (i.e., UE supports LP-WUS reception, but does not support RRM measurement relaxation and RRM measurement fully offloading) is supported, UE supporting MR RRM relaxation and full offloading via LR is used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS.

Serving cell measurement offloading

In legacy, if the MR serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements including [1]:

-	If the serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In RAN2#127 meeting, RAN2 agreed that entry conditions for MR serving cell measurement offloading (i.e., there is no serving cell measurement by MR) can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Entry conditions for serving cell measurement offloading should not be worse than conditions for legacy MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority, considering serving cell measurement offloading should be careful. Therefore, MR RSRP threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP, as shown in Figure 1. Similarly, MR RSRQ threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Proposal 4: MR RSRP threshold in the entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 5: MR RSRQ threshold in the entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement off no matter if MR serving cell measurement is relaxed or not, i.e. if the MR serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform MR intra-frequency measurements; If the MR serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform MR measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Proposal 6: Before entry conditions for serving cell measurement offloading are fulfilled, the UE may follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Figure 1: One example of the serving cell measurement offloading coverage

Conclusion

According to the analysis in Section 2, we propose:

Proposal 1: (RRC-7/38304-3) The exit condition of MR measurement relaxation is only based on LR serving cell quality.

Proposal 2: (RRC-8/38304-4) If Rel-19 RRM relaxation is configured in a cell, LP-WUS monitoring criteria are the same as Rel-19 RRM relaxation criteria. If only serving cell offloading is configured in a cell, LP-WUS monitoring criteria are the same as the criteria of serving cell offloading.

Proposal 3: (38304-1)

- If only Case 1 (i.e., UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading) is supported, UE supporting LP-WUS is used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS;

- If Case 2 (i.e., UE supports LP-WUS reception, but does not support RRM measurement relaxation and RRM measurement fully offloading) is supported, UE supporting MR RRM relaxation and full offloading via LR is used instead of LP-WUS UE in TS 38.304 running CR for LP-WUS.

Proposal 4: MR RSRP threshold in the entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 5: MR RSRQ threshold in the entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Proposal 6: Before entry conditions for serving cell measurement offloading are fulfilled, the UE may follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state;

- R2-2503899, Summary of [Post129bis][218][LPWUS] UE Capabilities (Huawei).