3GPP TSG-RAN WG2 Meeting #130	R2-2503663

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on support of PWS for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The R19 WID [1] for IoT NTN has been updated in RAN plenary#105 meeting to cover the objective for support of PWS (Public Warning System). In detail, the support of PWS messages for NB-IoT is expected to reuse the LTE mechanisms. Furthermore, the support of geo-fencing function for ETWS will be considered based on NR NTN outcome.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

The following agreements have been reached from RAN2-127bis to RAN2-129bis meeting.

RAN2-127bis Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

RAN2-128 Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

RAN2-129 Agreements:

1.	Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

2.	Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

3.	Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

4.	We don’t introduce a new SIB for carrying geographical information for emergency areas

5.	Send a LS to CT1 and CT4 indicating that RAN2 currently supports a maximum of 64 segments for SIB11-NB and SIB12-NB with a maximum TBS of 680bits and asking to provide feedback if they think we need to work on changes to allow longer messages (that would require more segments). Also indicate some possible drawbacks

RAN2-129bis Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In this contribution, we elaborate more on remaining issues related to PWS for NB-IoT NTN.

# 2	Discussion

## 2.1	Inter-cell PWS reception (open issue RRC-6)

According to current TS36.331 specification, the PWS message may be segmented to accomodate a subset of the message information within the SIB (which has size limitations). However, for the UE receiving the PWS message, the segment feature is only supported for intra-cell case. i.e., if the RRC idle UE enters a new cell, the UE shall discard any previously buffered warning message segment(s).

#### 36.331: 5.2.2.4 System information acquisition by the UE … 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

Observation 1: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Different from TN, in NTN, the cell (satellite) is moving hence the UE may experience cell change frequently. For Earth-Moving cell (EMC), a UE may experience a cell change every ~6 seconds as indicated in TR38.821 Table 7.3.2.1.4-1. Therefore, the NW may not manage to broadcast all segments of a PWS message in the current NTN serving cell to a UE due to the large PWS message size (and corresponding huge number of segments) and limited cell availability time (~6 seconds). This may cause the UE cannot receive all the segments for the PWS message if the serving cell is changed before the UE receives all the segments.

Furthermore, whether UE can receive all the segments of a PWS message also depends on when the NW starts to broadcast the segments and the UE’s location. The satellite/eNB may start to broadcast PWS message after it receives the PWS message from CN. However, as illustrated in Figure 1, when the satellite/eNB starts to broadcast PWS segments at current moment, the UE located in the cell coverage edge cannot receive all the segments from current serving cell since its cell coverage will be moved out very soon. Similarly, in an Earth Fixed Cell scenario, the UE may also start the PWS message segment acquisition in one cell, but then experience a cell change (according to t-service).

Figure 1: The UE may not receive all segments from current (earth moving) serving cell

If the legacy mechanism of discarding buffered PWS segments upon cell change is followed, there might be the risk of non-succesful delivery of PWS message due to frequent cell change in NTN scenario.

Observation 2: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell in moving cell scenario.

In our understanding, due to the frequent cell change in NTN and the short serving time duration of a NTN cell, it is not a corner case where UE cannot receive all the segments of a PWS message from a single cell. Therefore, we propose to support the continued reception of PWS segmentation of a message from different cells.

Proposal 1: Support continued reception of PWS segmentation of a message from different cells in moving cell scenario.

The PWS information (SIB11 and SIB12) already includes a message identifier, serial number and segment number. By checking the message identifier and serial number in PWS SIBs, the UE would know whether the segments are from the same PWS message hence it can be assembled or not. In other words, the same pair means the same PWS message and the sgements can be assembled even if the segements are received from different cells.

SystemInformationBlockType11 information element -- ASN1START SystemInformationBlockType11 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 ..., lateNonCriticalExtension OCTET STRING OPTIONAL } -- ASN1STOP

To support the inter-cell segments reception, the gNB should broadcast a new flag to inform UE that the new function is enabled by network. Hence the UE should not discard the segments as in legacy operation when it changes the serving cell.

Proposal 2: Introduce a new indication in SIB for inter-cell PWS segmentation reception.

Furthermore, the two different cells broadcasting the segments of the same PWS message may need to coordinate segmentation transmissions to faciliate the segments cobmination. Therefore, we think the serving cell and target cell should exchange the segmentation size in the inter-node message.

Proposal 3: The segementation size of PWS message should be exchanged via inter-node message to support inter-cell PWS reception.

## 2.2	Support PWS in S&F operation

When the cell capable of store-and-forward operates in normal mode the PWS transmission can be supported for the areas that are under the coverage of the cell at the time of normal mode operation. When the cell switches to S&F mode the time of delivery of PWS depends on the actual service area and the time the cell takes to start serving the new service area.  This impacts the delivery time requirements of PWS.

Observation 3: PWS delivery when cell operates in S&F mode may have impacts on meeting delay requirements of PWS services.

From RAN2 perspective the UE capability for S&F and PWS support are independent features and it is up to network implementation to configure these services together. However, whether such support is needed for PWS needs to be decided in SA2 based on service requirements and impacts.

Observation 4: RAN2 assumes no additional impact in supporting PWS functionality in S&F operation.

Based on the above analysis we propose that support of PWS in S&F operation is decided by SA2. RAN2 can indicate that this can be supported without additional signalling impacts.

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

## 2.3	PWS delivery during eDRX

eDRX is a key feature for NB-IoT devices that enhances their power efficiency and extends battery life. This makes it a key enabler for NB-IoT in massive IoT deployments. To further reduce the energy consumption for paging monitoring towards a UE configured with eDRX, the wake-up signals (WUS) feature can be used. The typical scenario is that NW configures a short eDRX cycle configurations to UE to support the case where the network command expects a UE response within specific duration.

Observation 5: WUS configuration helps to minimize downlink monitoring related energy consumption for UE configured with shorter eDRX cycle.

When network support PWS for NB-IoT and also shorter eDRX cycle for NB-IoT capable UE, network may configure WUS in the cell to minimize the impact of shorter eDRX cycle.

Observation 6: Network that supports PWS and shorter eDRX cycle configuration for UE may activate WUS functionality to reduce the overall energy consumption associated with faster PWS reception.

However, if PWS is supported and the transmission is indicated via paging messages (NB-IoT paging) or direct indication information message, it may wake up all the UEs in the satellite coverage area if the current WUS is used. For PWS scheduling in resource and energy efficient manner use of separate WUS for PWS services can be considered.

Observation 7: When WUS is configured in NB-IoT cell, the PWS transmission may false wake-up all the UE outside the PWS service area resulting in reducing the overall efficiency of WUS benefits.

The false wake-up problem of PWS for NB-IoT UE is a specific problem for NTN where the cell coverage will be larger than PWS service area. This issue can be mitigated if separate WUS is configured for PWS related paging. The network can selectively active this PWS in the areas where the notification is needed.

Proposal 5: RAN2 to consider PWS specific WUS configuration for improved paging efficiency for PWS related paging notification.

# 3	Conclusion

Inter-cell PWS reception (open issue RRC-6)

Observation 1: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Observation 2: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell in moving cell scenario.

Proposal 1: Support continued reception of PWS segmentation of a message from different cells in moving cell scenario.

Proposal 2: Introduce a new indication in SIB for inter-cell PWS segmentation reception.

Proposal 3: The segementation size of PWS message should be exchanged via inter-node message to support inter-cell PWS reception.

PWS Support in SF mode

Observation 3: PWS delivery when cell operates in S&F mode may have impacts on meeting delay requirements of PWS services.

Observation 4: RAN2 assumes no additional impact in supporting PWS functionality in S&F operation.

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

WUS Efficiency Impacts for PWS Transmission

Observation 5: WUS configuration helps to minimize downlink monitoring related energy consumption for UE configured with shorter eDRX cycle.

Observation 6: Network that supports PWS and shorter eDRX cycle configuration for UE may activate WUS functionality to reduce the overall energy consumption associated with faster PWS reception.

Observation 7: When WUS is configured in NB-IoT cell, the PWS transmission may false wake-up all the UE outside the PWS service area resulting in reducing the overall efficiency of WUS benefits.

Proposal 5: RAN2 to consider PWS specific WUS configuration for improved paging efficiency for PWS related paging notification.