3GPP TSG-RAN WG2 Meeting #130	R2-2503683

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on remaining issues of CLTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding conditional intra-CU LTM, the following agreements were reached.

Agreements on C-LTM:

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

5.  Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

In this contribution, we would like to share our opinions on remaining issues of CLTM.

- Discussion

Selection of target cell if multiple cells met CLTM execution condition

In RAN2#129 meeting, the following agreements were reached on CLTM.

Agreements in RAN2#129:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

As per the above agreements, for CLTM based on L3 measurement, the evaluation is per candidate cell. If there are multiple candidate cells met the execution condition, how to select a targer cell by the UE shall be discussed and confirmed. Similar issue has been discussed for CHO before, and it is up to UE implemetation to select the target cell. However, unlike CHO, early sycn is supported for CLTM, which is the most essential feature to reduce the HO latency. Thus, among the multiple candidate cells met the excution conditions, the candidate cell with a valid TA shall be selected as the target cell with a first priority. Then, if there are still multiple candidate cells with valid TA, it could be up to the UE implementation to select a target cell among them.

Proposal 1: For L3 based C-LTM events, the C-LTM execution is triggered when at least one candidate cell fulfills the C-LTM event condition. When multiple candidate cells satisfy the C-LTM condition, the candidate cell with valid TA shall be prioritized.

Proposal 2: If there are multiple candidate cells met the condition with valid TAs or all the candidate cells met the condition don’t have valid TA, it is up to the UE implementation to select a target cell among them.

Moreover, as per the agreement made in RAN2#129, NW configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. Thus, it is possible that different candidate cells may have different execution conditions (i.e. L1 based or L3 based execution conditions). Another open issue on target cell selection is whether to prioritize L1 or L3 based candidate cell met the execution condition. Similarly, the candidate cell with a valid TA shall be prioritized no matter it is configured with L1 based or L3 execution conditions. Regarding whether to prioritize L1 or L3 based candidate cells, we see no need to further differential them. If there are different priorities between L1 or L3 based execution conditions, the NW can just choose to configure L1 or L3 based execution condition to all the candidate cells. Thus, we prefer to leave the selection of L1 based or L3 based candidate cell to UE implementation.

Proposal 3: If there are different candidate cells met the L1 or L3 based conditions, the selection of L1 based or L3 based candidate cell as a target cell could be up to UE implementation.

Selection of the target beam in L3-based RACH-less CLTM

Another open issue is the beam selection in L3-based RACH-less CLTM. As per agreed in RAN2#129, for L1 based C-LTM, beam selection for RACH-less LTM with CG is based on the beam meets CLTM execution condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. It is obvious that the selected beam is relatively trustable since it shall meet the L1 based CLTM execution condition first.

Regarding L3-based RACH-less CLTM, the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). It may be hard for a UE to select a proper beam with good enough quality as L1 based RACH-less CLTM. In legacy L3 RACH-less HO, a RSRP threshold (i.e. cg-RRC-RSRP-ThresholdSSB) can be configured by the NW to help UE select a proper beam. Similar method could be considered for L3-based RACH-less CLTM as well. The NW could configure a RSRP threshold for beam selection. The UE shall select a beam with RSRP higher than the configured threshold and if there are multiple beams meet the thresholds, it is up to UE implementation to select a beam among them.

Proposal 4: Regarding L3-based RACH-less CLTM, the NW could configure a RSRP threshold for beam selection, similar to cg-RRC-RSRP-ThresholdSSB.

Proposal 5: If there are multiple beams meet the thresholds, it is up to UE implementation to select a beam among them.

- Conclusion

In this contribution, we discuss the remaining issues on CLTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For L3 based C-LTM events, the C-LTM execution is triggered when at least one candidate cell fulfills the C-LTM event condition. When multiple candidate cells satisfy the C-LTM condition, the candidate cell with valid TA shall be prioritized.

Proposal 2: If there are multiple candidate cells met the condition with valid TAs or all the candidate cells met the condition don’t have valid TA, it is up to the UE implementation to select a target cell among them.

Proposal 3: If there are different candidate cells met the L1 or L3 based conditions, the selection of L1 based or L3 based candidate cell as a target cell could be up to UE implementation.

Proposal 4: Regarding L3-based RACH-less CLTM, the NW could configure a RSRP threshold for beam selection, similar to cg-RRC-RSRP-ThresholdSSB.

Proposal 5: If there are multiple beams meet the thresholds, it is up to UE implementation to select a beam among them.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB