3GPP TSG-RAN WG2 Meeting #130	R2-2503690

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon, ZTE Corporation, Samsung, Sharp, NEC, CATT

Title: 	TP for using additional LCP priority for SR priority determination

Document for:	Discussion and Decision

# 1. Introduction

RAN2 discussed SR priority determination during intra-UE prioritisation whether to consider the additional priority based on remaining time of the data of a logical channel that triggers the SR as follow:

Agreements on LCP enhancements - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

In this contribution, we look into how to specify this in the MAC specification

# 2.	Discussion

## 2.1	Motivation of considering additional priority for SR priority determination

In legacy specification, the intra-UE prioritization procedure also considers the scenario of the collision between the resource of SR and resource of MAC PDU, where the priority of the SR is determined by the priority of the LCH that triggered SR. For example, if the priority of the LCH that triggered an SR is higher than that of an overlapping uplink grant, the MAC entity shall consider the SR transmission as a prioritized SR transmission. As the additional priority is introduced, there exists ambiguity about which priority should be applied to determine the priority of the SR transmission, especially when there is LCH priority-adjusted data buffered in the LCH that triggered the SR.

Based on our analysis, we observe the benefit of applying the additional priority for the SR transmission when there is LCH priority-adjusted data buffered. Considering two LCHs, LCH#1 and LCH#2, LCH#1 is configured with additional priority, where the default priority of LCH#1 is lower than that of LCH#2, but the additional priority is higher than that of LCH#2. Due to LCP restrictions in LCH#1 and LCH#2, the data buffered in LCH#2 is multiplexed in MAC PDU. However, the LCH#1 may trigger an SR transmission when there is buffered data becoming delay-critical, where the transmission time of the SR overlaps with that of the already assembled MAC PDU with LCH#2.  Note that if the DSR that triggered the SR is included into the MAC PDU with LCH#2, the SR will not have been triggered. Since the default priority of LCH#1 is lower than that of LCH#2 (and also the UL grant), the SR transmission will be deprioritized if the SR resource overlaps with the PUSCH carrying data from LCH#2. As a consequence, the delay-critical data cannot be successfully transmitted within PDB, which significantly deteriorates the experience of XR users. By contrast, if the additional priority of LCH#1 can be applied for determining the priority of the SR transmission, the transmitted SR can request more resources to transmit both the DSR MAC CE and the delay-critical data. Therefore, we believe that there is evident benefit of applying additional priority of the LCH for determining the priority of the SR transmission triggered by the LCH.

Observation 1:  The additional priority of an LCH should be considered for the SR (triggered by DSR) priority determination when the DSR is not included into the MAC PDU related to the overlapping UL grant.

## 2.2	Further consideration on specification impact

According to the current MAC running CR, the LCH priority used to determine the priority of an UL grant during intra-UE prioritization “shall be the highest priority that can be applied or has been applied for it in the LCP procedure”. In LCP procedure on the other hand, it is specified that delay-criticality of the data is determined according to its actual transmission time. We believe the same principle should be applied to determine whether default or additional priority of the LCH triggering an SR should be applied to determine the priority of the SR. Based on our understanding, the actual SR transmission time can reflect the status of the pending delay-critical data more precisely. To be more specific, when determining the priority of the SR, if there is LCH priority-adjusted data buffered in the LCH that triggered the SR at the time of SR transmission, the MAC entity should apply the additional priority of the LCH to determine the priority of the LCH triggering the SR.

Observation 2:	For intra-UE prioritization procedure, the actual SR transmission time should be considered to determine whether to apply the additional or default priority of the LCH that triggered the SR.

Based on Observation 2, the following TP is proposed to be included into Section 5.4.1 of the MAC specification:

When the MAC entity is configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold at the time of the SR transmission.

# 3. Conclusion

RAN2 is requested to discuss the observations and include following TP to Section 5.4.1 of the MAC specification:

Observation 1:  The additional priority of an LCH should be considered for the SR (triggered by DSR) priority determination when the DSR is not included into the MAC-PDU related to the overlapping UL grant.

Observation 2:	For intra-UE prioritization procedure, the actual SR transmission time should be considered to determine whether to apply the additional or default priority of the LCH that triggered the SR.

Proposed TP: [MAC-01]

When the MAC entity is configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold at the time of the SR transmission.

# 4. Reference

[1]https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN2/Inbox/Chair_Notes/R2_129bis%20R19%20XR%20session%20notes%20(Dawid)%202025-04-10_EOM.docx

# 5. Appendix

## 5.1 Appendix 1

### 5.4.1	UL Grant reception

…… Omitted……

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	If a configured uplink grant is associated with a multi-PUSCH configured grant, CURRENT_symbol refers to the symbol index of the first transmission occasion in the first configured uplink grant within the same periodicity. Otherwise, CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold at the time of the SR transmission.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup.

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.