# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2503710
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Remaining issues on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

By RAN2#129b [2], OD-SSB made some progress on MAC-CE format and L3 measurements. In this contribution, we continue to discuss remaining issues of OD-SSB.

# 2 Discussion

According to RAN1 agreement, OD-SSB is supported before (Scenario 2), when the UE receives the SCell activation command (Scenario 2A) and after completion of OD-SSB transmission (Scenario 3B), which is illustrated in Figure.1 for ease of followed discussion.

Figure.1 Illustration of supported scenarios (scenario 2/2A/3B) of OD-SSB

Meanwhile, RAN1 also agreed to support Case 1 and Case 2, which are illustrated in Figrue.2 for ease of followed discussion.

- Case 1: No always-on SSB on the SCell.

- Case 2: Always-on SSB is also periodically transmitted on the SCell besides OD-SSB.

Figure. 2 Illustration of Case 1 and Case 2 of OD-SSB

## 2.1 OD-SSB MAC CE for implicit deactivation

In RAN2#129b [2], the MAC-CE format for OD-SSB in case of explicit deactivation was concluded, and it was agreed that RAN2 aims to reuse the same MAC-CE format to support implicit OD-SSB deactivation.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

Based on above agreement, we provide an example of explicit activation / deactivation MAC-CE format with up to 7 SCells and 5bit configuration index in Figure. 3. Please note that the required bit number of configuration index depends on maximum number of OD-SSB configurations. 5bit is just an example for ease of illustration.

Figure. 3 Illustration of OD-SSB MAC-CE with 7 SCells and 5bit configuration index (Note: the bit number of configuration index depends on maximum number of OD-SSB configurations. Here, 5bit is just an example)

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Based on Figure. 3, we discuss how this MAC-CE format can support implicit deactivation. First, the related RAN1#120b agreements are copied as follows [3].

According to the agreement, we understand that the requirements for the MAC-CE design include:

- Implicit deactivation is enabled by checking if od-ssb-nrofBurst is present in the OD-SSB configuration indicated by the configuration index included in the MAC-CE (i.e. no need to introduce explicit indication in RRC on whether implicit deactivation is enabled).

- RAN1 may support early deactivation via MAC-CE before N transmissions of OD-SSB are ended.

Observation 1: According to RAN1 agreement, the requirements for MAC-CE on implicit deactivation include:

- The UE determines whether implicit deactivation is enabled by checking if od-ssb-nrofBurst is present in the OD-SSB configuration indicated by the configuration index included in the MAC-CE.

- RAN1 may support early deactivation via MAC-CE before N transmissions of OD-SSB are ended.

On requirement 1), we think the simplest way is to re-interpret the field Ci in the bitmap field:

- When Ci=1, it means OD-SSB is activated in the SCell with SCellIndex i. And the UE applies its corresponding OD-SSB configuration for this SCell.

  - In this case, it is same as explicit deactivation.

- When Ci=0, the UE may have different interpretations depending on whether this SCell is working under explicit deactivation or implicit deactivation.

  - If this SCell was not configured with od-ssb-nrofBurst before reception of MAC-CE (i.e. explicit deactivation), it means OD-SSB deactivation in this SCell.

  - If this SCell was configured with od-ssb-nrofBurst before reception of MAC-CE (i.e. implicit deactivation), it means “keep the same OD-SSB configuration”. And the UE continues the OD-SSB transmission number counting.

On requirement 2), we think NW implementation can handle early deactivation via MAC-CE. For example, at least the following alternative ways can work:

- Alt-1: NW provides one configuration that od-ssb-nrofBurst is present but its value =0 (i.e. NW sends MAC-CE with Ci=1, and its configuration index points to one configuration with od-ssb-nrofBurst=0).

- Alt-2: NW first sends one MAC-CE to switch to explicit deactivation (i.e. Ci=1, and its indicated configuration with od-ssb-nrofBurst being absent) and then send another MAC-CE to explicit deactivate OD-SSB (i.e. Ci=0)

We can use one example to illustrate the solution in Figure.3 and Table.1.

Figure. 4 Example on how to use single MAC-CE format to support implicit deactivation

|  | T1 | T2 | T3 | T4 |
| --- | --- | --- | --- | --- |
| NW behavior | Send MAC-CE 1 | Send MAC-CE 2 | N/A | Send MAC-CE 3 |
| UE behavior on SCell1 | Regard OD-SSB is activated and under explicit deactivation mode (i.e. no counter) | Regard OD-SSB is explicitly deactivated | Regard OD-SSB is explicitly deactivated | Regard OD-SSB is deactivated |
| UE behavior on SCell2 | Regard OD-SSB is activated, set N=5 and start counter from n=0 | Regard OD-SSB is activated, reset N=7 and start counter from n=0 | Regard OD-SSB is activated and continue counter (n=4) | Regard OD-SSB is early deactivated because N=0. |
| UE behavior on SCell3 | Regard OD-SSB is activated, set N=8 and start counter from n=0 | Continue counter (n=4) | Regard OD-SSB is implicitly deactivated as n=N (N=8) | Regard OD-SSB is deactivated |

Table.1 Illustration of UE behavior in different time instances

We believe that it is the simplest solution to use one single MAC-CE format to support both explicitly deactivation and implicit deactivation. Thus, we propose:

Proposal 1: To support implicit deactivation of OD-SSB, UE re-interprets the bitmap field per SCell (i.e. Ci with SCellIndex i) in the following way:

- If Ci=1, the UE regards OD-SSB is activated in the concerned SCell, applies the corresponding OD-SSB configuration indicated in the MAC-CE (i.e. set/reset od-ssb-nrofBurst) and resets counter to 0.

- If Ci=0, the UE regards the OD-SSB configuration in the concerned SCell is not changed (i.e. od-ssb-nrofBurst is same as the one indicated by preceding MAC-CE) and continues the counter.

Proposal 2: If RAN1 agree to support early deactivation via MAC-CE before N transmissions of OD-SSB are ended, it is left to NW implementation (e.g., NW sends MAC-CE with Ci=1 and its configuration index points to one configuration with od-ssb-nrofBurst=0).

## 2.2 L3 measurement on OD-SSB

L3 measurement on OD-SSB was discussed in RAN2#127 [4] and RAN2#129 [5], and the following agreements were made:

Agreements on OD-SSB in RAN2#127

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB in RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

We discuss L3 measurement for case 1 and case 2, respectively.

### 2.2.1 How to support L3 measurement in case 1

In case 1, there is only OD-SSB in SCell but AO-SSB is not transmitted, which is simpler than case 2. In RAN2#127 [4], it was agreed that the UE doesn’t expect to measure SSB when OD-SSB is deactivated in case 1, i.e. the UE measures SSB only when on-demand SSB is activated.

Agreements on OD-SSB in RAN2#127

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

Since the activation / deactivation of OD-SSB transmission is indicated by MAC-CE / RRC, we think the simplest solution is described as follows:

- RRC based OD-SSB activation / deactivation

  - No specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

- MAC-CE based OD-SSB activation / deactivation

  - Specify the UE to start L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE, and the UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

    - Please note that we can’t reuse the solution of RRC based OD-SSB activation/deactivation because the UE is mandated to perform serving cell measurement when servingCellMO is configured, according to Section 5.3.3.1 of TS 38.331 [6].

From Section 5.3.3.1 of TS 38.331

omit unrelated part

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

omit unrelated part

Observation 2: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Thus, we propose:

Proposal 3: In L3 measurement in OD-SSB case 1, if RRC based activation / deactivation, no specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

Proposal 4: In L3 measurement in OD-SSB case 1, if MAC-CE based activation / deactivation, introduce the following RRC specifications changes:

- The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE

- The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

### 2.2.2 How to support L3 measurement in case 2

In case 2, both OD-SSB and AO-SSB are transmitted in SCell. Meanwhile, According to RAN1#120 agreement [7], we can further divide into the following two sub-cases:

- Sub-case 1: AO-SSB and OD-SSB have the same frequency

- Sub-case 2: AO-SSB and OD-SSB have different frequencies

#### 2.2.2.1 Sub-case 1: AO-SSB and OD-SSB have the same frequency

When AO-SSB and OD-SSB share the same centre frequency, the key issue of L3 measurement is whether either of them is measured or both are measured. In RAN2#129 [4], it was agreed to leave the study of case 2 to RAN4, but we believe the intention is only target for sub-case 1 (i.e. no intention to cover sub-case 2 as well).

Agreements on OD-SSB in RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

However, since Rel-19 NES will be close in August meeting, RAN2 needs to conclude this issue from RAN2 perspective, so that its running CR work can start. Our understanding on status in RAN1 and RAN4:

- RAN4 introduced a new fast window for L3 measurement for purpose SCell activation (i.e. during SCell activation, the UE performs L3 measurement based on SSB periodicity in the fast window and then follow legacy L3 measurement for deactivated SCell).

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

RAN1 agreed to support scenario#3B (i.e. after completion of SCell activation). And in this scenario, MAC-CE is used only to update the transmission parameter (e.g. OD-SSB periodicity).

Based on above status, we think RAN2 can progress in the following way:

Before completion of SCell activation (i.e. scenario 2/2A)

In this case, the L3 measurement is on deactivated SCell. Please note that TS 38.331 doesn’t capture the procedure text on L3 measurement on deactivated SCell [6]. Its only specification impact is in field description of measCycleSCell and it refers to TS 38.133 [8].

measCycleSCell

The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

Observation 3: Existing TS 38.331 doesn’t capture the procedure text on L3 measurement on deactivated SCell. Its only specification impact is in field description of measCycleSCell and it refers to TS 38.133.

Thus, we think RAN2 can assume the similar way can be reused, i.e. RAN2 leave to RAN4 on L3 measurement on deactivated SCell, and RAN2 assume no RAN2 specification impact is foreseen as legacy.

Proposal 5: When AO-SSB and OD-SSB share the same center frequency before completion of SCell activation (i.e. scenario 2/2A), RAN2 leave to RAN4 on L3 measurement on deactivated SCell, and RAN2 assume no RAN2 specification impact is foreseen as legacy.

After completion of SCell activation (i.e. scenario 3B)

In this case, we understand it is quite similar to L3 measurement of SSB adaptation because MAC-CE can only change transmission parameter of OD-SSB (e.g. periodicity of OD-SSB) but can’t deactivate OD-SSB.

Observation 4: After completion of SCell activation (i.e. scenario 3B), it is quite similar to L3 measurement of SSB adaptation because MAC-CE can only change transmission parameter of OD-SSB (e.g. periodicity of OD-SSB) but can’t deactivate OD-SSB.

Then, we think the similar solution of SSB adaptation with adaptation of multiple SMTC can be reused. Thus, we propose:

Proposal 6: When AO-SSB and OD-SSB share the same center frequency after completion of SCell activation (i.e. scenario 3B), the same L3 measurement solution of SSB adaptation is reused:

- Multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between OD-SSB configuration and SMTC configuration.

- Upon reception of OD-SSB MAC-CE, UE selects one SMTC configuration accordingly for L3 measurement on its serving cell(s).

- No spec impact on neighbor cell L3 measurement (i.e. based on legacy SMTC).

#### 2.2.2.2 Sub-case 2: AO-SSB and OD-SSB have different frequency

In RAN2#129b [2], it was agreed as WA to introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Meanwhile, one FFS is whether to allow multiple OD-SSBs with the different frequencies for a given SCell.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

First, we would like to confirm the WA. According to online discussion of RAN2#129b [2], we understand majority support the WA because another alternative (i.e. frequency of OD-SSB is added in servingCellMO identified by frequency of AO-SSB) will introduce significant specification impacts.

Observation 5: According to online discussion of RAN2#129b, majority supported the WA because another alternative (i.e. frequency of OD-SSB is added in servingCellMO identified by frequency of AO-SSB) will introduce significant specification impacts.

Thus, we propose to confirm the WA:

Proposal 7: Confirm the WA on MO configuration of OD-SSB when AO-SSB and OD-SSB have different center frequency.

Then, on the FFS of multiple OD-SSBs with different frequencies, we think it is one RAN1 issue. According to our discussion with RAN1 colleague, this issue was not discussed in RAN1. To start work of drafting running CR, we think RAN2 can assume up to one OD-SSB with different frequency can be configured for a given SCell. This assumption can be re-visited if RAN1 make different conclusion.

Proposal 8: In running CR drafting, RAN2 assume that up to one OD-SSB with different frequency from AO-SSB can be configured for a given SCell. This assumption can be re-visited if RAN1 make different conclusion.

Finally, another important remaining issue is when the UE applies the new servingCellMO for L3 measurement on OD-SSB. We think one reasonable way is to apply it when OD-SSB is activated. Otherwise (i.e. when OD-SSB is deactivated), the UE applies the legacy servingCellMO. Thus, we propose:

Proposal 9: The UE applies the new servingCellMO for L3 measurement when OD-SSB is activated. Otherwise (i.e. when OD-SSB is deactivated), the UE applies the legacy servingCellMO for L3 measurement.

### 2.2.3 Measurement reporting

During RAN2#129b [2], some company proposed to introduce a new measurement reporting triggered by OD-SSB MAC-CE. The motivation is to trigger fast measurement reporting when OD-SSB is activated, which can reduce the transition time from unknown SCell to known SCell.

We can understand the motivation, but we disagree to introduce such enhancement in late stage of Rel-19:

- The same motivation (i.e. reduce the transition time from unknown SCell to known SCell) can be achieved by legacy measurement reporting triggered by SCell A/D MAC-CE introduced in Rel-18. The extra benefit on top of existing mechanism is not clear to us

- OD-SSB MAC CE is expected to be sent earlier than SCell activation MAC-CE, and it will cause some unpredictable impacts to RAN4 work on the legacy L3 measurement reporting during SCell activation. It should be avoided in late stage of Rel-19.

Observation 6: Measurement reporting triggered by OD-SSB MAC-CE will lead to below issues:

- The same motivation (i.e. reduce the transition time from unknown SCell to known SCell) can be achieved by legacy measurement reporting triggered by SCell A/D MAC-CE introduced in Rel-18.

- OD-SSB MAC CE is expected to be sent earlier than SCell A/D MAC-CE, and it will cause some unpredictable impacts to RAN4 work on the legacy L3 measurement reporting during SCell activation.

Thus, we propose:

Proposal 10: RAN2 not pursue measurement reporting triggered by OD-SSB MAC-CE.

# 3 Conclusion

In this contribution, we discuss remaining issues on-demand SSB for SCell. Our observations are:

Observation 1: According to RAN1 agreement, the requirements for MAC-CE on implicit deactivation include:

- The UE determines whether implicit deactivation is enabled by checking if od-ssb-nrofBurst is present in the OD-SSB configuration indicated by the configuration index included in the MAC-CE.

- RAN1 may support early deactivation via MAC-CE before N transmissions of OD-SSB are ended.

Observation 2: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Observation 3: Existing TS 38.331 doesn’t capture the procedure text on L3 measurement on deactivated SCell. Its only specification impact is in field description of measCycleSCell and it refers to TS 38.133.

Observation 4: After completion of SCell activation (i.e. scenario 3B), it is quite similar to L3 measurement of SSB adaptation because MAC-CE can only change transmission parameter of OD-SSB (e.g. periodicity of OD-SSB) but can’t deactivate OD-SSB.

Observation 5: According to online discussion of RAN2#129b, majority supported the WA because another alternative (i.e. frequency of OD-SSB is added in servingCellMO identified by frequency of AO-SSB) will introduce significant specification impacts.

Observation 6: Measurement reporting triggered by OD-SSB MAC-CE will lead to below issues:

- The same motivation (i.e. reduce the transition time from unknown SCell to known SCell) can be achieved by legacy measurement reporting triggered by SCell A/D MAC-CE introduced in Rel-18.

- OD-SSB MAC CE is expected to be sent earlier than SCell A/D MAC-CE, and it will cause some unpredictable impacts to RAN4 work on the legacy L3 measurement reporting during SCell activation.

Based on above observations, our proposals are:

MAC-CE format

Proposal 1: To support implicit deactivation of OD-SSB, UE re-interprets the bitmap field per SCell (i.e. Ci with SCellIndex i) in the following way:

- If Ci=1, the UE regards OD-SSB is activated in the concerned SCell, applies the corresponding OD-SSB configuration indicated in the MAC-CE (i.e. set/reset od-ssb-nrofBurst) and resets counter to 0.

- If Ci=0, the UE regards the OD-SSB configuration in the concerned SCell is not changed (i.e. od-ssb-nrofBurst is same as the one indicated by preceding MAC-CE) and continues the counter.

Proposal 2: If RAN1 agree to support early deactivation via MAC-CE before N transmissions of OD-SSB are ended, it is left to NW implementation (e.g., NW sends MAC-CE with Ci=1 and its configuration index points to one configuration with od-ssb-nrofBurst=0).

L3 measurement in NES case 1

Proposal 3: In L3 measurement in OD-SSB case 1, if RRC based activation / deactivation, no specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

Proposal 4: In L3 measurement in OD-SSB case 1, if MAC-CE based activation / deactivation, introduce the following RRC specifications changes:

- The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE

- The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

L3 measurement in NES case 2 when AO-SSB and OD-SSB share the same center frequency

Proposal 5: When AO-SSB and OD-SSB share the same center frequency before completion of SCell activation (i.e. scenario 2/2A), RAN2 leave to RAN4 on L3 measurement on deactivated SCell, and RAN2 assume no RAN2 specification impact is foreseen as legacy.

Proposal 6: When AO-SSB and OD-SSB share the same center frequency after completion of SCell activation (i.e. scenario 3B), the same L3 measurement solution of SSB adaptation is reused:

- Multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between OD-SSB configuration and SMTC configuration.

- Upon reception of OD-SSB MAC-CE, UE selects one SMTC configuration accordingly for L3 measurement on its serving cell(s).

- No spec impact on neighbor cell L3 measurement (i.e. based on legacy SMTC).

L3 measurement in NES case 2 when AO-SSB and OD-SSB have different center frequency

Proposal 7: Confirm the WA on MO configuration of OD-SSB when AO-SSB and OD-SSB have different center frequency.

Proposal 8: In running CR drafting, RAN2 assume that up to one OD-SSB with different frequency from AO-SSB can be configured for a given SCell. This assumption can be re-visited if RAN1 make different conclusion.

Proposal 9: The UE applies the new servingCellMO for L3 measurement when OD-SSB is activated. Otherwise (i.e. when OD-SSB is deactivated), the UE applies the legacy servingCellMO for L3 measurement.

Measurement reporting

Proposal 10: RAN2 not pursue measurement reporting triggered by OD-SSB MAC-CE.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129b, Chair Notes.

[3] RAN1#120b, Chair Notes.

[4] RAN2#129, Chair Notes.

[5] RAN2#127, Chair Notes.

[6] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[7] RAN1#120, Chair Notes.

[8] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.