3GPP TSG-RAN WG2 Meeting #130	R2-2503725

# St. Julians, Malta, 19-23 May 2025

Agenda item:	8.13.2

Source:	Apple

Title:	Relay discovery and selection for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2#127 [1] has made the following agreements on relay discovery and selection:

- From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

RAN2#128[2] meeting has further reached the following agreements for relay discovery and selection:

- A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. - If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. - The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. - The following cases are supported for L2 multi-hop relay: - - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. - FFS if the last relay UE can use the same L2ID for both of the connections in either case. - Cases with two indirect paths to the gNB for the same remote UE are not supported. - An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. - RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

RAN2#129 [3] has made the following agreements on relay discovery and selection:

- The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell. FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive. - From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment. - Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. - Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay. - For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). - For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established - For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. - For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. - L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3.

RAN2#129bis [4] has made the following agreements on relay discovery and selection:

- The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop. - If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration. If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages. - For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold. - For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE. FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first. - For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE. - For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. - Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected). - When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. - RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item. This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI. - When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. - FFS detailed information in the indication and child UE handling. - FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. - FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In this paper, we continue to discuss some controversial issues and FFS(s) in relay discovery and selection aspects for Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 Remaining issue for Model B discovery

For Model B discovery, there is still an FFS “FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.”, which is related to last relay UE need check AS conditions and make a decision  on whether to send discovery response to the intermediate relay UE or not.  “

Not sure what “the intermediate relay UE always establishes itself as a remote UE first“ exactly mean, but if the intermediate relay UE receives a Model B discovery request message, the right behavior for the intermediate relay UE in model B is to re-broadcast a Model B discovery request message, instead of generating its own discovery request message as a remote UE. Because the last relay UE may accept a single-hop remote, but may not want to support remote UE multiple hops away. Therefore, executing a single-hop Model B discovery procedure does not help much in this case, and the intermediate relay UE still need to re-broadcast remote UE’s model B discover request to see if there is a nearby relay willing to respond. Therefore, I think the smart intermediate relay implementation will not commit itself to a certain relay UE as parent node before receiving any remote UE’s request yet. Also, it would be too restrictive to assume an intermediate relay UE, as a candidate itself, will only lock itself to a single relay and only forward the response message from a single (last) relay, that will largely suppress the likelihood of the success of model-B discovery.

Proposal 1 	RAN2 confirms that there is valid case for the last relay UE to check AS conditions before responding to intermediate relay UE’s model B discovery request.

### 2.2 IDLE/INACTIVE Relay UE reselection

Based on latest spec of 24.544 [5] for ProSe Relay, the remote UE discovers the full (candidate) path during model-B discovery procedure as the full path information is included in model-B discovery response message. If the IDLE/INACTVE relay UE has conducted a relay reselection, even if it is still under the same serving cell without changing the hop count, the full path has been changed and not as same as the one conveyed in the earlier discovery message. Hence, we think, such a notification is always needed in regardless of whether the cell or hop count changes or not.

Proposal 2 	No suppression of indication is allowed when the IDLE/INACTIVE intermediate relay UE reselection under the same serving cell without changing the hop count.

Then, regarding how the notification has been sent. This can be done either with a unicast message to the downstream node or send as a broadcast message. Note that the relay reselection may occur w/ or w/o PC5 link establishment from the relay UE to its downstream node.

Before the PC5 link is established: In this case, the relay UE has not received SI/Paging forward request yet, so the relay UE may not sure the remote UE has select itself as a forwarder in the selected path. As it is still just a candidate relay UE, it will need to update this information with discovery-like signaling, which should be a broadcast message

It is also worth noting that the relay UE should not just wait for the reception of discovery solicitation to send updated path information in a discovery response. Because the remote UE does not know the relay reselection occurrence, it will not send a solicitation again. As a result, then there is no way for this intermediate relay UE to update the information with discovery message.

Proposal 3 	When PC5 link is not established, the IDLE/INACTIVE relay UE send a broadcast signaling to notify the downstream nodes, which should be part of discovery message design.

After the PC5 link is established: in this case, the relay UE has already been selected by the downstream node which initiated the PC5 link setup. So, it is logical for the relay UE to deliver a PC5 unicast transmission to send this notification. A possible candidate for this is the PC5-RRC message NotificaitonMessageSidelink

Proposal 4 	When PC5 link is already established, the IDLE/INACTIVE relay UE send a NotificaitonMessageSidelink via unicast to notify the downstream nodes about the relay UE reselection.

### 2.3 Resource allocation for discovery transmission

Regarding the FFS in the following agreement [1]:

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

We think for remote UE in Multi-hop R19 scenario, it does not have a direct Uu link and it cannot receive DCI signal transmitted by gNB. Therefore, mode 1 resource allocation is anyway crippled this remote UE as not all mode 1 mechanisms are working. It is much simpler to not support mode 1 and only allow remote UE to use resource allocation mode 2.

It is also worth clarifying that the agreement “Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs;” means for legacy R17/R18 relay which has to be the last relay UE. For intermediate relay UE which connects to the gNB via multiple hops, mode 1 support is also questionable. Hence, a much cleaner design is to only have mode 2 resource allocation for both remote UE and intermediate relay UEs.

Proposal 5 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

# 3 Conclusion

In this contribution, we discuss the Relay discovery and selection for Multi-hop UE-to-NW Relay, and have the following proposals:

Proposal 1 	RAN2 confirms that there is valid case for the last relay UE to check AS conditions before responding to intermediate relay UE’s model B discovery request.

Proposal 2 	No suppression of indication is allowed when the IDLE/INACTIVE intermediate relay UE reselection under the same serving cell without changing the hop count.

Proposal 3 	When PC5 link is not established, the IDLE/INACTIVE relay UE send a broadcast signaling to notify the downstream nodes, which should be part of discovery message design.

Proposal 4 	When PC5 link is already established, the IDLE/INACTIVE relay UE send a NotificaitonMessageSidelink via unicast to notify the downstream nodes about the relay UE reselection.

Proposal 5 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

# 4 References

[1] Chairman’s notes, RAN2#127

[2] Chairman’s notes, RAN2#128

[3] Chairman’s notes RAN2#129

[4] Chairman’s notes RAN2#129bis

[5] TS 24.554 v19.1.0