3GPP TSG-RAN WG2 Meeting #130	R2-2503736

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item	: 8.1.4

Source	: LG Electronics Inc.

Title	: Discussion on UE-side data collection

Document for	: Discussion and Decision

Introduction

At the RAN#106 meeting, the new SID for AI/ML air Phase 2(RP-243245) [1] was agreed and the further discussion for UE-side data collection was approved in RP-243307[2] with the following guidance for RAN2 during Rel-19:

- Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

In this contribution, we will continue discuss the topics assigned to RAN2 and left FFS in previous RAN2#129bis meeting [5].

Discussion

Configuration of UE side data collection

Based on the RAN2#129 agreement [4], it is now clear that NG-RAN is responsible for the control and configuration of UE-side data collection for all RAN-centric use cases (e.g., beam management, CSI prediction). For both CP and UP solutions, configuring radio resources for UE-side data collection is essential. There is no specific reason to configure RS differently for each method, nor is there any need for unnecessary variations in the settings. Therefore, the RS configuration for methods aimed at UE-side data collection should be consistent across both CP and UP solutions.

Proposal 1: The RS configuration for UE-side data collection must be consistent across both CP and UP solutions.

Since the configurations for both UE-side and NW-side data collection are related to CSI reporting configuration, the RS configuration framework used for NW-side data collection (including associated IDs if needed) can be applied. This approach will help avoid redundant discussions.

Proposal 2. RS configuration framework for NW-side data collection can be reused for RS configuration framework for UE-side data collection.

Open issue RRC-3: UE data collection request

According to the agreements made in RAN2#129bis meeting regarding the data collection configuration [5], the details of signaling of preferred configuration is left as FFS.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

According to the RAN1 agreement, a CSI-ReportConfig without an actual CSI report can be configured for UE data collection purposes. In support of this, the UE may initiate requests for RS configuration in two possible ways:

- RS request based on configured CSI report configuration used for data collection

- RS request based on configured CSI related configuration

A similar discussion is observed in the context of applicability report. In ongoing RAN1 and RAN2 discussions, two options are being considered for how a UE may inform the network of preferred CSI-related configuration:

- Option A, which informs the preferred/applicable CSI report configuration

- Option B, which requests for the preferred CSI report configuration.

Observation 1. For requesting the UE preferred RS configuration, a similar discussion is ongoing in the context of the applicability report.

Therefore, it is worth considering whether the mechanism used for applicability reporting could also be reused for UE-initiated RS configuration requests for data collection purposes.

In Option A, where the UE informs the network of applicable configurations among already configured CSI report configurations, the same mechanism could serve as a simple approach to indicate a preferred RS configuration. In contrast, Option B, which allows the UE to request a specific RS configuration, would likely require further input from RAN1 regarding how such a request can be supported within the CSI framework.

Following the RAN1#121 meeting, RAN1 has initiated further offline discussions [5], leading to the consideration of two additional detailed options under Option B.

- Option1, which involves providing information related to Set A and Set B for inference, specifically the associated IDs for Set A and Set B (AssociatedIdforSetA-r19 / AssociatedIdforSetB-r19) and/or size of Set A and Set B (as a new RRC parameter)

- Option2, which involves providing information related to Set A and Set B for inference, specifically the associated IDs for Set A and Set B (AssociatedIdforSetA-r19 / AssociatedIdforSetB-r19) and/or the resources for Set A and Set B (resourceForSetA-r19 / resourceForSetB-r19)

For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. Therefore, while the detailed signaling for Option B depends on further RAN1 input, the Option B mechanism could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection.

Observation 2. In Option A, the UE informs the network of applicable configurations among already configured CSI report configurations. The same mechanism could serve as a simple approach to indicate a preferred RS configuration for the UE request.

Observation 3. For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. Option B could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection.

Issue. Where/what the NW provides as candidate configurations

Issue. What the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.)

In addition to the flag indicating that UE requests are allowed, the network may also provide RS configuration-related settings. The method and parameters by which the network provides candidate configurations for data collection depend on the selected option, as described below.

- For Option A like mechanism, NW will provide the full RS configuration used for data collection through CSI-ReportConfig, which may be identical to that NW provides full inference configuration through CSI-ReportConfig. Upon receiving a full RS configuration used for data collection, the UE may inform the list of preferred RS configuration among which NW already configured via RRCReconfigurationComplete message or UAI.

- On the other hand, for Option B like mechanism, the NW will provide the parameters related to the data collection through DataCollectionPreferenceConfig-r19 in OtherConfig. Based on the parameter information received from the NW, the UE requests a specific RS configuration that it prefers, and the NW configures CSI-ReportConfig to provide the specific RS configuration requested by the UE.

Issue. How to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

To indicate a preferred RS configuration in UAI, the following methods can be adopted for each option:

- For Option A like mechanism, the UE may inform the list of preferred RS configurations among which NW already configured CSI report configuration used for data collection. For the list of preferred RS configurations, an IE similar to the ApplicabilityReportList-r19, which is defined for applicability report, may be defined and used.

- For Option B like mechanism, the UE may inform the specific preferred RS configuration (e.g., ID of DataCollectionPreferredConfiguration-r19) based on the further input for applicability report from RAN1 regarding the CSI framework.

Proposal 3. For the UE request configuration, the RS configuration for the applicability report can be re-used

- Option A-like mechanism: The network provides a full RS configuration for data collection via CSI-ReportConfig. The UE indicates its preferred RS configuration(s) from among the already configured CSI report configurations.

- Option B-like mechanism: The network provides data collection-related parameters via OtherConfig. The UE explicitly indicates a preferred RS configuration. Further RAN1 input is needed to support Option B.

Data transfer over UP

According to the RAN2#127 meeting [3], data transmission via the UP tunnel is considered for Option 1a and Option 1b, while it remains as FFS for Option 2 and Option 3. The table below illustrates a baseline example of the data transmission process through the UP tunnel for each option, depending upon the location of the server for UE data collection, whether it is located inside or outside the MNO. As shown in the table, when the server is located inside the MNO, the pathways of data transfer and the MNO's scope for Option 1b and Option 2 are identical. Likewise, when the server is located outside the MNO, the pathways of data transfer and the MNO's scope for Option 1a, Option 1b, and Option 2 are identical.

| Location of the server Option | Server inside the MNO | Server outside the MNO |
| --- | --- | --- |
| Option 1a | N/A |  |
| Option 1b |  |  |
| Option 2 |  |  |
| Option 3 |  |  |

From the RAN2 perspective, the UP tunnel facilitates data transmission from UE to the Core Network via the gNB and UPF. Consequently, the UP solutions for Option 1a, Option 1b, and Option 2 appear to be equivalent, irrespective of the location of the server. In contrast, Option 3 presents a different scenario, as the UP tunnel for this option routes data from the UE to the Core Network through the gNB and OAM, thereby distinguishing it from the other options.

As a result, the data transmission through the UP tunnel can be categorized into two distinct groups from the RAN2 perspective, based on the above analysis.

- data transfer through UP tunnel via UPF (i.e., Option 1a, Option 1b, Option 2)

- data transfer through UP tunnel via OAM (i.e., Option 3)

Proposal 4. From the RAN2 perspective, UP solution can be classified into two : i) data transfer through UP tunnel via UPF(i.e., Option1a, Option1b, Option2), ii) data transfer through UP tunnel via OAM(i.e., Option3).

Discussion on the controllability aspects of CP&UP solutions

In terms of controllability, the MNO ensures full control of the CP solution, as it fully manages the transmission of SRBs. This extensive control empowers the MNO to efficiently handle communications, optimize resource allocation, and ensure high-quality service for users. By managing SRB transmission, the MNO can dynamically adjust parameters, handle signaling traffic, and respond in real-time to fluctuating network conditions, thereby significantly enhancing overall network performance and reliability.

Similarly, with respect to the UP solution, the NG-RAN also maintains full control, as the SMF within the core network takes charge of controlling the UPF and OAM. Whether the data collection server is located inside or outside the MNO, the MNO can achieve complete controllability by differentiating AIML data from non-AIML data, thus establishing separate PDU sessions with unique LCH, DRB, and QoS requirements.

Proposal 5. MNO can have full controllability over the data collection procedure through both CP solution and UP solution.

Discussion on the visibility aspects of CP&UP solutions

In addition to its controllability features, the NG-RAN also ensures full scalability for the transmission of standardized data through the CP solution, as the management of data transmission via SRBs falls under NG-RAN's purview. Similarly, the UP solution within the core network guarantees full scalability for standardized data transfer, given that each PDU session is under the control of the SMF. Moreover, full visibility for standardized data collection is already achievable in legacy operations, as outlined in the spec related to other relevant working groups. For instance, the TS 26.531 specifies that when the data collection server is located inside the MNO, full visibility for standardized data can be assured for UE-side model training. In this context, it is anticipated that full visibility for standardized AIML data will also be achievable.

Proposal 6. For both CP solution and UP solution, MNO can have full visibility over standardized data.

During the discussions at the RAN2#127 meeting regarding various data collection options for UE-side model training [7], it was noted that "RAN2 has not reached a conclusion on the necessity of partial and no visibility options.". To address these options effectively, it is essential to first define the content of partially standardized data and non-standardized data. Some companies advocate for the inclusion of partial or non-standardized data to protect proprietary information. However, the existing data transfer requirements, such as L1-RSRP values, are sufficiently fulfilled with standardized data. To advance discussions on partial and non-standardized data, it is essential to clarify which data types should be classified as partial or non-standardized and to specify the information that must be included for data transfer purposes. Should this clarification indicate a need for the introduction of partial or non-standardized data, further discussions can be scheduled at that time.

Proposal 7. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

According to the objective of the new SID (SP-250413) from SA plenary on March [9] and key issue #1 of TR23.700-04 [10], SA is only considering support for the standardized transfer of standardized data over the UP for UE data collection. Therefore, it would be prudent for RAN2 to also focus solely on standardized data during the Rel-19 discussion period, as SA is not considering partial or non-standardized data during this time.

Proposal 8. For Rel-19, RAN2 only focuses on standardized data, disregarding partial/non-standardized data.

Discussion on the scalability aspects of CP solution

From the RAN2 perspective, the data transfer through the CP tunnel can be classified into two categories:

- CP solution via NAS from UE to AMF (i.e., Option 2)

- CP solution via AS from UE to gNB to OAM (i.e., Option 3)

  - The current MDT framework can be taken as a baseline for the Option3. Therefore, it shares the advantages of MDT, such as cost efficiency, extensive coverage, substantial data collection capability, real-time data acquisition, and user-centric analysis for precise network optimization.

Both the AS protocol and the NAS protocol are not ideally suited for transferring large volumes of data, which is required for AIML purposes. This limitation arises from the fact that both AS and NAS ultimately rely on RRC messages, which are constrained to a maximum size of 9000 bytes. Consequently, transferring substantial amounts of data necessitates multiple RRC transmissions, resulting in increased signaling overhead. Moreover, it is crucial to specify the data type for each use case during transmissions via both AS and NAS. This requirement introduces scalability challenges, as any new use cases arising from advancements in AI/ML technology will necessitate updates to the UE to support these new use case-specific data types. Additionally, the network infrastructure must be fully deployed or updated to support these new data types.

In contrast, the UP solution can effectively address these limitations by replacing the CP solution. This can be achieved by configuring legacy PDU sessions without requiring enhancements. The transfer of large volumes of AIML data can then be managed similarly to any other UP traffic. Based on the configuration of MNO, the network can establish a PDU session and manage this traffic with specific QoS flows and related QoS parameters. At this point, it is essential to ensure that the transmission of AIML data, which may have relaxed latency requirements, does not interfere with the transmission of other UP traffic. Therefore, considerations such as dedicated bearers may be explored, since they do not impose significant impacts on the specifications.

Proposal 9. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

Conclusion

Based on the above discussion, we made the following observations and proposals:

Configuration of UE side data collection

Proposal 1. The RS configuration for UE-side data collection must be consistent across both CP and UP solutions.

Proposal 2. RS configuration framework for NW-side data collection can be reused for RS configuration framework for UE-side data collection.

Observation 1. For requesting the UE preferred RS configuration, a similar discussion is ongoing in the context of the applicability report.

Observation 2. In Option A, the UE informs the network of applicable configurations among already configured CSI report configurations. The same mechanism could serve as a simple approach to indicate a preferred RS configuration for the UE request.

Observation 3. For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. Option B could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection.

Proposal 3. For the UE request configuration, the RS configuration for the applicability report can be re-used.

- Option A-like mechanism: The network provides a full RS configuration for data collection via CSI-ReportConfig. The UE indicates its preferred RS configuration(s) from among the already configured CSI report configurations.

- Option B-like mechanism: The network provides data collection-related parameters via OtherConfig. The UE explicitly indicates a preferred RS configuration. Further RAN1 input is needed to support Option B.

Data transfer over UP

Proposal 4. From the RAN2 perspective, UP solution can be classified into two : i) data transfer through UP tunnel via UPF (i.e., Option1a, Option1b, Option2), ii) data transfer through UP tunnel via OAM (i.e., Option3).

Discussion on the controllability aspects of CP&UP solutions

Proposal 5. MNO can have full controllability over the data collection procedure through both CP solution and UP solution.

Discussion on the visibility aspects of CP&UP solutions

Proposal 6. For both CP solution and UP solution, MNO can have full visibility over standardized data.

Proposal 7. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

Proposal 8. For Rel-19, RAN2 only focuses on standardized data, disregarding partial/non-standardized data.

Discussion on the scalability aspects of CP solution

Proposal 9. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

References

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- R2_127_ChairNotes_24-08-23_17-00_eom.docx

- R2_129_ChairNotes_25-02-21_final.docx

- R2_129b_ChairNotes_25-04-11_16-45 final.docx

- RE: /AIML for BM RAN 1 #121/ RRC parameter for Option B

- R2-2407785

- RRC open issues AIML PHY_v14_Apple2.docx

- SP-250413, “New SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2”, Mar 2025

- TR23.700-04, “Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML); Phase2”