3GPP TSG-RAN WG2 Meeting #130	R2-2503748

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:		8.8.4 Support of broadcast service

Title:		Remaining issues on the support of broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behaviour is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

During the 129th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list. 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH). 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented).

During the 129bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. 4. The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on support of broadcast service in NTN.

# 2. Discussion

2.1	Mapping between ISA and Session ID

During the 127th meeting of RAN WG2, it was agreed that each MBS service shall include one or more Intended Service Area (ISA) IDs in MCCH. In the 129th meeting, it was further agreed that it shall be possible to signal multiple service area IDs to one MBS service (we insert a list of service area IDs in MCCH).

In this regard, the following two approaches have been considered for representing the mapping between TMGI and ISA in the MCCH:

- To include the mapping in the MBSBroadcastConfiguration IE

- To extend the MBS-SessionInfoList.

While the first solution is relatively simple and extensible, it may lead to TMGI duplication, increased signalling overhead, and complexity—particularly in cases where multiple sessions share the same ISA, or where a single MBS session is broadcast over multiple frequencies or cells.

Therefore, ETRI prefers the second approach, where the mapping between TMGI and ISA ID is handled by extending the MBS-SessionInfoList. This allows each MBS session to explicitly list its associated ISAs, enabling:

- One-to-many and many-to-one mapping

- Avoidance of TMGI repetition

- Easier session-specific area identification at the UE.

Proposal 1: RAN WG2 to extend the MBS-SessionInfoList to support the mapping between TMGI and ISA ID.

2.1	Service Continuity

At the 129bis meeting, the following agreements were reached regarding service continuity for MBS in NTN:

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

In large-scale or regionally fragmented MBS deployments, the current MBS architecture might allow decoupling of the MBS session ID from the physical layer carrier—meaning the same MBS session can be broadcast over different carrier frequencies in neighboring cells. This enables operators to optimize frequency reuse, comply with regional spectrum allocations, and ensure service continuity for UEs moving across areas served by different frequencies.

As NTN deployments introduce multi-beam, multi-frequency service regions with overlapping footprints and independently scheduled beams, a single MBS session identified by a TMGI may be broadcast over multiple frequencies in adjacent or overlapping service areas—following the same architectural principle already applied in TN-based MBS..

Observation 1: A single MBS service can be broadcast over different carrier frequencies in NTN.

Based on this, ETRI believes that for effective service continuity, the ISA IDs of neighbouring cells should also be provided by the serving cell via a new SIB that defines the ISAs. Furthermore, the association between ISAs and frequencies should be made explicit.

Proposal 2: RAN WG2 to confirm that the list of ISAs for neighbouring cells should be provided by the serving cell via a new SIB.

Proposal 3: RAN WG2 to agree that ISA IDs should be associated with carrier frequencies to efficiently support service continuity.

There are two possible approaches for defining a new SIB that provides ISA information:

- Option 1: Area-specific SIB

  - The same ISAs are shared across a serving cell and its neighbouring cells, resulting in low signalling overhead.

  - However, ISAs cannot be distinguished per frequency or per cell, resulting in reduced system flexibility.

  - Regarding association with frequency:

    - The same MBS content may need to be provisioned as different MBS services (i.e., different TMGIs) across cells and frequencies, due to a lack of explicit association between ISA and frequency.

    - Consequently, reusing the same TMGI over multiple frequencies is not supported, which limits service continuity and session management consistency.

- Option 2: Cell-specific SIB

  - Each cell may broadcast a different ISA configuration, providing greater system flexibility, albeit with higher signalling overhead.

  - ISAs can be clearly distinguished per cell or per frequency.

  - Regarding association with frequency:

    - Option 2-1: To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs. This approach would involve modifying SIB21 to include:

      - The ISA ID(s) per each carrier frequency

    - Option 2-2: To utilize intended service area information of neighbouring cells from a new SIB and MCCH. This approach would include:

      - The ISA IDs of neighbouring cells in MBS-NeighbouringCellList

In case of option 1, the area-specific SIB assumes that the same set of ISAs is shared among a serving cell and its neighbouring cells. This design can be advantageous in terms of signalling efficiency, as a common SIB instance may be reused across multiple cells. However, this approach brings several limitations. Specifically, it becomes difficult to distinguish ISA configurations on a per-cell or per-frequency basis. This reduces system flexibility and complicates precise service area mapping, especially when different frequencies are used in adjacent areas. Moreover, under Option 1, there is no explicit association between the ISAs and the carrier frequencies. This limitation often leads to situations where the same MBS content needs to be provisioned as separate MBS sessions—i.e., different TMGIs—across different frequencies or cells. Consequently, a single MBS session identified by the same TMGI cannot be consistently maintained over multiple frequencies, limiting the support for service continuity and efficient TMGI reuse.

In contrast, option 2 allows each cell to independently broadcast its own ISA configuration. This enables a network to provide different ISAs for each cell and for each frequency layer. As a result, system flexibility is significantly increased. Networks can apply more fine-grained control over service areas and MBS session mapping, including frequency-aware configurations. However, this flexibility comes at the cost of higher signalling overhead, as each cell must individually broadcast its own SIB instance. To support association between ISAs and frequencies within Option 2, two complementary mechanisms are proposed:

The first one is to define a direct mapping between carrier frequencies and ISA IDs within SIB21. This would involve modifying SIB21 to include a new structure that lists the ISA ID(s) associated with each broadcast frequency. This enhancement would enable the UE to understand, within the context of a single cell, how ISAs relate to the current and/or neighbouring frequencies. It also supports reusing the same TMGI across multiple frequencies without duplication of MBS session IDs. The second way is to utilize neighboring cell ISA information conveyed through a new SIB or via the MCCH. In this approach, the network provides a list of neighboring cells, their respective frequencies, and the associated ISA IDs. This information can be encapsulated in a structure such as MBS-NeighbouringCellList. It enables the UE to anticipate which MBS services are available in surrounding cells and supports proactive cell re-selection based on MBS service continuity needs.

In summary, while Option 1 minimizes signaling, it limits the ability to distinguish ISAs per frequency and prevents the reuse of TMGIs across carriers. In contrast, Option 2 increases system flexibility and supports advanced session continuity features, especially in complex NTN scenarios.

ETRI therefore recommends Option 2 (Cell-specific SIB), specifically enhanced with Option 2-1 (ISA-to-frequency mapping in SIB21) for precise and scalable signaling.

Proposal 4: RAN WG2 to confirm that a new SIB defining the ISAs should be cell-specific.

Proposal 5: For service continuity, RAN WG2 to decide to introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21

# 3. Conclusions

Proposal 1: RAN WG2 to extend the MBS-SessionInfoList to support the mapping between TMGI and ISA ID.

Observation 1: A single MBS service can be broadcast over different carrier frequencies in NTN.

Proposal 2: RAN WG2 to confirm that the list of ISAs for neighbouring cells should be provided by the serving cell via a new SIB.

Proposal 3: RAN WG2 to agree that ISA IDs should be associated with carrier frequencies to efficiently support service continuity.

Proposal 4: RAN WG2 to confirm that a new SIB defining the ISAs should be cell-specific.

Proposal 5: For service continuity, RAN WG2 to decide to introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21.