3GPP TSG-RAN WG2 Meeting #130	 R2-2503776

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.3

Source:	MediaTek Inc.

Title:	Further Discussion on Network-Side Data Collection

Document for:	Discussion, Decision

Introduction

The email discussion titled "[POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)" identifies the remaining open issues related to RRC for AI PHY features. This contribution aims to address the identified open issues specifically related to network-side data collection.

Discussion

  - Open issue RRC-7: NW control on retaining logged data at HO

In RAN2#129bis meeting, we made following agreements on network-side data collection on HO and RRC station transition:

Agreements on Idle/inactive and HO Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

When the UE is about to perform HO, the source gNB should coordinate with the target gNB regarding the unretrieved data during the handover preparation phase via HANDOVER PREPARATION REQUEST/RESPONSE messages. The process involves the following steps:

- The source gNB informs the target gNB that the UE has unretrieved data. The source gNB coordinates with the target gNB to determine if the unretrieved data can be retrieved and forwarded by the target gNB. Based on the coordination, the source gNB decides and informs the target gNB whether to release or retain the unretrieved data.

- The target gNB sends the indication to the UE in the RRCReconfiguration message.

Proposal 1: RAN2 assumes that the source gNB and the target gNB will coordinate on whether to release or retain the unretrieved data for the UE during the HO preparation phase.

Proposal 2: The source gNB decides whether to release or retain the unretrieved data based on the coordination with the target gNB. The target gNB then sends the indication to the UE in the RRCReconfiguration message.

  - Open issue RRC-8: Reporting assistance information related to logged measurements

It is not yet clear what the OtherConfig should contain to enable the UE to report assistance information via UAI related to the logging of radio measurements. Specifically, it is uncertain whether indications such as low power, buffer full, and buffer threshold reached should be considered jointly or separately.

We believe that a single SetupRelease flag is sufficient to enable the UE to report assistance information related to logged measurements. There is no need to consider low power, buffer full, and buffer threshold reached separately. We do not see a strong motivation for the network to have the flexibility to configure these factors individually.

If the flag is set to "setup," the data availability indication via UAI can be triggered by any of the following conditions: low power, buffer full, or buffer threshold reached. This approach simplifies the configuration and ensures that the UE can effectively report assistance information without unnecessary complexity.

Proposal 3: Use a single SetupRelease flag to enable the UE to report assistance information related to logged measurements.

  - Open issue RRC-9: Further procedures for UE assistance information related to logging

In RAN2#129bis meeting, we made following agreements on data availability indication:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

The availability indication can be triggered by buffer status (full buffer or buffer threshold being reached) or power status (low power), which are two separate triggers for UAI.

For buffer status:

The data availability indication is intended to inform the network that there is a certain amount of data stored, specifically when the full buffer or buffer threshold is reached. The network's behavior in response to this trigger is to retrieve the logged data to relieve the occupation of the buffer. Explicit data availability indication is not necessarily needed, as the reason for the trigger (full buffer or threshold) already implies data availability.

Given that the total buffer size has been reported as part of the UE capability, the network can estimate buffer occupation during the data retrieval process. There is no need for additional procedures or indications to inform the network that the buffer is no longer full.

For power state:

RAN2#128 agreed that the UE should report to the network when the power state is low. The network should then de-configure the data collection, as the UE will not autonomously stop data logging when a low power state is detected. The low power state indication is not intended to indicate whether data is available or not. Instead, it indicates that the UE is in a low power state and may not be able to start or continue data logging based on network configuration. The network's following behavior is to de-configure the data collection procedure.

In this case, an additional indication of data availability could be useful. If there is data available, even lower than the threshold or full buffer, an additional indication can help the network decide whether to retrieve the data before de-configuring the data collection procedure. The UE should send a UAI indicating low power and optionally data availability if configured by the network.

Power state changes are typically static and occur over time scales of minutes. UE implementation is not expected to dynamically determine and frequently report power state changes. When the UE reports a low power state, the network should de-configure the data collection procedure. The network can collect data from other UEs that are in normal power states. Therefore, there is no need for additional procedures or indications to inform the network that the battery state is no longer low or other mechanisms such as a prohibit timer for UE low power state indication.

Proposal 4: Buffer status (full buffer or buffer threshold being reached) and power status (low power) are two separate triggers for UAI.

- When the UAI is triggered by a low power state, the UE can indicate the availability of data explicitly if configured by the network.

- When the UAI is triggered by buffer status, the UE can indicate whether it is a full buffer or buffer threshold being reached.

Proposal 5: There is no need for a prohibit timer or additional indications for UAI triggered by buffer status and UE power state.

  - Open issue RRC-10: Time related content of logged data

In RAN2#129bis meeting, we made following agreements on time related content for logged data:

Agreements - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

In legacy Immediate MDT, timestamping is performed by the network (eNodeB or gNodeB). This mode operates during RRC_CONNECTED state, where the network collects real-time performance metrics and timestamps these measurements using its own synchronized clock. This ensures high accuracy and consistency, which is critical for real-time network optimization tasks. By relying on the network’s clock, operators gain precise temporal alignment across all measurements, enabling reliable correlation with other network events.

In contrast, legacy logged MDT delegates timestamping to the UE itself. This mode activates when the device is idle or out of coverage, allowing it to autonomously record measurements like signal quality or location data. The UE timestamps these logs using its internal clock, which lacks synchronization with the network.

For AI/ML model training, data collection primarily occurs in RRC_CONNECTED state, where network-based timestamping remains feasible. However, UE-generated timestamps are essential in specific scenarios:

- Event-Triggered Logging: When the UE autonomously starts logging due to a configured event (e.g., signal quality below a threshold), the network cannot predict the trigger timing. The UE must timestamp the first sample at logging initiation.

- Buffer/Power Constraints: If logging pauses due to a full buffer or low-power state, the UE timestamps the first sample upon resuming. This ensures temporal continuity despite interruptions.

In both the above cases, using absolute timestamps (e.g., UTC-based) avoids complexity from relative time calculations. The network can reconstruct timelines by aligning these anchors with subsequent samples, even if gaps exist.

An open issue that needs clarification is whether UE-side data collection should adopt network timestamps or rely solely on the UE's timestamps. Considering the need for a unified mechanism across both network-side and UE-side data collection, relying on UE-based timestamping is more feasible and simpler. This approach minimizes network involvement in the timestamping process and ensures consistency in data collection.

Proposal 6: The UE provides an absolute timestamp on the first data sample when it starts or resumes data logging.

Proposal 7: RAN2 discuss whether the same timestamping mechanism is applied to both UE-side data collection and network-side data collection.

  - Open issue RRC-12: Cell ID stored with logged data for NW-side data collection

To unambiguously identify the cell where the UE performed data logging, the choice of Cell ID type must balance uniqueness, network utility, and UE reporting capabilities.

CGI (Cell Global Identity) combines PLMN, LAC, and Cell ID to globally distinguish a cell. However, it requires the UE to decode SIB1. In contrast, PCI-ARFCN (Physical Cell ID + Frequency) is lightweight and always measurable, even in weak coverage, but lacks global uniqueness. PCI collisions (e.g., neighboring cells sharing the same PCI-ARFCN) can lead to ambiguous data, complicating post-analysis.

In network-side data collection during RRC_CONNECTED state, the network can map PCI-ARFCN to CGI using its internal database. However, this mapping relies on ideal network conditions and assumes no PCI conflicts, which may not hold in dense or poorly planned deployments.

For unified data logging for UE-side and network-side data collection, a limitation arises: the UE cannot map PCI-ARFCN to CGI independently, as it lacks access to the network’s PCI planning data. If the UE logs only PCI-ARFCN, offline analysis risks misinterpreting measurements due to unresolved PCI collisions. Thus, CGI remains preferred for UE-side logging to ensure consistency and accuracy across datasets, even if it occasionally requires additional effort and signaling overhead.

Legacy logged MDT prioritizes CGI for accuracy but allows PCI-ARFCN as a fallback to ensure data continuity in challenging radio conditions, e.g., in poor coverage. For data collection for model training, the UE should log the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

Proposal 8: The UE should log the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

  - Other Issue

In RAN2#129meeting, it was agreed to support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.

For the use case of AI-BM, only the measurement of serving cell measurement needs to be considered, therefore, only Event A1 (Serving becomes better than threshold) and Event A2 (Serving becomes worse than threshold) can be configured for the purpose of triggering measurement logging at the UE.

Proposal 9: Only A1 (Serving becomes better than threshold) and A2 (Serving becomes worse than threshold) events can be configured optionally to the UE for data logging.

If A1 or A2 event is configured for data logging, when the UE starts and stops the data logging needs to be discussed. One straightforward approach is that the the UE starts data logging when the entering condition of the event is fulfilled and stops data logging when the leaving condition is fulfilled. This ensures consistency in UE's measurement behavior if event-triggered measurement and reporting are configured. However, different RRM measurement and report that the UE removes the remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList, the UE needs to keep the logged measurement.

Proposal 10: If the L3 measurement event (A1, A2) is configured by the network, the UE starts data logging when the entering condition is fulfilled and stops data logging when the leaving condition is fulfilled.

Conclusion

Open issue RRC-7: NW control on retaining logged data at HO

Proposal 1: RAN2 assumes that the source gNB and the target gNB will coordinate on whether to release or retain the unretrieved data for the UE during the HO preparation phase.

Proposal 2: The source gNB decides whether to release or retain the unretrieved data based on the coordination with the target gNB. The target gNB then sends the indication to the UE in the RRCReconfiguration message.

Open issue RRC-8: Reporting assistance information related to logged measurements

Proposal 3: Use a single SetupRelease flag to enable the UE to report assistance information related to logged measurements.

Open issue RRC-9: Further procedures for UE assistance information related to logging

Proposal 4: Buffer status (full buffer or buffer threshold being reached) and power status (low power) are two separate triggers for UAI.

- When the UAI is triggered by a low power state, the UE can indicate the availability of data explicitly if configured by the network.

- When the UAI is triggered by buffer status, the UE can indicate whether it is a full buffer or buffer threshold being reached.

Proposal 5: There is no need for a prohibit timer or additional indications for UAI triggered by buffer status and UE power state.

Open issue RRC-10: Time related content of logged data

Proposal 6: The UE provides an absolute timestamp on the first data sample when it starts or resumes data logging.

Proposal 7: RAN2 discuss whether the same timestamping mechanism is applied to both UE-side data collection and network-side data collection.

Open issue RRC-12: Cell ID stored with logged data for NW-side data collection

Proposal 8: The UE should log the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

Other Issue

Proposal 9: Only A1 (Serving becomes better than threshold) and A2 (Serving becomes worse than threshold) events can be configured optionally to the UE for data logging.

Proposal 10: If the L3 measurement event (A1, A2) is configured by the network, the UE starts data logging when the entering condition is fulfilled and stops data logging when the leaving condition is fulfilled.

Reference

[1] [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)