3GPP TSG-RAN WG2 Meeting #130	R2-2503794

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.7.4.2

Source:	China Telecom

Title:	Discussion on Remaining Issues of DSR Enhancements

Document for:	Discussion and Decision

- Introduction

In the last meeting, the agreements one DSR enhancements were made as follow.

Agreements on DSR enhancements To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

The paper aims to discuss the remaining issues on DSR enhancements and provide some considerations.

2 Discussion

In the RAN#129 meeting, the following options were proposed regarding to the data volume calculation.

- Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting.

- Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

Per TS38.321, the buffer size field in DSR MAC CE specifies the total volume of delay-critical UL data for an LCG, and the buffer size is calculated according to the data volume calculation procedure for the associated RLC in TS38.322 and the data volume calculation procedure for the associated PDCP in TS 38.323.

In TS38.322, it specifies that the RLC layer should consider the following fields as part of the delay-critical data volume calculation.

- RLC data PDUs that are pending for retransmission (RLC AM)

- STATUS PDU

And in TS38.323, it specifies that the PDCP layer should consider the following fields as part of the delay-critical data volume calculation.

- the PDCP Control PDUs;

- for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

- for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

It can be observed that the RLC entity prioritizes the control PDU and retransmitted RLC data PDUs. Similarly, the PDCP entity also priorities the control PDUs and retransmitted PDCP data PDUs/ PDCP SDUs over other delay-critical data transmission. Therefore, it is reasonable to consider the control PDUs and the PDCP SDUs/PDCP PDUs to be retransmitted in the buffer size calculation associated with the smallest remaining time threshold. In other words, with regard to data volume calculation, it is proposed to support Option 2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

Proposal 1: For data volume calculation, both PDCP/RLC Control PDUs and retransmitted data should be associated to the shortest configured reporting threshold (Option 2 is preferred).

Regarding the ​​remaining time field​​ setting for each reporting threshold in the ​​DSR MAC CE​​, it is determined by the smallest remaining time until discard timer expiry among the associated PDCP SDUs. However, it remains unclear how this field should be set when the corresponding buffer contains ​​only PDCP/RLC control PDUs and retransmitted data, as there is no data related to a discard timer. In the last meeting, it has proposed that the remaining time field can be set to smallest configured or smallest reporting threshold. In our view, to reduce the complexity, the remaining time field should be set to the smallest reporting threshold.

Proposal 2: Set the remaining time field as the smallest reporting threshold.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: For data volume calculation, both PDCP/RLC Control PDUs and retransmitted data should be associated to the shortest configured reporting threshold (Option 2 is preferred).

Proposal 2: Set the remaining time field as the smallest reporting threshold.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China