3GPP TSG-RAN WG2 Meeting #130							R2-2503796

Malta, MT, 19th – 23th May, 2025

Agenda Item:	8.7.6

Source:	China Telecom

Title:	Discussion on Remaining Issues of XR Rate Control

Document for:	Discussion

1	Introduction

During RAN#105 meeting, a new objective was introduced in XR for NR Phase 3 WID to specify MAC layer XR rate control for uplink congestion adaption.

-	Specify uplink congestion signalling [RAN2]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

XR rate control was discussed in RAN2 and RAN3 simulatensouly. In RAN2#129bis meeting, the following agreements were made:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In RAN3#127bis meeting, the following agreements were achieved:

- Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

- Introduce the same indication in XnAP to inform target gNB.

In this contribution, we are continue discussing on XR rate control and provide our views.

2	Discussion

In the RAN2#129bis meeting, RAN2 preferred to define per-QoS flow level XR rate control using a MAC CE. The QoS flow identifiers and corresponding rate recommendations may be included in this newly defined MAC CE. During the online discussion, some companies raised concerns about the impact on the F1 interface. The network configures which QoS flows are subject to rate control via RRC signaling, which is transmitted from the gNB-CU to the UE. However, the gNB-DU does not have access to the rate control information and therefore cannot generate the rate control MAC CEs.From the RAN3 perspective, the gNB-CU could provide at least the necessary QoS flow information related to rate control to the gNB-DU. Any additional required information will depend on the progress made in RAN2. Additionally, both the gNB-CU and gNB-DU could support XR rate monitoring or congestion monitoring at a per-QoS flow level. And how to support per-QoS flow rate monitoring in gNB-DU side is up to implementation. Technically, the gNB-CU could also provide the per-QoS flow level recommended rate to the gNB-DU. With this information, the gNB-DU would be able to generate the new MAC CE format for XR rate recommendation or receive the MAC CE format for XR data rate queries. Anyway, we can wait the response from RAN3.

Proposal 1: RAN2 to agree the per flow indication in MAC CE.

Per TS38.413 and TS23.501, the maximum number of QoS flows is 64. From the perspective of MAC CE, there is no need to retain the existing size of the QoS Flow ID within the MAC CE. To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session, or a MBS QoS flow within a MBS session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

Proposal 2: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

3 	Conclusion

Based on the above analysis, we have the following proposals:

Proposal 1: RAN2 to agree the per flow indication in MAC CE.

Proposal 2: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

4	References

[1]	TR 38.838, Study on XR (Extended Reality) evaluations for NR, V17.0.0, 2022-04