3GPP TSG-RAN WG2 #130                                             	    R2-2503802

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	Discussion on L1 event-triggered measurement reporting for LTM

Document for:	Discussion

# 1	Introduction

RAN2#129 has agreed on the following for L1 event-triggered measurements for LTM [1]:

Agreements (Organizational) - No L1/L2 filtering issue will be discussed in RAN2. - Select two values of 20ms and 60ms for the additional values. - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. - When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE. - When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed. - The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE. - FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. Agreements on L1 event-driven MR: - Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. - The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id. - In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to). - In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam. - RAN2 will consider only the following options: - Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting. Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) - Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR. - For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

This paper discusses further aspects related to L1 event-triggered measurement report enhancements for LTM.

# 2	SP CSI-RS resource management

## 2.1	New activation/deactivation MAC CE design

In RAN2#129b, the following agreements were made for SP CSI-RS support in LTM measurements.

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. - The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

Based on the current MAC running CR and RAN1’s ASN.1 code proposal [2], the Rel-19 LTM NZP-CSI-RS resource set would have the same flattened structure as Rel-18 LTM SSB resource set, where NZP-CSI-RS resource IDs from different candidate cells are grouped in to one LTM CSI-RS resource list, and there is another LTM candidate ID list with candidate cell IDs which has the same length as LTM CSI-RS resource list (i.e., one-to-one mapping between NZP-CSI-RS resource ID and candidate cell ID).

Snippet of R1-2502627 [2] LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-CSI-SSB-ResourceSet-r18 LTM-CSI-SSB-ResourceSet-r18, ..., [[ ltm-NZP-CSI-RS-ResourceSet-r19 LTM-CSI-RS-ResourceSet-r19 OPTIONAL --Need R ]] } LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-NZP-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, repetition ENUMERATED {off}, resourceType-r19 ENUMERATED {periodic, semipersistent} ... }

From activation/deactivation point of view, the legacy SP-CSI-RS activation/deactivation is done in an NZP-CSI-RS Resource Set level, but there are some differences between legacy CSI resource configuration and LTM CSI resource configuration.

- LTM NZP-CSI-RS Resource Set (ltm-NZP-CSI-RS-ResourceSet-r19) has no corresponding resource set ID associated.

- NZP-CSI-RS Resource Sets defined under LTM Config (for LTM CSI measurement purpose) has no linkage to NZP-CSI-RS Resource IDs (ltm-CSI-RS-ResourceList-r19) under LTM NZP-CSI-RS Resource Set (ltm-NZP-CSI-RS-ResourceSet-r19).

- A particular candidate cell can be present in multiple LTM NZP-CSI-RS Resource Sets.

Due to above reasons, {SP CSI-RS resource set id and/or candidate id} captured in the previous agreement could cause complications and ambiguities in SP CSI-RS resource management. Therefore, we believe the SP CSI-RS Resource management for LTM should be done per LTM CSI Resource. By doing this, a particular LTM CSI resources for an LTM CSI report can be controlled as a group.

Observation 1: Unlike legacy SP CSI-RS resource configuration, LTM NZP-CSI-RS Resource Set doesn’t have any ID associated, and NZP-CSI-RS Resource Sets under LTM Config are not linked to NZP-CSI-RS Resource IDs under LTM NZP-CSI-RS Resource Set, either. Therefore, {SP CSI-RS resource set id and/or candidate id} could cause unnecessary complications and ambiguities.

Proposal 1: RAN2 agree to use LTM CSI Resource Config ID for LTM SP CSI-RS activation/deactivation MAC CE.

In legacy SP CSI-RS activation/deactivation MAC CE, the activation/deactivation operation was applied to all NZP-CSI-RS Resources associated with the signalled NZP-CSI-RS Resource Set, i.e., for activation, only TCI state IDs were provided without specifying NZP-CSI-RS Resource ID.

However, in LTM, an LTM CSI Resource may include many NZP-CSI-RS Resources across different candidate cells, if we follow the legacy approach, then all the configured NZP-CSI-RS Resources would need to be activated or deactivated, and this could increase the measurement overhead at UE. In most cases, the serving cell would need to activate only a subset of NZP-CSI-RS Resources (w.r.t. spatial beam direction and/or distance from candidate cells, therefore it would be sensible to allow selective activation/deactivation of SP CSI-RS resources for LTM.

Observation 2: The legacy SP CSI-RS management (i.e., activating/deactivating all NZP-CSI-RS Resources per set) may cause unnecessary activations of all NZP-CSI-RS Resources under the LTM CSI Resource, which could cause increased measurement overhead at UE.

Proposal 2: RAN2 agree to support the selective activation/deactivation of SP CSI-RS resources by introducing an ordinal index to LTM CSI-RS Resource List (ltm-CSI-RS-ResourceList-r19) along with TCI state ID in LTM SP CSI-RS activation/deactivation MAC CE.

When selective activation/deactivation of LTM SP CSI-RS resources is supported, the delta update of SP CSI-RS resources, i.e., additional SP CSI-RS resources are newly activated or partial activated SP CSI-RS resources are deactivated, would provide more flexibility and less signalling overhead in maintaining the SP CSI-RS resources.

Observation 3: Along with the selective activation/deactivation, the delta update of SP CSI-RS resources can provide more flexibility and less signalling overhead for SP CSI-RS resource management.

Proposal 3: RAN2 agree to support the delta activation/deactivation of SP CSI-RS resources, where additional SP CSI-RS resources are newly activated or partial activated SP CSI-RS resources are deactivated while UE keep the activation status for other SP CSI-RS resources than the specified.

## 2.2	Activated SP CSI-RS resource handling after LTM cell switch

In RAN2#129b, it was discussed how to handle the activated SP CSI-RS resources after LTM cell switch, but it was left FFS.

RAN2 decide how to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options. - Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch - Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch - Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

In LTM CSI report framework, each cell (serving or candidate) is responsible for creating its own LTM CSI Report Config and LTM CSI Resource Config, where LTM CSI Resource Config contains NZP-CSI-RS Resource configurations from neighbouring candidate cells. This would mean, in turn, each cell can have different LTM CSI Resource Configs, each of which could have non-overlapping or partially overlapping NZP-CSI-RS Resources from candidate cells.

Observation 4a: Each cell (serving and candidate) has its own LTM CSI Resource Configs, and they may or may not have overlapping NZP-CSI-RS Resources.

Taking this into account, if UE continues to measure the activated SP CSI-RS resources after LTM cell switch, then the following complications/issues could occur:

- Some of activated SP CSI-RS resources may not be part of LTM CSI Resource Config from the target cell.

  - This is due to the independent LTM CSI Resource Config across cells.

- Even though activated SP CSI-RS resources are indeed part of LTM CSI Resource Config from the target cell, it is not guaranteed that those activated SP CSI-RS resources are of interest for the target cell.

  - Once activated, UE would have to measure those SP CSI-RS resources, which would cause unnecessary measurement overhead at UE and scheduling restrictions imposed at NW.

- During LTM cell switch, UE would inevitably have a service interruption period, therefore there would need to be defined RAN4 requirements on how long interruption period UE requires to resume measuring the activated SP CSI-RS resources.

Due to abovementioned reasons, we believe making UE continue to measure any SP CSI-RS resource after LTM cell switch (i.e., Option 2 and Option 3) is not a desirable solution. Instead, UE can stop measuring activated SP CSI-RS resources, i.e., UE assumes to deactivate all activated SP CSI-RS resources, upon LTM cell switch. This would require the target cell to re-activate SP CSI-RS resources, but the signalling overhead could be less than Option 2 & 3. That’s because Option 1 needs just one SP CSI-RS activation MAC CE transmission while Option 2 & 3 may end up transmitting totally two MAC CEs - one for deactivating SP CSI-RSs of no interest and the other for activating SP CSI-RSs of new interest. And it should be noted that this SP CSI-RS handling at UE is agnostic to NW-side NZP-CSI-RS transmissions, i.e., even though UE deactivates SP CSI-RS resources after LTM cell switch, NW may still transmit NZP-CSI-RS for the purpose of other LTM UEs and/or potential reactivation by the target cell.

Observation 4b: Any SP CSI-RS that UE continues to measure after LTM cell switch may not be defined as LTM CSI Resource Config, or even if it’s configured, it may not be of interest from the target cell point of view. Then those SP CSI-RS resources of no interest would need to be still measured by UE which causes measurement overhead and potential scheduling restriction.

Observation 4c: When UE continues to measure some SP CSI-RS resources unnecessarily, the target cell may eventually end up sending two different MAC CEs – one for deactivating SP CSI-RS resources of no interest and the other for activating SP CSI-RS resources of interest. This incurs more signalling overhead than sending one activation MAC CE after LTM cell switch (assuming all SP CSI-RS resources are deactivated).

Proposal 4: RAN2 agree to have UE deactivate all activated SP CSI-RS resources after LTM cell switch, and have the target cell send LTM SP CSI-RS activation MAC CE to activate SP CSI-RS resources of target cell’s interest.

# 3	L1 MR MAC CE

## 3.1	Beam type identification

In RAN2#129b, the following WA was made on the codepoint definition for beam type identification.

- RAN2 will consider only the following options: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) - Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

As discussed during the CB session, Option 1 is a straightforward way to distinguish between Type 1 and Type 3 beams, and the explicit distinction between them could be beneficial for the truncated MAC CE where Type 1 beams can be prioritized over Type 3 beams.

Observation 5: Option 1 is a straightforward way to distinguish between Type 1 and Type 3 beams, and the explicit distinction between them could be beneficial for the truncated MAC CE where Type 1 beams can be prioritized over Type 3 beams.

Proposal 5: RAN2 agree to support Option 1, i.e., to introduce four codepoints to represent one of {type1, type2, type3, type4}.

## 3.2	Event LTM2 reporting

### 3.2.1	Current beam RS mapping

In RAN1#120, the following agreement was made for Event 1 (Quality of the current beam is worse than a certain threshold).

- On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement:

  - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

    - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

      - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

      - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Given that LTM CSI resource configuration is mandatorily present under LTM CSI report configuration, the same approach of an implicit RS mapping rules for Event 1 can be applied to Event LTM2 as below:

- UE implicitly determines the current beam RS based on the RS type of the configured LTM CSI resource (similar to Event LTM3 case).

    - If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

    - If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

Observation 6: LTM CSI resource is present for all events, and for Event LTM2 hence Event LTM2 can leverage the concept of implicitly determining the current beam RS based on RS type in LTM CSI resource, which is effectively the same as current beam RS determination for Event LTM3.

Proposal 6: For Event LTM2, the current beam RS is implicitly determined by the RS type of the configured LTM CSI resource:

- If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

- If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

### 3.2.2	RS measurement and reporting

In RAN2#129b, the decision on candidate beam measurements & reporting in L1 MR MAC CE was left FFS.

FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

If Event LTM2 only reports the current beam information in L1 MR MAC CE, then the usage of Event LTM2 becomes questionable in two folds:

- Event LTM2 serves the exactly same functionality as Event1 in MIMO UE-initiated/event-driven beam report.

- From mobility point of view, Event LTM2 doesn’t provide enough information by itself, i.e., lack of candidate beam measurement results.

As discussed in the previous section 3.2.1, LTM CSI resource is mandatorily present and used for the current beam RS determination, which is exactly same as Event LTM3. Therefore, it would be also sensible to have the common measurement and reporting procedure between Event LTM2 and LTM3, where UE measures the RSs configured in LTM CSI resource when an event condition is satisfied, and the current beam measurement report is controlled by NW configuration of current beam reporting (reportCurrentBeam-r19). This will also allow NW to get the candidate beam results along with the current beam measurement.

Observation 7a: Current beam measurement only for Event LTM2 in L1 MR MAC CE would serve the same purpose as Event1 in UE-initiated/event-driven beam report, without providing any other information from mobility point of view.

Observation 7b: Given that LTM CSI resource is mandatorily present and also used for the current beam RS determination, then it would be sensible to have the common measurement & reporting procedure between Event LTM2 and LTM3.

Proposal 7: RAN2 agree to support the same report format for Event LTM2 as LTM3, i.e., candidate RSs under LTM CSI Resource Config are measured and included in L1 MR MAC CE, and the current beam reporting is configured by reportCurrentBeam-r19 (FFS whether reportCurrentBeam-r19 shall be set to TRUE for Event LTM2).

## 3.3	L1-RSRP representation for truncated L1 MR MAC CE

In RAN2#129b, the following agreement was made on differential L1-RSRP reporting for regular L1 MR MAC CE, and it was left FFS on truncated L1 MR MAC CE.

RAN2#129 - Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP RAN2#129b - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. - When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

The differential reporting method (i.e., the first beam which has the highest report metric serves as a reference beam and the rest beams are represented as a differential value) has been used for both L1 and L3 RSRP reporting, and it’s also been agreed for event-triggered L1 MR MAC CE. Therefore, it would be sensible to use the same differential reporting method for truncated L1 MR MAC CE.

Observation 8: The differential reporting method (i.e., the first beam which has the highest report metric serves as a reference beam and the rest beams are represented as a differential value) has been used for legacy L1 and L3 RSRP reporting as well as event-triggered L1 MR MAC CE, therefore it can be used for truncated L1 MR MAC CE.

Proposal 8: For truncated L1 MR MAC CE, RAN2 agree to support the same differential L1-RSRP reporting method as regular L1 MR MAC CE.

## 3.4	Prioritization among triggered events

Since gNB can configure multiple event configurations with different sets of candidate beams and/or different event types, it is possible that more than one event can be triggered at UE and those reports need to be transmitted simultaneously through the available PUSCH grant instance. When the grant size is not sufficient enough to accommodate all the triggered events (even with the truncated MAC CE), then UE would have to drop one or more triggered events’ report to fit into the grant size. It would be desirable to discuss how UE can handle this scenario.

Observation 9: It is possible that UE detects multiple events triggered but the available grant size is not sufficient to accommodate all triggered events’ report even with the truncated MAC CE hence the prioritization among triggered events needs to be discussed.

Proposal 9: RAN2 to discuss the following event prioritization rule and down-select for truncated L1 MR MAC CE:

- Option 1: Define a priority order among Event LTM 2/3/4/5 in the spec.

- Option 2: Introduce an RRC parameter for the priority value to be associated with the event configuration

- Option 3: Leave it up to UE implementation

## 3.5	Prioritization among beam types within a triggered event report

In previous RAN2 meetings, the following agreements were made on the maximum N beam reporting w/o counting the current beam and the non-event triggering beam reporting based on the measured quality.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam. - Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)).

Given that the last RAN2 meeting agreed a working assumption of 4 different beam types, it would be good to discuss general rules on what kind of beam types need to be included in L1 MR MAC CE UE. From the priority point of view, 4 beam types can be categorized into 3 different priority levels as shown below:

| Priority | Beam type | Note |
| --- | --- | --- |
| High | Type1 and Type2 | Newly triggered candidate beams |
| Medium | Type3 | Previously triggered candidate beams |
| Low | Type4 | Non-triggering candidate beams |

When L1 MR MAC CE is prepared, the following operation is expected to occur in UE, which can be commonly applicable to both regular and truncated MAC CE:

  - Determine the value M (<=N max number of beams) based on the available PUSCH grant, where M is the number of candidate beams that can be included in L1 MR MAC CE w/ or w/o the current beam report (depending on configuration).

  - Include beams in Type1 and Type2

    - FFS on priority between Type1 and Type2 in case M is not enough to cover both Type1 and Type2 beam results.

  - Include beams in Type3 (only if M beams are not filled up after including Type1/Type2 beams)

    - FFS whether Type3 beams are also based on the measured quality (L1-RSRP) or not.

  - Include beams in Type4 (only if M beams are not filled up after including Type1/Type2/Type3 beams)

    - The selected beams are based on the measured quality (L1-RSRP).

  - Sort the beams in the decreasing order of the measured quality (L1-RSRP).

    - This is to ensure the differential RSRP reporting method works for both regular and truncated MR MAC CE.

  - Include current beam (if configured).

Observation 10: Based on 4 different beam types (working assumption) from the last RAN2 meeting, the prioritization among beam types could be discussed to clarify what kind of beams and beam types should be included in L1 MR MAC CE (regular and truncated).

Proposal 10: RAN2 to confirm above steps (highlighted) as a baseline for L1 MR MAC CE payload generation.

## 3.6	Handling pending event report(s) and/or remaining beam result(s) following truncated L1 MR MAC CE

In RAN2#129b, the following agreement was made, and it is also discussed in the MAC running CR email discussion.

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

When UE sends a truncated L1 MR MAC CE due to the lack of sufficient PUSCH grant, UE may be in a situation where there are still one or more of pending beam results from this truncated MR MAC CE (i.e., some beams could not be included into this truncated L1 MR MAC CE) and/or one or more of pending reports (i.e., these triggered events’ reports were dropped/deprioritized due to lack of PUSCH grant and event prioritization).

In the current L1 MR MAC CE design, there is no indication of such pending reports and/or beam results to gNB, and these pending measurement reports cannot be indicated by BSR, either. Therefore, in the worst case where there’s no dedicated SR, UE has no on-going PUSCH traffic, and multiple events are triggered, UE may need to send SR multiple times to send those triggered L1 MR MAC CEs, which could take a long time for UE to send all of them. Considering the motivation of event-triggered measurement report is to enable a timely transmission of measurement results for LTM cell switch, it would be beneficial to have an explicit indication of pending event reports and/or beam results in L1 MR MAC CE.

Observation 11: The presence of pending beam results (beam measurements that are dropped via truncation) and/or pending reports (triggered events’ reports that are dropped due to event prioritization) cannot be signalled to gNB, which could take a long time for UE to send all triggered events’ report to gNB.

Proposal 11: RAN2 agree to support explicit indication(s) of pending beam results and/or pending reports in L1 MR MAC CE.

## 3.7	L1-RSRP change rate information

In RAN2#127b, L1-RSRP was agreed to be included as the beam quantity in event-triggered L1 measurement report for LTM. While reporting the ‘snapshot’ of measurement results (filtered or unfiltered) has been traditionally adopted as the beam quantity included in L1 or L3 measurement reports, the lack of additional information such as the change rate of measurement quantity change has brought several challenges (e.g., handover failures and ping-pong handovers) in real deployment scenarios.

Observation 12a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

In Figure 2 below, the left and right figures show two different expectations after LTM execution which is assumed to be done based on the event-triggered measurement report from UE. If UE can include additional information of the L1-RSRP change rate (or “slope”) along with the L1-RSRP value, serving cell could make the better choice for the candidate beam & candidate cell for the LTM cell switch.

Figure 2. (left) negative L1-RSRP slope and (right) positive L1-RSRP slope in event-triggered L1 measurement report MAC CE transmission

Observation 12b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 12: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

# 4	LTM event evaluation

## 4.1	Prohibit timer

When the event is evaluated per-beam level, it is possible that another beam satisfies the event condition, shortly after the measurement report for that event has been transmitted. Even though it is possible to let UE trigger a new measurement report whenever the event condition is satisfied, it may lead to too many event-triggered L1 measurement reports for each event configuration. And as more event configurations are provided to UE, the overall number of event-triggered reports could be unnecessarily large.

To achieve timely event-triggered but less frequent report transmissions, a new event-triggered measurement report can be only triggered after a “sufficient amount of time” has passed since the last measurement report for the same event. This sufficient amount of time can be controlled by a prohibit timer which starts after the measurement report for the triggered event is sent, and while this timer is running, no new measurement report can be triggered. Triggering event-triggered measurement reports for every triggering beam can be also supported by setting this prohibit timer value to zero.

Observation 13: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 13a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 13b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

## 4.2	Different HYS/TTT values for leaving condition

In RAN2#127b, it was agreed that a measurement report can be sent when the leaving condition is met based on NW configuration, and in RAN2#128, it was agreed that TTT is applied to the leaving condition.

In L3 event-based measurement reports, one set of HYS and TTT values are configured per event configuration, and they are applied to both entering and leaving conditions as illustrated in Figure 1. Applying the same HYS and TTT values to both entering and leaving conditions wouldn’t make much difference in triggering the L3 measurement reports and subsequent L3 HOs because (i) the cell metric is used for the evaluation hence the metric is less sensitive to the beam level metric differences, (ii) the measurement report through RRC signaling has a larger time scale than lower layer signaling (UCI or event-triggered L1 measurement MAC CE), and (iii) L3 HO has a longer overall switching latency than LTM switch (i.e., from the measurement report trigger to the cell switch complete).

Observation 14a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Figure 1. Illustration of entering and leaving condition in L3 measurement Event A3

In event-triggered L1 measurement reports for LTM, however, the situation becomes different. For a seamless LTM cell switch, it is crucial to keep track of the best candidate beam among those who fulfilled the entering condition for the potential cell switch so that serving cell can perform the candidate TCI state activation and/or early UL synchronization with that chosen beam. While a candidate beam satisfying the entering condition can be chosen conservatively via HYS/TTT, any candidate beam that is no longer good for cell switch (i.e., leaving condition) should be identified by serving cell through a measurement report as quickly as possible. In this aspect, applying the same HYS and TTT could delay the leaving condition to be triggered significantly.

Observation 14b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

In order to expedite the event-triggered L1 measurement report for leaving condition, there can be two options to be considered:

- Option 1: Keep one set of HYS and TTT for both entering and leaving conditions, and set them to smaller values

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Option 1 can be the simplest solution without any specification impact, but the smaller HYS and TTT values would impact the entering condition to be triggered quicker as well, which could lead to unexpectedly frequent measurement reports triggered due to potential noisy L1 measurements (without any spec-defined filtering applied). Both Option 2 and 3 provide the configurability of different HYS and/or TTT values between entering and leaving conditions, but Option 3 could be preferrable due to its flexibility in not providing additional HYS and/or TTT for leaving condition if NW chooses not to.

Observation14 c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 14: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Observation 1: Unlike legacy SP CSI-RS resource configuration, LTM NZP-CSI-RS Resource Set doesn’t have any ID associated, and NZP-CSI-RS Resource Sets under LTM Config are not linked to NZP-CSI-RS Resource IDs under LTM NZP-CSI-RS Resource Set, either. Therefore, {SP CSI-RS resource set id and/or candidate id} could cause unnecessary complications and ambiguities.

Proposal 1: RAN2 agree to use LTM CSI Resource Config ID for LTM SP CSI-RS activation/deactivation MAC CE.

Observation 2: The legacy SP CSI-RS management (i.e., activating/deactivating all NZP-CSI-RS Resources per set) may cause unnecessary activations of all NZP-CSI-RS Resources under the LTM CSI Resource, which could cause increased measurement overhead at UE.

Proposal 2: RAN2 agree to support the selective activation/deactivation of SP CSI-RS resources by introducing an ordinal index to LTM CSI-RS Resource List (ltm-CSI-RS-ResourceList-r19) along with TCI state ID in LTM SP CSI-RS activation/deactivation MAC CE.

Observation 3: Along with the selective activation/deactivation, the delta update of SP CSI-RS resources can provide more flexibility and less signalling overhead for SP CSI-RS resource management.

Proposal 3: RAN2 agree to support the delta activation/deactivation of SP CSI-RS resources, where additional SP CSI-RS resources are newly activated or partial activated SP CSI-RS resources are deactivated while UE keep the activation status for other SP CSI-RS resources than the specified.

Observation 4a: Each cell (serving and candidate) has its own LTM CSI Resource Configs, and they may or may not have overlapping NZP-CSI-RS Resources.

Observation 4b: Any SP CSI-RS that UE continues to measure after LTM cell switch may not be defined as LTM CSI Resource Config, or even if it’s configured, it may not be of interest from the target cell point of view. Then those SP CSI-RS resources of no interest would need to be still measured by UE which causes measurement overhead and potential scheduling restriction.

Observation 4c: When UE continues to measure some SP CSI-RS resources unnecessarily, the target cell may eventually end up sending two different MAC CEs – one for deactivating SP CSI-RS resources of no interest and the other for activating SP CSI-RS resources of interest. This incurs more signalling overhead than sending one activation MAC CE after LTM cell switch (assuming all SP CSI-RS resources are deactivated).

Proposal 4: RAN2 agree to have UE deactivate all activated SP CSI-RS resources after LTM cell switch, and have the target cell send LTM SP CSI-RS activation MAC CE to activate SP CSI-RS resources of target cell’s interest.

Observation 5: Option 1 is a straightforward way to distinguish between Type 1 and Type 3 beams, and the explicit distinction between them could be beneficial for the truncated MAC CE where Type 1 beams can be prioritized over Type 3 beams.

Proposal 5: RAN2 agree to support Option 1, i.e., to introduce four codepoints to represent one of {type1, type2, type3, type4}.

Observation 6: LTM CSI resource is present for all events, and for Event LTM2 hence Event LTM2 can leverage the concept of implicitly determining the current beam RS based on RS type in LTM CSI resource, which is effectively the same as current beam RS determination for Event LTM3.

Proposal 6: For Event LTM2, the current beam RS is implicitly determined by the RS type of the configured LTM CSI resource:

- If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

- If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

Observation 7a: Current beam measurement only for Event LTM2 in L1 MR MAC CE would serve the same purpose as Event1 in UE-initiated/event-driven beam report, without providing any other information from mobility point of view.

Observation 7b: Given that LTM CSI resource is mandatorily present and also used for the current beam RS determination, then it would be sensible to have the common measurement & reporting procedure between Event LTM2 and LTM3.

Proposal 7: RAN2 agree to support the same report format for Event LTM2 as LTM3, i.e., candidate RSs under LTM CSI Resource Config are measured and included in L1 MR MAC CE, and the current beam reporting is configured by reportCurrentBeam-r19 (FFS whether reportCurrentBeam-r19 shall be set to TRUE for Event LTM2).

Observation 8: The differential reporting method (i.e., the first beam which has the highest report metric serves as a reference beam and the rest beams are represented as a differential value) has been used for legacy L1 and L3 RSRP reporting as well as event-triggered L1 MR MAC CE, therefore it can be used for truncated L1 MR MAC CE.

Proposal 8: For truncated L1 MR MAC CE, RAN2 agree to support the same differential L1-RSRP reporting method as regular L1 MR MAC CE.

Observation 9: It is possible that UE detects multiple events triggered but the available grant size is not sufficient to accommodate all triggered events’ report even with the truncated MAC CE hence the prioritization among triggered events needs to be discussed.

Proposal 9: RAN2 to discuss the following event prioritization rule and down-select for truncated L1 MR MAC CE:

- Option 1: Define a priority order among Event LTM 2/3/4/5 in the spec.

- Option 2: Introduce an RRC parameter for the priority value to be associated with the event configuration

- Option 3: Leave it up to UE implementation

Observation 10: Based on 4 different beam types (working assumption) from the last RAN2 meeting, the prioritization among beam types could be discussed to clarify what kind of beams and beam types should be included in L1 MR MAC CE (regular and truncated).

Proposal 10: RAN2 to confirm above steps (highlighted) as a baseline for L1 MR MAC CE payload generation.

Observation 11: The presence of pending beam results (beam measurements that are dropped via truncation) and/or pending reports (triggered events’ reports that are dropped due to event prioritization) cannot be signalled to gNB, which could take a long time for UE to send all triggered events’ report to gNB.

Proposal 11: RAN2 agree to support explicit indication(s) of pending beam results and/or pending reports in L1 MR MAC CE.

Observation 12a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

Observation 12b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 12: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

Observation 13: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 13a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 13b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

Observation 14a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Observation 14b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

Observation14 c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 14: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# References

- Chairman Notes; TSG RAN WG2 Meeting #129

- R1-2502627, Higher layer parameters for Rel-19 NR mobility enhancements Ph4