3GPP TSG-RAN WG2#130	R2-2503805

St. Julians, Malta, May 19th – 23rd, 2025

Source: 	Fujitsu

Agenda item:	8.5.2

Title: 	Remaining issues on on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#129bis meeting, RAN2 has discussed the on-demand SSB SCell operation, and the following agreement was made [1]:

Agreements on OD-SSB 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. 6. Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, we would like to discuss the remaining issues on on-demand SSB SCell operation.

# Discussion

## On-demand SSB SCell

### L3 measurements

Framework of L3 measurements

In RAN2#127bis meeting, the following two options were discussed for L3 measurements framework but RAN2 have not agreed because it is not clear for RAN4 impact.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

On the other hand, RAN4 agreed the fast mode window for L3 measurements at on-demand SSB transmission. During the fast mode window, the measurement requirement is defined based on on-demand SSB periodicity.

For option 1, it has less impact from RAN4 perspective, because currently L3 measurement requirement in TS 38.133 is defined based on the SMTC configuration, not for the SSB configuration. From RAN2 perspective, although on-demand SSB has different (typically short) periodicity from that of always-on SSB, the same measurement mechanism can be applied by adaptive SMTC window for on-demand SSB periodicity in the fast mode window. Since RAN2 already supports multiple SMTC lists (e.g., smtc3list, smtc4list), it would have small impacts on the specifications in RAN2.

For option2, it may be beneficial if on-demand SSB pattern needs full flexibility. For example, at least applicable value for on-demand SSB periodicity can be indicated by a new MAC CE for on-demand SSB transmission activation. As the SSB periodicity is set by one of {ms5, ms10, ms20, ms40, ms80, ms160}, then in case of option 1, at worst the NES UE should be pre-configured with 6 SMTCs for on-demand SSB measurements. However, even if on-demand SSB needs some flexibility for quick DL synchronisations and/or L1/L3 measurements, in practical, at most 2 SSB periodicities would be sufficient (i.e., for scenarios 2/2A).

Observation 1:	In practical, it is enough to support at most 2 SSB periodicities for on-demand SSB (i.e., for scenarios 2/2A).

From signalling perspective, option 2 may be simpler than option 1 because option 2 can reuse the configured on-demand SSB patterns for L3 measurements whereas option 1 needs an additional configuration for adaptive SMTC. However, in RAN2#129 meeting, a similar issue was discussed on SSB adaptation and RAN2 finally made the following agreement.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Observation 2:	For SSB adaptation, RAN2 agreed to keep SMTC based L3 RRM framework.

Because it is a similar concept of the option1, it means that signalling reduction/optimization is not a critical issue for L3 measurement.

Considering the above, there is no clear motivation to support such a flexibility for L3 measurements, and option 1 has less impact on other WGs with reasonable RAN2 impact. So, we prefer to support option 1 for L3 measurements framework of on-demand SSB. The limited number of SMTC windows for on-demand SSB can be pre-configured by RRC at the SCell addition. The UE can apply the SMTC window for on-demand SSB indicated by MAC CE or RRC when the on-demand SSB transmission is activated. It is also beneficial for RAN2 to adopt a unified L3 measurements framework between on-demand SSB and SSB adaptation.

Proposal 1:	RAN2 to adopt “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 2:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

L3 measurements for neighbour cell:

In RAN4#112bis meeting, RAN4 has agreed the legacy measurement framework is applied for neighbour cell measurement [2].

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. This agreement can be revisited if it is not aligned with further agreement in other WGs.

From RAN2 perspective, as the UE does not know the SSB transmission status (on-demand or broadcasted) of neighbour cells, it is difficult to measure the on-demand SSB per neighbour cell. Hence, it is reasonable to follow the above RAN4 agreement as is.

Proposal 3:	RAN2 to confirm the legacy measurement framework (i.e., legacy SMTC) is applied for neighbour cell measurements.

### On-demand SSB activation/deactivation MAC CE

In RAN1#118 meeting, RAN1 has agreed that multiple candidate values of SSB periodicity can be configured. Besides that, in RAN1#120-bis meeting, RAN1 also agreed the following OD-SSB parameters can be configured by RRC, and one of multiple candidate values can be indicated by MAC CE [3].

- SSB positions within an on-demand SSB burst

- Number N of on-demand SSB bursts

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

It means currently three types of OD-SSB parameters may have multiple candidate values. We assume the above parameters are configured as a list of the combinations and identified by the index. Otherwise, MAC CE has to indicate the candidate values directly and consequently it would increase the complexity of specification and implementation.

Proposal 4:	No need to explicitly indicate the candidate values for OD-SSB via MAC CE.

The candidate values with index could be configured in OD-SSB SCell configuration by RRC. Although it may depend on RAN1 discussion how many bits will be needed for the index field per on-demand SSB SCell, we think it could be enough to support two configurations for OD-SSB adaptation. If more than two OD-SSB configurations can be configured in RRC, it requires two or more bits per index field. In order to keep the MAC CE design simple, it is reasonable to limit it up to two OD-SSB configurations. Can ask RAN1 to check it if any concern.

Proposal 5:	RAN2 to support that at most two OD-SSB configurations can be configured per SCell in RRC. Inform RAN1 if any concern.

The index field can be optionally present. If the NW does not configure the multiple candidate values for all SCells, the NW does not include this field in the MAC CE.

In summary, Figure.1 (with up to 7 SCells and 2 OD-SSB configurations) and Figure.2 (with up to 31 SCells and 2 OD-SSB configurations) are examples of new MAC CE format design.

Figure 1: Example of OD-SSB activation/deactivation MAC CE (up to 7 SCells)

Figure 2: Example of OD-SSB activation/deactivation MAC CE (up to 31 SCells)

### Other on-demand SSB SCell procedure

In RAN1#120-bis meeting, the following agreement was made.

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

From RAN2 perspective, it can be interpreted that NW ensures the OD-SSB deactivation is not indicated by RRC/MAC CE after SCell activation is completed, in case of Case#1. In other words, OD-SSB deactivation by RRC/MAC CE is only performed during SCell deactivation state.

Proposal 6:	NW ensures that OD-SSB deactivation by RRC/MAC CE is only performed during SCell deactivation state. FFS how to capture it in running CR (e.g., Note X in TS 38.321).

# Conclusion

In this contribution, we have some discussions about on-demand SSB SCell operations, and the following proposals are made:

Framework of L3 measurements

Proposal 1:	RAN2 to adopt “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 2:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

Proposal 3:	RAN2 to confirm the legacy measurement framework (i.e., legacy SMTC) is applied for neighbour cell measurements.

On-demand SSB activation/deactivation MAC CE

Proposal 4:	No need to explicitly indicate the candidate values for OD-SSB via MAC CE.

Proposal 5:	RAN2 to support that at most two OD-SSB configurations can be configured per SCell in RRC. Inform RAN1 if any concern.

Other on-demand SSB procedure

Proposal 6:	NW ensures that OD-SSB deactivation by RRC/MAC CE is only performed during SCell deactivation state. FFS how to capture it in running CR (e.g., Note X in TS 38.321).

# References

- R2-2502981, Report from session on V2X/SL, R19 NES and MOB

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1

- RAN1#120-bis chairman’s note