3GPP TSG-RAN WG2#130	R2-2503806

St. Julians, Malta, May 19th – 23rd, 2025

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Remaining issues on on-demand SIB1 procedure

Document for:	Discussion and decision

# Background

In RAN2#129-bis meeting, RAN2 made some agreements as follows [1].

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution, we would like to discuss and provide our views on the remaining on-demand SIB1 issues.

# Discussion

## UL WUS procedure

### UL WUS configuration

Cell access information in UL WUS configuration

In the last meeting, RAN2 discussed whether to include cell access information (i.e., barring information) in UL WUS configuration, but it was postponed to the next meeting. If cell access information is provided beforehand, the UE can know the target/candidate NES cell is not suitable before SIB1 request, and it could be useful to avoid transmitting unnecessary SIB1 request.

Proposal 1:	Cell access information is optionally included in UL WUS configuration.

Basically, NES cell and Cell A will be deployed in the same area, and it needs inter-node coordination to have an energy saving gain. It implies NES cell and Cell A have similar access restriction policy. In practice, at least barring information of NES cell should be a subset (or the same) of that of Cell A. If the UE is barred by some barring information in Cell A, the UE cannot acquire SIBX from the Cell A and consequently the UE cannot trigger SIB1 request to the NES cell for cell reselection. Therefore, it is reasonable to assume that cell access information of NES cell is a subset (or the same) of that of Cell A.

Observation 1:	If the UE is not able to camp on Cell A by barring information, the UE cannot trigger SIB1 request.

On the other hand, barring information has been added in each release, (e.g., cellbarredNTN-r17, cellBarredRedCap1Rx-r17 in Rel-17, cellBarred2RxXR-r18, cellBarredNES-r18 in Rel-18). Furthermore, the UE behaviour is explicitly defined in the current TS 38.304, if some of barring information is not broadcast in SIB1. Considering forward compatibility, we think it is natural that a similar principle should be applied when barring information is not included in UL WUS configuration. That means all cell access information should be defined as optional and the NW can just indicate different cell access information in UL WUS configuration as delta configuration. In the last meeting, several companies were concerned about signalling overhead, but this assumption is useful to reduce a number of barring bits in UL WUS configuration.

Proposal 2:	If cell access information is not included in UL WUS configuration, the UE should follow the legacy UE behaviour.

Proposal 2a:	If cell access information is different between Cell A and NES cell, it can be indicated in UL WUS configuration as delta configuration.

Different values between OD-SIB1 and UL WUS configuration

In RAN1#120 meeting, RAN1 concluded that it is up to RAN2 whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration as follows.

Conclusion

It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

According to the RAN1 agreements, the UL WUS configuration could include different values for the same IEs provided in SIB1 of the NES cell. However, in the current specification, the UE simply applies or forwards the received IEs in SIB1 whenever it is acquired. Hence, there is no impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

Proposal 3:	RAN2 to confirm there is no specification impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

SIBX acquisition in NES cell in RRC connected mode

In RAN2#129-bis meeting, RAN2 agreed “SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF”. It means, in RRC connected mode, the UE in NES cell has a valid SIBX by legacy SI update, dedicated RRC message from the NW, or SI request for SIBX. In addition, as also agreed in RAN2#129-bis meeting that “SIBX is not defined as essential SI”, therefore in RRC connected mode, there is no specific UE behaviour to ensure the validity of SIBX in the NES cell. In other words, legacy UE behaviour is reused for acquiring SIBX of NES cell in RRC connected mode.

Proposal 4:	Legacy UE behaviour is used for acquiring SIBX of NES cell in RRC connected mode.

Handling of legacy excluded cell list

In the post e-mail discussion for 38.304 CR and 38.331 CR, a new open issue about the excluded cell list was raised, and it is captured in clause 5.2.4.1 in the 38.304 CR as an EN: FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided. This issue can be resolved by a new UE behaviour (options 1 and 2 in [3]) or NW implementation (option 3).

-	Option 1:	Support size 0 of the dedicated excluded cell list. The UE applies the dedicated excluded cell list, if configured. Otherwise, the UE applies the legacy list.

-	Option 2:	The UE always ignores the legacy excluded cell list received from a cell in which SIBX is provided, irrespective of whether dedicated excluded cell list being provided.

-	Option 3:	NW may configure dummy cell information in the dedicated excluded cell list.

In our view, option 1 is not aligned with the legacy RRC because currently cell list has at least one neighbour cell information, if configured. Hence, it may impact on UE implementation. Option 3 can work without specification changes, but it would be a bit tricky and seems workaround way. In addition, signalling overhead is slightly increased than other two options. In this sense, option 2 should be adopted.

Proposal 5:	The UE always ignores the legacy excluded cell list received from a cell in which SIBX is provided, irrespective of whether dedicated excluded cell list being provided.

### UL WUS transmission

On-demand SIB1 transmission status

In RAN1#120-bis meeting, it was discussed how/whether to check the on-demand SIB1 transmission status of the NES cell before the UL WUS transmission and RAN1 agreed it is up to UE implementation whether to monitor PDCCH for on-demand SIB1 as follows.

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

However, in our understanding, it was not aligned with the agreement in RAN2#127-bis meeting, because according to the highlighted part, it would be allowed to skip SIB1 transmission status check before UL WUS transmission. From power saving perspective, RAN2 agreed it is beneficial that “UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”, because when the SIB1 is transmitted on-demand, it is up to NW implementation whether to change or maintain the value of Kssb.

Observation 2:	It is up to NW implementation whether to change or maintain the value of Kssb in MIB when the SIB1 is transmitted on-demand.

In other words, such an UE behaviour is always required before requesting SIB1 and not allowed to skip. Otherwise, the UE may transmit unnecessary SIB1 request to NES cell. In clause 5.2.2.3.1 in the current RRC running CR, it correctly reflects the original RAN2 intention as follows:

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3> if the UE has a stored valid version of od-SIB1-Config for this cell:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5> if SIB1 is not broadcasted

6> perform the actions as specified in clause 5.2.2.3.3x;

Proposal 6:	RAN2 to reconfirm the UE will check SIB1 is being broadcasted in the NES cell before requesting SIB1. This should be caputured as normative text in TS 38.331 (as in the RRC running CR).

One question here is how long the UE needs to check the SIB1 is not broadcasted (i.e., monitor type-0 PDCCH) before triggering the SIB1 request. We think there are three options.

- Option 1: the same duration of SIB1 periodicity.

  - Based on the current periodicity, as SIB1 can be transmitted with 160 ms periodicity, therefore the UE may need to monitor type-0 PDCCH up to 160 ms for SIB1 acquisition.

- Option 2: provided in UL WUS configuration [2].

  - The UE can attempt to monitor type-0 PDCCH of NES cell based on the information in UL WUS configuration.

- Option 3: it is up to UE implementation how to monitor type-0 PDCCH for on-demand SIB1,

Considering specification impact, it should be up to UE implementation (i.e., option3). Because SIB1 might be acquired from the NES cell before cell reselection criterion is fulfilled, it is difficult to define the explicit timing/duration to check SIB1 transmission status of NES cell in the procedural text. Hence,  it is reasonable to capture it as informative text in TS 38.331.

Proposal 7:	It is up to UE implementation how to check SIB1 is not broadcasted before requesting SIB1. This should be captured as informative text in TS 38.331.

### On-demand SIB1 reception

On-demand SIB1 update in NES cell

In RAN2#127 meeting, it was agreed that UE keeps SIB1 updated while on the NES cell via regular SI modification procedure. An issue that can be further discussed is how long the network should transmit the on-demand SIB1 from the network modification period, as shown in the following figure.

Figure 1. On-demand SIB1 update in the NES cell

In our view, the network can transmit the on-demand SIB1 during the whole next modification period (i.e., legacy SI update), and the UE can assume the updated SIB1 is transmitted from the next modification period. If the UE is unable to acquire the updated SIB1, the UE should consider the NES cell as barred (i.e., it does not trigger SIB1 request), because the UL WUS configuration may be obsoleted due to SI update but the UE does not know it.

Proposal 8:	The updated SIB1 is transmitted during whole next modification period.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1 after SI update reception.

# Conclusion

In this contribution, the following proposals and observations were made:

Cell access information in UL WUS configuration

Proposal 1:	Cell access information is optionally included in UL WUS configuration.

Proposal 2:	If cell access information is not included in UL WUS configuration, the UE should follow the legacy UE behaviour.

Proposal 2a:	If cell access information is different between Cell A and NES cell, it can be indicated in UL WUS configuration as delta configuration.

Different values between OD-SIB1 and UL WUS configuration

Proposal 3:	RAN2 to confirm there is no specification impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

SIBX acquisition in NES cell in RRC connected mode

Proposal 4:	Legacy UE behaviour is used for acquiring SIBX of NES cell in RRC connected mode.

Handling of legacy excluded cell list

Proposal 5:	The UE always ignores the legacy excluded cell list received from a cell in which SIBX is provided, irrespective of whether dedicated excluded cell list being provided.

On-demand SIB1 transmission status

Proposal 6:	RAN2 to reconfirm the UE will check SIB1 is being broadcasted in the NES cell before requesting SIB1. This should be caputured as normative text in TS 38.331 (as in the RRC running CR).

Proposal 7:	It is up to UE implementation how to check SIB1 is not broadcasted before requesting SIB1. This should be captured as informative text in TS 38.331.

On-demand SIB1 update in NES cell

Proposal 8:	The updated SIB1 is transmitted during whole next modification period.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1 after SI update reception.

# References

- R2-2502981, Report from session on V2X/SL, R19 NES and MOB

- R2-2502149

- R2-2503608, Report of [POST129b][111][NES] (Ericsson)