3GPP TSG-RAN WG2 Meeting #130	R2-2503817

# St Julian’s, Malta, May 19th – 23rd, 2025

Source: KDDI Corporation, Samsung

Title:　 Discussion on singling design for Non-collocated feature

Agenda Item:	8.20.1	RAN4

Document for:	Discussion

- Introduction

In the following LS, RAN4 agreed to task RAN2 to develop relevant signalling for new UE capability and new BS signalling for Type 4a/4b related to intra-band non-collocated NR-CA, EN-DC feature [1]. In this contribution, we list some points to be discussed before working on CRs.

- Overall Description: RAN4 discussed the applicable UE requirements, new UE capability and new BS signalling for Type 4a/4b1 in previous meetings for which the following agreements were reached. RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signalling accordingly. UE capability Type - For intra-band non-contiguous NR-CA, Type 4b is considered, Type 4a is not considered - For inter-band non-contiguous EN-DC with overlapping or partially overlapping bands, both Type 4a and Type 4b are considered UE requirements: - Reuse the power imbalance of Type 2 - Reuse the MRTD and MTTD requirements of Type 2 UE capability: - Define one capability for NR CA and one capability for EN-DC - For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier - If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS) BS signalling: - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2 NOTE: maxMIMO-layers intends for the legacy BS signaling NOTE 1: Type4a/4b is mentioned in the WID RP-242083. 2. Actions: To RAN2: ACTION: RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signaling.

- Discussion

  - Interpretation of the RAN4 LS

From the above RAN4 LS, for NR-CA, RAN4 is requesting one new capability and one new BS signaling. While for EN-DC, For EN-DC, RAN4 considers that Type 4a and Type 4b can be distinguished using MIMO-CapabilityDL-r10, and therefore, one new UE capability and one new BS signaling would be sufficient. It seems until now the RAN2 guideline is to define signaling that reflects what the UE supports, rather than relying on UE capability. There is a possibility that having separate signaling and separate capabilities for Type 4a and Type 4b might provide a clearer understanding. We do not have a strong view on this matter and believe that if the majority of companies agree that one capability and one BS signaling work well, we can proceed with that approach. Otherwise, we can consider defining separate capabilities and BS signaling.

Proposal 1: RAN2 confirm to specify one capability and one BS signaling for NR-CA

Proposal 2: RAN2 discuss and confirm that one capability and one BS signaling for EN-DC, type 4a and type 4b capability can be differentiated by legacy capability, MIMO-CapabilityDL-r10 on E-UTRA carrier can work effectively. If not, separate capabilities and BS signaling may be considered.

From the RAN LS, it can also be understood that a Rel-19 UE supporting type 4a/4b should indicate its capability of type 2, which implies that type 2 is a prerequisite capability for type 4a/4b. It can also be inferred that the new Rel-19 BS signaling is provided to the UE only when the UE is configured with 4 MIMO layers. If the UE is configured with 2 MIMO layers, then the gNB will indicate the Rel-18 signaling, if supported.

Proposal 3: RAN2 confirm that a Rel-19 UE supporting type 4a/4b should also support and report type2 capability.

Proposal 4: RAN2 confirms that, when both the gNB and UE support the Rel-19 signaling associated with the non-collocated feature, the gNB may provide Rel-19 signaling. If the Rel-19 signaling is not supported by either the gNB or UE, the gNB using Rel-18 signaling associated with the Rel-18 non-collocated feature (if supported).

  - UE behaviour & BS signalling under 4 MIMO layers configuration

In Rel-18, BS signaling uses a "present/absent" indicator—where "present" indicates Type 1 and "absent" indicates Type 2—to instruct the UE to operate as Type 1(collocated) or Type 2(non-collocated).

Before discussing the detailed Rel-19 BS signaling design, we believe it is important to first establish a common understanding of the expected UE behavior — specifically, the RAN4 requirements that the Rel-19 UE should follow when configured with 4 MIMO layers but without Rel-19 BS signaling.

- 2.2.1 Rel-19 BS signaling is absent

Rel-19 UE applies the type 4 (non-collocated) requirements without Rel-19 BS signaling

Pros:

Aligns with the signaling design principle defined in Rel-18.

Cons:

May lead to a mismatch between a Rel-18 gNB and a Rel-19 UE and not power efficient for the Rel-19 UE, for the following reasons:

According to the Rel-18 specification, BS signaling is limited to indicating support for up to 2 MIMO layers. When a UE is configured with 4 MIMO layers—regardless of the presence or absence of Rel-18 BS signaling—it interprets this as an instruction to operate as a type 1 (collocated) UE. Therefore, if a gNB configures the UE with 4 MIMO layers and does not provide any Rel-18 signaling, the intention is to control the UE as a type 1 UE based on Rel-18 behavior.

However, since a Rel-18 gNB cannot provide the new Rel-19 BS signaling, a Rel-19 UE would interpret the absence of this signaling as "absent." As a result, the Rel-19 UE may interpret the combination of 4 MIMO layers and absent Rel-19 signaling as an instruction to operate as type 4 (non-collocated). This creates a mismatch between the UE's interpretation and the gNB’s intention, potentially leading to inconsistent behavior.

A type4 UE inherently supports type1 capability, so even if there is a mismatch between the UE an the gNB, the UE might still function properly and receive signals. However, if the UE can distinguish between a collocated scenario and a non-collocated scenario (when MaxMIMO-Layers = 4), it could optimize its behavior — for example, by deactivating some RF chains to save power. In the absence of an explicit indication from gNB, the UE have to keep all RF chains active, which may lea to increased power consumption.

Rel-19 UE follow the Rel-18 behavior without Rel-19 BS signaling e.g. applies the type 1 (collocated) requirements

Pros:

Avoids mismatch between Rel-18 gNB and Rel-19 UE behavior.

Power efficient for the UE.

Rel-18 and Rel-19 behaviors are aligned.

Cons:

Does not strictly follow the signaling design principle defined in Rel-18.

Both of the above solutions involve trade-offs, but in our view, the advantages of the second option introduce fewer issues and may offer a more practical path forward

Proposal 5: RAN2 discuss the pros and cons of the two options below and clarify that the UE follow the Rel-18 behavior has less demerits when

- the gNB does not support Rel-19 BS signaling, or

- the gNB supports Rel-19 BS signaling but does not provide it to the UE,

That is, when configured with 4 MIMO layers, the UE applies the type 1 (collocated) requirements as specified in Rel-18.

Op1: Rel-19 UE applies the type 4 (non-collocated) requirements without Rel-19 BS signaling

OP2: Rel-19 UE follow the Rel-18 behavior without Rel-19 BS signaling e.g. applies the type 1 (collocated) requirements

- 2.2.2 Rel-19 BS signaling is present

Based on the above proposal, we now consider the UE behavior when Rel-19 BS signaling is provided.

Rel-19 UE applies the type 4 (non-collocated) requirements with Rel-19 BS signaling

Pros:

Aligns with the behavior described in Proposal 5.

Allows a simplified ASN.1 structure using a single enumerated value {true}, similar to the approach used in Rel-18.

Cons:

Fundamentally reverses the signaling design principle defined in Rel-18, where absence of signaling implied a default behavior.

Example ASN.1 format

nonCollocatedType4MRDC-r19                  ENUMERATED {true}

nonCollocatedType4NR-CA-r19                 ENUMERATED {true}

Rel-19 UE applies either type1 (collocated) or type4 (non-collocated) requirements based on explicit Rel-19 BS signaling with value 0/1

Pros:

Also aligns with Proposal 5.

Treats Rel-19 BS signaling as an enhancement over the Rel-18 design, rather than a reversal

Makes the signaling more explicit and flexible.

Cons:

Requires the Rel-19 gNB to always include the BS signaling to explicitly indicate whether the UE should apply type1 or type4 behavior.

Deviates from the Rel-18 signaling principle, where signaling was only needed to override the default behavior.

Introduce ambiguity in interpretation due to the UE applies type 1 both when Rel-19 signaling is absent and when it is present with value 0/1

Example ASN.1 format

nonCollocatedType4MRDC-r19                  ENUMERATED {type1, type4}

nonCollocatedType4NR-CA-r19                 ENUMERATED {type1, type4}

Proposal 6: RAN2 discuss the pros and cons of the two options and select one of the options as a way forward

- Op1. Rel-19 UE applies the type 4 (non-collocated) requirements with Rel-19 BS signaling

- Op2. Rel-19 UE applies either type1 (collocated) or type4 (non-collocated) requirements based on explicit Rel-19 BS signaling with value 0/1

- 2.2.3 Reuse Rel-18 signaling vs new Rel-19 signaling

Instead of introducing new Rel-19 signaling, another option could be to extend the existing Rel-18 signaling to support up to 4 MIMO layers—provided companies can confirm that this does not introduce any backward compatibility issues.

Pros: Less effort required for RAN2 to update the related specifications.

Cons: Potential mismatch between the gNB and UE, and increased UE power consumption, as described in section 2.2.1 (UE applies type 4 without BS signaling).

We think it is much cleaner to have a new Rel-19 signaling associated to the Rel-19 non-collocated requirements.

Proposal 7: RAN2 discuss and decide whether to introduce new Rel-19 signaling, rather than extending the existing Rel-18 signaling to support 4 MIMO layers non-collocated requirements.

- 3. Conclusion

Proposal 1: RAN2 confirm to specify one capability and one BS signaling for NR-CA

Proposal 2: RAN2 discuss and confirm that one capability and one BS signaling for EN-DC, type 4a and type 4b capability can be differentiated by legacy capability, MIMO-CapabilityDL-r10 on E-UTRA carrier can work effectively. If not, separate capabilities and BS signaling may be considered.

Proposal 3: RAN2 confirm that a Rel-19 UE supporting type 4a/4b should also support and report type2 capability.

Proposal 4: RAN2 confirms that, when both the gNB and UE support the Rel-19 signaling associated with the non-collocated feature, the gNB may provide Rel-19 signaling. If the Rel-19 signaling is not supported by either the gNB or UE, the gNB using Rel-18 signaling associated with the Rel-18 non-collocated feature (if supported).

Proposal 5: RAN2 discuss the pros and cons of the two options below and clarify that the UE follow the Rel-18 behavior has less demerits when

- the gNB does not support Rel-19 BS signaling, or

- the gNB supports Rel-19 BS signaling but does not provide it to the UE,

That is, when configured with 4 MIMO layers, the UE applies the type 1 (collocated) requirements as specified in Rel-18.

Op1: Rel-19 UE applies the type 4 (non-collocated) requirements without Rel-19 BS signaling

OP2: Rel-19 UE follow the Rel-18 behavior without Rel-19 BS signaling e.g. applies the type 1 (collocated) requirements

Proposal 6: RAN2 discuss the pros and cons of the two options and select one of the options as a way forward

- Op1. Rel-19 UE applies the type 4 (non-collocated) requirements with Rel-19 BS signaling

- Op2. Rel-19 UE applies either type1 (collocated) or type4 (non-collocated) requirements based on explicit Rel-19 BS signaling with value 0/1

Proposal 7: RAN2 discuss and decide whether to introduce new Rel-19 signaling, rather than extending the existing Rel-18 signaling to support 4 MIMO layers non-collocated requirements.

- 4. Reference

[1] R4‑2505161		LS to RAN2 on Type 4 for non-collocated deployment

[2] R4-2217734       WF on NonCol_intraB_ENDC_NR_CA for New Type UE