3GPP TSG-RAN WG2 Meeting #R2-130		        R2-2503824      St Julians, Malta, May 19-23, 2025

Agenda Item:	8.5.2

Source: 	Rakuten

Title:  	Remaining issues of OD-SSB

Document for:	Discussion and decision

# Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP-240170) was agreed in RAN#102 [1], the WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

# Discussion

## UE triggered activation of OD-SSB cell

Agreement (RAN2#116)

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement (RAN2#116b)

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

RAN1#116 and RAN1#116b made below agreements:

Whenever a UE’s QoS requirements are not satisfied, a UE may request activation of NES Scell(s). For this purpose, we think UE-initiated OD-SSB should be supported.

A UE could be configured with minimum Scells (SSBs) to optimize power consumption. With time, the radio conditions of these configured SSBs may degrade and some other SSB may have a better radio condition. In such a scenario, it is beneficial for the UE to send a wake-up signal and request additional or updated SSB transmission.

UE triggered on-demand SSB operation is supported.

## L3 measurements of OD-SSB

Below options were discussed in the context of L3 measurements of OD-SSB. RAN2 needs to select whether the L3 measurement of OD-SSB is based on SMTC or it is based on the OD-SSB pattern or both.

- Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted - Need more time to think of.

L3 measurements are based on the measConfig, which uses measIds to link measObjects and reportConfigs. measObjects provides SMTC for the UE to monitor SSB. UE measurement reporting is also based on measIds.

For OD-SSB, the OD-SSB pattern of the serving cell is expected to be informed to the UE.

For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), UE can perform OD-SSB Scell measurements if the OD-SSB pattern is informed to the UE. There is no need of an SMTC for Scell measurement purposes.

For Case #2 (always-on SSB), there is always-on SSB within the legacy SMTC. Here, there are 2 options. UE ignores the always-on SSBs or UE measures both always-on SSB and the OD-SSB and combines them to produce the final measurement results.

From a simplicity pov, we think that the NW can make sure that the OD-SSB is transmitted with enough periods/occasions to satisfy the measurement requirements to ensure that UE doesn’t have to measure the always-on SSB.

We think combining the measurement results of always-on and no always-on SSB is complex.

L3 measurements of OD-SSB are performed based on OD-SSB pattern ignoring SMTC, both in case #1 and case #2.

Parameters required for performing OD-SSB measurements include SSB start offset, duration, periodicity and, a bitmap indicating the transmitted SSB indexes (similar to ssb-ToMeasure or ssb-PositionsInBurst).

The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB are indicated to the UE to enable L3 measurements of OD-SSB.

# Conclusion

We have the following proposals:

Proposal 1	UE triggered on-demand SSB operation is supported.

Proposal 2	L3 measurements of OD-SSB are performed based on OD-SSB pattern ignoring SMTC, both in case #1 and case #2.

Proposal 3	The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB are indicated to the UE to enable L3 measurements of OD-SSB.

# Reference

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

RAN1 Chairman Notes : RAN1 #116bis