3GPP TSG-RAN WG2 Meeting #130	R2-2503825

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.3

Source:	NEC

Title:	Open issues on random access for Ambient IoT devices

Document for:	Discussion, Decision

Introduction

In RAN2#129bis meeting, a good progress was achieved on random access for Ambient IoT.

This contribution further discusses remaining open issues of A-IoT random access, which are summarized as the output of the email discussion, [POST129bis][017] [AIoT] 38.391 Running CR (Huawei).

Discussion

Issue 3-5: D2R message type and Issue 2-9: AS ID assignment in multi-device CFRA

RAN2#129bis meeting reached following agreements on Msg1.

Agreements on msg1 - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes. - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario.

Regarding the first FFS, D2R message types in case of CBRA are summarized below:

- CBRA Msg1 containing 16 bits random ID only

- CBRA Msg3 containing upper layer data (device ID)

- D2R message containing upper layer data

Since the current assumption is that the upper layer data is transparent to the A-IoT MAC layer, we think there is no difference between CBRA Msg3 and D2R message containing upper layer data from A-IoT radio interface point of view.

In Rel-19, all D2R transmissions are scheduled by a corresponding R2D message. Therefore, we don’t see a need to support message type to differentiate D2R message containing upper layer data from CBRA Msg1. In our understanding, the only motivation to support message type in Rel-19 is for forward compatibility.

(MAC 3-5) RAN2 to determine whether to support message type for D2R message containing upper layer data in Rel-19 for forward compatibility.

Regarding the second FFS, in the SA2 agreed pCR for the command procedure [3], the AIOTF firstly performs Inventory procedure, and secondly for each successful Inventory response received, the AIOTF sends Command Request message to the A-IoT RAN. When multiple A-IoT device identifications are transferred to the reader during Inventory procedure, the reader may perform one CFRA for each A-IoT device identification and summarize all response to CFRA into an Inventory Report to AIOTF. In our understanding, multiple device scenarios may exist during Command Request procedure.

(MAC 2-9) Multiple device scenarios may exist during Command Request procedure.

To handle multiple devices during Command Request procedure, the agreed AS ID is sufficient for addressing the device in R2D command message.

(MAC 2-9) RAN2 confirm that AS ID is sufficient for addressing the device in R2D command message for the multiple device scenario during Command Request procedure.

Issue 1-4: AO number field (How to indicate the number of access occasions)

On Msg1 resource, RAN1#120 meeting achieved the following agreement for FDMA of Msg1 in CBRA:

Agreement

Support FDMA with Y≥1 D2R transmissions for Msg1 from multiple devices in response to a R2D transmission triggering random access.

Furthermore, RAN1#120bis achieved following agreements for TDMA of Msg1 in CBRA:

Agreement
For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19.
•  All devices support the above
•  Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized

(MAC 1-4) Both TDMA and FDMA are supported for Msg1 for Rel-19 A-IoT CBRA.

Then, the slotted-ALOHA-based access utilizes TDMA (with a maximum of 2 time domain resources) and FDMA (using Y frequency domain resources), as illustrated in Fig. 1.

Fig.1 Slotted-ALOHA-based access: TDMA (X=2), FDMA (Y freq. domain resources) for Msg1

The A-IoT MAC running CR defines an access occasion as “An opportunity of time-frequency resource for device(s) to transmit Msg1 (i.e., the Random ID message) during a CBRA procedure”. Since both TDMA and FDMA are supported for Msg1 resources, a set of access occasion(s) (referred to as an Access Occasion Set in Fig.1) contains X time domain resources and Y frequency domain resources, as shown in Fig.1. It is proposed to define access occasion set on top of access occasions.

(MAC 1-4) Access Occasion Set is defined as a set of access occasions, i.e., an access occasion set hosts X*Y access occasions.

On the R2D transmission, which determines the Msg1 time resource(s), RAN2#129bis reached following agreements:

Agreements on new R2D message 1. A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. 2. Assumption: The R2D message does not include slot number/count down number.

RAN1 is discussing the starting time for the first and second Msg1 time domain resource and RAN1#120bis has agreed the following agreement and work assumption:

Agreement

For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access.

•	FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access.

•	FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact

•	FFS: how to define the end of the R2D transmission in 9.4.1

Working assumption

For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource:

•	Derived based on the starting time of the first Msg1 time domain resource plus Toffset2

•	FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

Therefore, as we understand it, the newly introduced R2D message serves to indicate the start of access occasion sets that were configured in the paging message. Thus, we propose naming this new R2D message the 'Access Occasion Set Trigger message,' as shown in Fig. 1.

(MAC 1-4) The new R2D message introduced for A-IoT devices to determine Msg1 resources indicates the start of an access occasion set. It is named the "Access Occasion Set Trigger message."

As shown in Fig.1, the following factors indicate Msg1 transmission:

- the number of Access Occasion Sets (M in time domain)

- the number of time domain resources within an access occasion set (X)

- the number of frequency domain resources within an access occasion set (Y)

Since Msg1 resources have been agreed to be included in the A-IoT paging message, we believe it is straightforward for the same number of time domain resources (X) and frequency domain resources (Y) to apply to all access occasion sets.

(MAC 1-4) RAN2 confirm that all access occasion sets triggered by an A-IoT paging message have the same number of time domain resources (X) and frequency domain resources (Y).

RAN1#120bis meeting agreed that “Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s).”

(MAC 1-4) RAN1 has agreed to use 1 bit to indicate the value of X time domain resources for Msg1 transmission.

It is reasonable to assume that the value of Y frequency domain resources for Msg1 transmission is also explicitly included in A-IoT paging message. This way, the A-IoT paging message can encompass all three factors to notify devices about the number of Msg1 resources.

(MAC 1-4) The number of Msg1 resources included in the A-IoT paging message is represented by the following parameters:
- the number of Access Occasion Sets in time domain (M)
- the number of time domain resources within an access occasion set (X)
- the number of frequency domain resources within an access occasion set (Y)

Issue 2-1: Msg1 resource selection

Upon receiving the above three parameters, there needs to be a discussion on how A-IoT devices that meet filter criteria should select their Msg1 resources accordingly.

There may be three options for the A-IoT device to randomly choose a Msg1 resource.

Option-1: The A-IoT device randomly picks three random values within the ranges (1, M), (0, X-1), and (0, Y-1) to select the access occasion set and the time and frequency resources within the chosen access occasion set.

Option-2: The A-IoT device randomly selects an access occasion set from all available sets (i.e., M sets) and then randomly chooses an access occasion from the available ones within that access occasion set (i.e., X*Y resources).

Option-3: The A-IoT device can randomly choose a value among (0, (M*X*Y-1)) and map that random value to an access occasion among all of the available access occasions.

(MAC 2-1) RAN2 to discuss the following options for the A-IoT device to select resource (i.e., access occasion) for Msg1 transmission upon receiving A-IoT paging message for CBRA on selection of Msg1 resources
Option1: randomly picks three random values within the ranges (1, M), (0, X-1), and (0, Y-1) to select the access occasion set and the time and frequency resources within the chosen access occasion set. 
Option 2: firstly, randomly choose an access occasion set from all available sets (i.e., M sets) and then randomly chooses an access occasion from the available ones within that access occasion set (i.e., X*Y resources). 
Option-3: randomly choose a value among (0, (M*X*Y-1)) and map that random value to an access occasion among all of the available access occasions.

Since the starting times of the first and second Msg1 time domain resources within an access occasion set will be defined by RAN1, we understand that only one counter is needed to determine the access occasion set, similar to the slot counter in RFID.

(MAC 2-1) One counter is needed for counting the Access Occasion set (similar to the slot counter in RFID).

The “Access Occasion Set Trigger message” is used to indicate the start of an access occasion set. The expected device behavior upon receiving the “Access Occasion Set Trigger message” is as follows:

- decrement its Access Occasion Set counter by 1.

- if the Access Occasion Set counter equals to 0 after the decrement, reply to the A-IoT paging message immediately using the randomly picked time/frequency domain resources within the current Access Occasion Set.

(MAC 2-1) The expected device behavior upon receiving the “Access Occasion Set Trigger message” is:
- decrement its Access Occasion Set counter by 1.
- if the Access Occasion Set counter equals to 0 after the decrement, reply to the A-IoT paging message immediately using the randomly picked time/frequency domain resources within the current Access Occasion Set.

Issue 2-2: Paging & first R2D trigger message

So far, the discussion of the R2D trigger message focused on CBRA, and it’s not crystal clear whether the R2D trigger message is also needed in CFRA, and whether it can be merged to paging message if it’s the first R2D trigger message in CBRA. In our understanding, the R2D trigger message (i.e, Access Occasion Set Trigger message) is used to indicate the start of an access occasion set. Then for CFRA paging intending to a single device, such a R2D trigger message is not needed since there is only one access occasion set included in the CFRA paging. Also, for CBRA paging case, the first Access Occasion Set can be triggered by the A-IoT paging message, therefore such a R2D trigger message is not needed.

(MAC 2-2) RAN2 confirm that the R2D trigger message used to indicate the start of AO sets is not needed for CFRA paging and the first AO set in CBRA paging.

Issue 2-5: random ID differentiation in Msg2

RAN2#129bis reached following agreements on Msg2 for CBRA:

Agreements on Msg2 1. A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. 2. Same Msg2 format is used for initial transmission and retransmission of Msg2.

Since both TDMA and FDMA are supported for Msg1, there would be a case in which multiples devices generate the same random ID using different Msg1 resources, i.e., random ID collision. In such a case, the reader can decide to allow one device to reuse the random ID whereas assign a new AS ID to others and indicate its decision to devices by using Msg2. Therefore, to correctly indicate the reader’s decision (whether to reuse the generated random ID or to use the new assigned AS ID) to devices, only including the random ID in Msg2 is not sufficient. In other words, Msg2 needs to include the Msg1 resource index to differentiate devices which generate the same random ID.

(MAC 2-5) To correctly indicate the reader’s decision (whether to reuse the generated random ID or to use the new assigned AS ID) to devices which generate the same random ID, Msg2 needs to include the Msg1 resource index.

For A-IoT Msg2 containing multiple echoed random ID(s), including Msg1 resource index for each echoed random ID(s) is in-efficient since such Msg1 resource index is not needed for collision free devices. Moreover, including Msg1 resource index for each echoed random ID(s) may raise the concern on Msg2 size. On the other hand, to indicate whether Msg1 resource index is included or not per echoed random ID(s) may increase the complex of device behavior. Therefore, we propose to include the Msg1 resource index at least in A-IoT Msg2 containing one echoed random ID.

(MAC 2-5) Support to include the Msg1 resource index at least in A-IoT Msg2 containing one echoed random ID.

Issue 2-6: number indication of echoed random IDs

When the Msg1 resource index is only included in A-IoT Msg2 containing one echoed random ID, the multiplexed Msg2 can be used to echo collision free random IDs, i.e., only include random ID in the multiplexed Msg2 is sufficient. Then as Rapporteur proposed in the email discussion, it is reasonable to assume all the other parts in Msg2 (i.e. message type, scheduling info if any) have fixed length and are put in the beginning of the MAC PDU, the device can consider the left part is random ID/random ID list, and then decode random ID one by one. Therefore,

(MAC 2-6) No need to indicate the number of random ID(s) in the multiplexed Msg2 assuming all the other parts in Msg2 (i.e. message type, scheduling info if any) have fixed length and are put in the beginning of the MAC PDU, and the device can decode the random ID one by one.

Issue 2-10: NACK before paging or R2D trigger message

RAN2#129bis reached following agreements on NACK based mechanism:

Agreements on NACK 1. For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). 2. For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging. 3. For CFRA, NACK feedback and re-access is not supported. FFS how to achieve. 4. FFS on end of procedure

SA2#168 meeting agreed following command procedure of TS 23.369 in [3]. Steps in which A-IoT RAN is involoved are listed below:

- Step 7 in [3]: Step 7 to Step 11 of Procedure for Inventory in Clause 6.2.2 of TS23.369 (copied below)

  - Step 7 in TS23.369 6.2.2: The AIOTF sends the Inventory Request message to the selected NG-RAN.

  - Step 8 in TS23.369 6.2.2: The NG-RAN sends an Inventory Response to the AIOTF.

  - Step 9 in TS23.369 6.2.2: Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation.

  - Step 10 in TS23.369 6.2.2: NG-RAN sends one or more Inventory Report messages to the AIOTF.

  - Step 11 in TS23.369 6.2.2: The AIOTF validates the results.

- Step 8 in [3]: For each successful Inventory response received, the AIOTF sends Command Request message to the A-IoT RAN.

- Step 9 in [3]: The A-IoT RAN sends the AS R2D message (NAS Command Request) to the A-IoT Device.

- Step 10 in [4]: The A-IoT Device sends the AS D2R message (NAS Command Response) to the A-IoT RAN. The NAS Command Response message may include the A-IoT data.

Regarding how to merge the CBRA random access procedure into the above command procedure, a reasonable approach is that the reader to perform the command procedure for each access occasion set, as illustrated in Fig. 3.

Fig.3 One candidate for merging the CBRA random access procedure into the command procedure

It is obvious that the D2R data procedure is only performed by devices that have already successfully completed the Inventory procedure. When D2R data retransmission is needed in some failure cases, the reader can choose to repeat the R2D transmission to trigger the D2R data retransmission, meaning that re-access based on NACK (i.e., re-access from Inventory procedure) is not needed.

(MAC 2-10) RAN2 confirm that for CBRA, NACK based mechanism is applied only to Msg3.

Regarding the subsequent R2D message for determining re-access, RAN2#129 meeting reached following agreement:

Agreements

3. Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

We don’t see any motivation and/or need to re-discuss the R2D message for triggering/determining re-access.

(MAC 2-10) RAN2 confirm the RAN2#129 agreement to reuse the A-IoT paging message to trigger re-access. No other R2D message is supported for triggering or determining re-access.

Issue 2-12: multiplexing for NACK indication

Another issue is whether to support a NACK indication that contains multiple AS IDs, similar to Msg2. We understand that such multiplexed NACK transmission could be beneficial for reducing overhead. Considering that the Msg2 multiplexing mechanism can be reused for the NACK case, we prefer to support a NACK indication with multiple AS IDs in Rel-19.

(MAC 2-12) Support a NACK indication that contains multiple AS IDs.

Issue 2-8: no re-access for CFRA

Regarding how to achieve the last meeting agreement that “For CFRA, NACK feedback and re-access is not supported.”, our understanding is that the expected device behaviour is to always respond to a CFRA without considering whether it has responded before or not.

(MAC 2-8) Device always responds to a CFRA if it is identified by the CFRA without considering whether it has responded before or not.

Conclusion and Proposal

We have the following observations and proposals:

- Observations:

Observation 1	(MAC 2-9) Multiple device scenarios may exist during Command Request procedure.

Observation 2	(MAC 1-4) Both TDMA and FDMA are supported for Msg1 for Rel-19 A-IoT CBRA.

Observation 3	(MAC 1-4) RAN1 has agreed to use 1 bit to indicate the value of X time domain resources for Msg1 transmission.

Observation 4	(MAC 2-5) To correctly indicate the reader’s decision (whether to reuse the generated random ID or to use the new assigned AS ID) to devices which generate the same random ID, Msg2 needs to include the Msg1 resource index.

Proposals:

Proposal 1 (MAC 3-5) RAN2 to determine whether to support message type for D2R message containing upper layer data in Rel-19 for forward compatibility.

Proposal 2	(MAC 2-9) RAN2 confirm that AS ID is sufficient for addressing the device in R2D command message for the multiple device scenario during Command Request procedure.

Proposal 3 (MAC 1-4) Access Occasion Set is defined as a set of access occasions, i.e., an access occasion set hosts X*Y access occasions.

Proposal 4 (MAC 1-4) The new R2D message introduced for A-IoT devices to determine Msg1 resources indicates the start of an access occasion set. It is named the "Access Occasion Set Trigger message."

Proposal 5	(MAC 1-4) RAN2 confirm that all access occasion sets triggered by an A-IoT paging message have the same number of time domain resources (X) and frequency domain resources (Y).

Proposal 6 (MAC 1-4) The number of Msg1 resources included in the A-IoT paging message is represented by the following parameters:

- the number of Access Occasion Sets in time domain (M)

- the number of time domain resources within an access occasion set (X)

- the number of frequency domain resources within an access occasion set (Y)

Proposal 7	(MAC 2-1) RAN2 to discuss the following options for the A-IoT device to select resource (i.e., access occasion) for Msg1 transmission upon receiving A-IoT paging message for CBRA on selection of Msg1 resources

Option1: randomly picks three random values within the ranges (1, M), (0, X-1), and (0, Y-1) to select the access occasion set and the time and frequency resources within the chosen access occasion set.

Option 2: firstly, randomly choose an access occasion set from all available sets (i.e., M sets) and then randomly choose an access occasion from the available ones within that access occasion set (i.e., X*Y resources).

Option-3: randomly choose a value among (0, (M*X*Y-1)) and map that random value to an access occasion among all of the available access occasions.

Proposal 8	(MAC 2-1) One counter is needed for counting the Access Occasion set (similar to the slot counter in RFID).

Proposal 9	(MAC 2-1) The expected device behavior upon receiving the “Access Occasion Set Trigger message” is:

- decrement of its Access Occasion Set counter by 1.

- if the Access Occasion Set counter equals to 0 after the decrement, reply to the A-IoT paging message immediately using the randomly picked time/frequency domain resources within the current Access Occasion Set.

Proposal 10 (MAC 2-2) RAN2 confirm that the R2D trigger message used to indicate the start of AO sets is not needed for CFRA paging and the first AO set in CBRA paging.

Proposal 11 (MAC 2-5) Support to include the Msg1 resource index at least in A-IoT Msg2 containing one echoed random ID.

Proposal 12 (MAC 2-6) No need to indicate the number of random ID(s) in the multiplexed Msg2 assuming all the other parts in Msg2 (i.e. message type, scheduling info if any) have fixed length and are put in the beginning of the MAC PDU, and the device can decode the random ID one by one.

Proposal 13 (MAC 2-10) RAN2 confirm that for CBRA, NACK based mechanism is applied only to Msg3.

Proposal 14 (MAC 2-10) RAN2 confirm the RAN2#129 agreement to reuse the A-IoT paging message to trigger re-access. No other R2D message is supported for triggering or determining re-access.

Proposal 15 (MAC 2-12) Support a NACK indication that contains multiple AS IDs.

Proposal 16 (MAC 2-8) Device always responds to a CFRA if it is identified by the CFRA without considering whether it has responded before or not.

Reference

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

- TR38.769

- S2-2504286, Update on Command procedure,