3GPP TSG-RAN WG2 Meeting #130	                                    R2-2503826

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:		8.1.3

Source:			Sharp

Title:		Discussion on NW-side Data Collection

Document for:	 	Discussion

# Introduction

During Rel-18, data collection for the training of NW-side models was discussed with the following agreements in TR 38.843 [1]. The following principles apply to all the currently identified used cases (i.e., the beam management (BM), the positioning, and the CSI compression and prediction use cases).

- A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging - UE to report the collected data periodically, event-based, and on-demand - The UE memory, processing power, energy consumption, signalling overhead should be considered

In RAN2#129bis meeting, the following agreements were made [2].

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

In our view, some issues are addressed for the BM use cases and can be further designed for the next step, while for the positioning use cases, it is still not clear how the NW-side data collection can be achieved. As a result, in this contribution, we would like to share our views on 1) Further details of the data collection in BM use cases (or general to BM use cases and positioning use cases) and 2) data collection designs specific to the positioning use cases.

# Discussion

## UE Capability for Data Collection

First, since the measurements are performed by the UE, the data collection for NW-side model needs both the consent and the capability of the UE. That is, whether the UE measures the SSB/CSI-RS (for BM use cases) and PRS (for positioning use cases) and reports the result for NW-side data collection is based on the UE capability. The NW can provide configurations for NW-side data collection and obtain the measurement results for NW-side model only when the UE supports the function of NW-side data collection and is able to perform such function (e.g., has sufficient power and memory for data collection for NW-side model).

In our views, for BM use cases, the existing RRC messages UECapabilityEnquiry and UECapabilityInformation can be used to request/indicate the UE’s capabilities related to data collection for NW-side model, while the existing LPP messages ProvideCapabilities and RequestCapabilities can achieve the coordination for positioning use cases. Specifically for the positioning use cases, the UE may not be capable of some UE-based positioning methods (e.g., DL TDoA) to acquire a ground truth label, but may be able to perform the measurement and collect the data (e.g., RSTD). In this case, the UE can provide such information in the corresponding IEs (e.g., DL-TDOA-ProvideCapabilities) to indicate the detailed capabilities. Alternatively, a new IE specifically for AI/ML can be considered. For example, the LMF and the UE can exchange the AI/ML-related capabilities via AIML-RequestCapabilities and AIML-ProvideCapabilities messages. In our views, we consider the new IE for AI/ML would be beneficial to incorporate all the capabilities related to different positioning methods.

Additionally, it was agreed to have a minimum AS layer memory size in the last meeting. Considering that the UE’s memory status configured for data collection may vary, we propose that the UE includes the memory size information in the RRC/LPP message providing the capability information. The memory size information can be an absolute value or can be an estimated value (e.g., the maximum number of instances that a UE can log).

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information.

## Positioning-Specific NW-side Data Collection

For positioning use cases, the measurement configuration can take the legacy ProvideAssistanceData message as a baseline. In our view, the LMF should be capable of flexibly switching among different NW-side models corresponding to different positioning methods (e.g., TDOA, AOD, Multi-RTT). As a result, LMF needs to collect data with different measurement quantities (e.g., RSTD, RSRP, RxTxTimeDiff, etc.) in different additional conditions, and the LMF needs to explicitly indicate the required measurement quantities in the measurement configuration. To allow the LMF to dynamically configure the measurement quantities, we propose to use a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) to configure the UE what to measure. Similar to the legacy ProvideAssistanceData message, the AIML-ProvideAssistanceData message contains PRS resource configurations; additionally, it also configures whether one or more of the measurement quantities (i.e., RSTD, RSRP, RxTxTimeDiff ) need to be measured by the UE. In the AIML-ProvideAssistanceData message, the associated ID (or the area ID or CGI) which identifies the NW-side additional conditions can also be provided.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Proposal 4: For Positioning use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

According to RAN2#127bis agreements, the periodic and event-based reporting is not supported for BM use case. In our view, the same concept can be applied to the positioning use cases for the reporting behavior; namely, on-demand reporting is supported, while periodic and event-triggered reporting is not supported. Also, since the event-triggered logging is supported, the UE’s indication of data availability would be pursued.

Proposal 5: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

As for the report configuration, the legacy RequestLocationInformation message can be taken as a baseline. Considering some possible AI/ML-specific parameters that can be leveraged for the data collection, we propose a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration. The AIML-RequestLocationInformation message) as the report configuration can include the following information:

- A counter to limit the number of logged instances.

- A condition to trigger the UE to start logging.

- An indication whether the ground truth label obtained by the UE should be included in the report.

- The measurement quantities to be reported.

For the above-mentioned information, the first and the second bullets define the logging behavior from the perspective of UE memory and from the perspective of the channel status. Depending on the UE’s capability to obtain the ground truth label, the LMF can determine whether to include the indication to request that the UE reports the ground truth label along with the measurement results. Additionally, since multiple measurement quantities can be configured for the UE to measure, it would be flexible for the LMF to decide which measurement quantities are needed at some given time. In addition, since the time at which the UE obtains the measurement result (e.g., the RSTD) and the time at which the UE obtains the ground truth label (e.g., the location) may be different, the LMF can provide a validity period to the UE. The UE can then associate the measurement results with the ground truth label according to the validity period. By doing so, the timestamps of the measurement result and the ground truth label are not needed to be reported.

Observation 2: For Pos use cases, the UE may obtain the ground truth label by itself.

Proposal 6: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 7: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 8: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label.

As for the pos use cases, based on the analysis above, it is desirable to have on-demand reporting and an indication of data availability. For the data availability indication, it can be achieved by adding the indication in the ProvideLocationInformation message. As for the on-demand request/report procedure, the messages RequestLocationInformation and ProvideLocationInformation can be reused.

Proposal 9: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

# Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Observation 2: For Pos use cases, the UE may obtain the ground truth label by itself.

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information.

Proposal 4: For Positioning use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

Proposal 5: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

Proposal 6: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 7: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 8: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label.

Proposal 9: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

# Reference

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, December 2023

- “Draft Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China”, Chairman (InterDigital).