3GPP TSG-RAN WG2 Meeting #130	R2-2503835

Malta, 19 – 23 May, 2025

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

In RAN2 #129-Bis meeting, several agreements for RLC enhancement [1] were made, which are provided in the Appendix. In this contribution we further discuss the RLC enhancement topics. Specifically, we further discuss enhancement to avoid unnecessary retransmission and timely RLC retransmission.

# Discussion

  - 2.1 Timely RLC retransmissions

Autonomous retransmission:

According to legacy [2], retransmission of an RLC SDU (segment) due to NACK status report is always prioritized over initial transmission of another RLC SDU (segment). Another issue is how to prioritize between autonomous retransmission of an RLC SDU and retransmission of another RLC SDU (segment) due to NACK status report. It is possible that the RLC SDU (segment) considered for autonomous retransmission has been successfully received from the receiver RLC entity. On the other hand, the RLC SDU (segment) subject for retransmission due to NACK report from the receiver absolutely requires retransmission to be successfully received. Therefore, this RLC SDU (segment) should be prioritized over any other autonomous retransmission RLC SDU (segment).

Proposal 1:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

Since autonomous retransmission is performed only once for an RLC SDU, it seems unnecessary to have any special handling for the maximum re-tx count triggered RLF – NW could take such additional re-transmission trigger into account when configuring the maximum re-tx threshold. On the other hand, since the auto re-tx might not still be triggered when there is a problem in the channel, another more viable option could be to disregard the autonomous re-transmission trigger in the RETX_COUNT incrementing,

Proposal 2:	Disregard the autonomous re-transmission trigger in the RETX_COUNT incrementing.

Polling enhancement:

As RAN2 agreed in the last meeting that polling can be triggered when the remaining time of a RLC SDU (segment) falls below a threshold. Like autonomous retransmission case, the remaining time of a RLC SDU (segment) may fall below a threshold before the original RLC SDU is completely transmitted. In this case, enabling polling may not be beneficial as no status for such RLC SDU is expected in the status report. It is necessary to enable polling bit when the RLC SDU is completely transmitted. In another words, polling bit can be enabled at earliest when the RLC SDU or the last segment of the RLC SDU is being transmitted.

Proposal 3: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

One remaining issue for polling enhancement is whether/how to avoid excessive polling for polling enhancement as described in the following editor note:

Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

In the legacy polling procedure, polling bit is enabled in one AMD PDU upon polling triggering condition is satisfied. Therefore, or remaining time based RLC polling, polling bit should be enabled in only one AMD PDU when the remaining time of a RLC SDU falls below a threshold. In case remaining time based RLC polling has been met for multiple RLC SDUs before one transmission occasion, enabling polling bit in one AMD PDU is sufficient for the Rx RLC entity to detect polling indication.

Another issue for remaining time based RLC polling is whether to prohibit the UE to perform polling in subsequent transmission when polling is enabled in one previous transmission. In our view, the UE may have multiple RLC PDUs satisfying the remaining time based RLC polling at different time. In this case, excessive polling may happen, and the UE may enable polling in multiple transmissions. Excessive polling may be redundant as the UE may already trigger status reporting due to reception of the first polling. Therefore, it is beneficial to introduce a prohibit timer for remaining time based RLC polling to prohibit the UE from excessive polling. The timer may be started when polling bit for remaining time based RLC polling is enabled. The UE may not be allowed to enable polling when the timer is running.

Proposal 4:	Upon remaining time based RLC polling for one or multiple RLC SDUs has been met, polling is enabled in only one AMD PDU in one transmission opportunity.

Proposal 5:	Introduce a prohibit timer for remaining time based RLC polling, which is started when polling bit for remaining time based RLC polling is included in an AMD PDU.

  - 2.2 Avoiding unnecessary retransmissions

In the previous meeting, the following was agreed wrt. t-RxDiscard expiry:

- When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

If t-Rassembly is running when the SR (Status Report) is triggered by the t-RxDiscard, the HARQ reordering may still be ongoing and by using the Rx_Highest_Status variable, the Rx RLC entity might report some of the RLC SDUs as missing which may still be under the HARQ reordering. Hence, when the SR is triggered by the t-RxDiscard, the SR should acknowledge SDUs only up to the RLC SDU that is still not considered as obsolete – that should also limit the SR size basically just to ACK_SN.

Proposal 6:	When the Status Report is triggered by the expiry of t-RxDiscard, the ACK_SN is set to first RLC SDU which is not considered as obsolete in the receiving RLC entity.

# Conclusion

In this contribution, the following proposals are made for timely RLC retransmission and avoiding unnecessary RLC retransmission of RLC enhancement:

Proposal 1:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

Proposal 2:	Disregard the autonomous re-transmission trigger in the RETX_COUNT incrementing.

Proposal 3: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

Proposal 4:	Upon remaining time based RLC polling for one or multiple RLC SDUs has been met, polling is enabled in only one AMD PDU in one transmission opportunity.

Proposal 5:	Introduce a prohibit timer for remaining time based RLC polling, which is started when polling bit for remaining time based RLC polling is included in an AMD PDU.

Proposal 6:	When the Status Report is triggered by the expiry of t-RxDiscard, the ACK_SN is set to first RLC SDU which is not considered as obsolete in the receiving RLC entity.

# References

- RAN2#129-Bis chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: RAN2 #129-Bis on RLC enhancement for XR

Agreements on RLC enhancements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.