3GPP TSG-RAN WG2 Meeting #130	R2-2503836

Malta, 19 – 23 May, 2025

Agenda item:	8.7.6

Source:	InterDigital

Title:	Discussion on UL congestion signaling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2#129-Bis reached the following agreements:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

# 2	Discussion

## 2.1	Identification of a QoS flow for rate control

From RAN2 point of view, the per QoS flow indication for XR rate control was agreed and it should be discussed how the QoS flow is identified in the Uplink Rate Control MAC CE. Since different PDU sessions can use the same QFI for different QoS flows, the QFI itself would not suffice as an identification of a QoS flow. On the other hand, using a DRB ID/LCID on top of the QFI to identify the PDU session may be unnecessarily long given the UE would not usually be configured with as many PDU sessions as it is configured with DRBs.

However, since the LCID is already known to the MAC entity and since the Uplink Rate Control MAC CE is rather a small MAC CE, it seems simplest to use the LCID + QFI combination to identify a specific QoS flow for UL rate control.

Proposal 1: Use the LCID + QFI (6 + 6 bits) combination to identify a specific QoS flow for UL rate control.

## 2.2	Single or multiple QoS flow entries in the UL rate control MAC CE

Another question is whether to allow including multiple QoS flow entries in the new UL rate control MAC CE to be able to provide rate control information for multiple QoS flows concurrently. Since the new MAC CE will be using eLCID space, mandating NW to send a separate MAC CE for every QoS flow it intends to provide the throttling information seems unnecessary. Hence, indicating multiple QoS flows should be enabled.

Proposal 2: The new UL Rate Control MAC CE can provide UL rate control information for multiple QoS flows.

However, whether to allow full flexibility where the UL rate control MAC CE could provide throttling information over multiple DRBs and QoS flows requires discussion. One option is that one UL rate control MAC CE can provide rate control information for QoS flows within a DRB (ie., only one LCID indication would be required in this case). Another option could allow full flexibility where multiple pairs of LCID + QFI can be provided. In any case, it seems useless to provide same LCID more than once in the UL rate control MAC CE in either of the options, which would allow some overhead savings when many QoS flows need to be indicated at the same time.

Proposal 3: For a DRB, only one LCID indication within the UL rate control MAC CE is required which can follow number of QFIs for QoS flows for which the rate control information is provided for the given DRB.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: Use the LCID + QFI (6 + 6 bits) combination to identify a specific QoS flow for UL rate control.

Proposal 2: The new UL Rate Control MAC CE can provide UL rate control information for multiple QoS flows.

Proposal 3: For a DRB, only one LCID indication within the UL rate control MAC CE is required which can follow number of QFIs for QoS flows for which the rate control information is provided for the given DRB.