3GPP TSG-RAN WG2 Meeting #130										R2-2503869

St Julian’s, Malta, 19 – 23 May 2025

Agenda item		: 8.4.4

Title				: Open issues on LP-WUS operation in CONNECTED mode

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

This contribution discusses the open issues related to LP-WUS operation in CONNECTED mode.

As part of the R2#129bis post email discussion, following open issues were identified:

- Open issue 1: Whether/How to support the LP-WUS (including Option 1-1 and 1-2) and dual DRX group.

- Open issue 2: In Option 1-2, whether the UE should start the lpwus_PDCCHMonitoringTimer (as if LP-WUS was detected) when the UE is not able to monitor the LP-WUS occasion(s).

# Discussion

## 2.1 On supporting LP-WUS and Dual DRX case:

One of the open issues is the support of LP-WUS along with dual DRX groups. In DCP case, this configuration is not allowed and DCP along with dual DRX groups. Its simpler to extend this principle for LP-WUS but this reduces the power saving that can be achieved using LP-WUS feature which is of more significance in case of FR1-FR2 CA case (where the dual DRX scenario occurs). It is beneficial to support LP-WUS in case of CA with dual DRX but with minimal RAN1 at this stage in release.

Its simpler design to support monitoring of LP-WUS in PCell/PSCell only and activate cells belonging to both the DRX groups.

Proposal 1: Support LP-WUS monitoring when dual DRX groups are configured

Proposal 2: LP-WUS monitoring is configured only in PCell/PSCell and upon receiving LP-WUS, the corresponding DRX timer (legacy onDurationTimer option 1-1and timer related to option 1-2) is started in cells associated to both the DRX groups

2.2 Enabling/disabling LP-WUS operation in connected mode:

In CONNECTED mode, the UE can be configured by the network with LP-WUS configuration to monitor LP-WUS. The network can indicate an implicitly enabling/disabling the LP-WUS feature by providing the UE with the related configurations. Since LP-WUS operation in Connected mode serves both for power saving (Option 1-1) and also for reducing latency (Option 1-2), network can configure the mode of LP_WUS operation based on the UE’s traffic requirements. Since these requirements do not change very frequently, there is no need for dynamic enabling/disabling of the feature based on L1 (DCI based) or L2 (MAC based) signaling. In case the network wants to disable the LP-WUS feature, it can reconfigure the UE to release the LP-WUS configuration.

Proposal 3: LP-WUS feature is enabled implicit at UE if it receives LP-WUS configuration via RRC.

Proposal 4: LP-WUS feature is disabled at UE by network when the LP-WUS configuration is released with a RRC signaling.

2.3 UE behavior when LR is unable to monitor LP-WUS occasion:

There could be cases when the UE cannot use the LR to perform LP-WUS monitoring and a consistent UE behavior is expected as this can lead to loss of data as UE would not monitor PDCCH without LP-WUS reception but network could have scheduled some data towards the UE. This can be avoided if UE reports to the network on whether LR is in use or not via L1/L2 signaling which can keep both the network and UE in sync on ability to use LP-WUS feature. But this solution requires introduction of new signaling and thus not preferable in this stage.

In case of option 1-1, since this case is associated with the legacy DRX cycle, this can be handled similar to the missing DCP case where the NW can configure the UE to start the drx-ondurationTimer upon not detecting a LP-WUS.

In case of option 1-2, its simpler to apply the same principle and use same flag to enable UE starting the lpwus_PDCCHMonitoringTimer when LR is not able to monitor LP-WUS or LP-WUS is not detected in the monitoring occasion.

Proposal 5: A new indication is configured by network to indicate the UE to start drx-ondurationTimer (option 1-1) or lpwus_PDCCHMonitoringTimer (option 1-2) when UE does not detect a LP-WUS or LR is not able to monitor the LP-WUS monitoring occasion.

# Conclusion

Proposal 1: Support LP-WUS monitoring when dual DRX groups are configured

Proposal 2: LP-WUS monitoring is configured only in PCell/PSCell and upon receiving LP-WUS, the corresponding DRX timer (legacy onDurationTimer option 1-1and timer related to option 1-2) is started in cells associated to both the DRX groups

Proposal 3: LP-WUS feature is enabled implicit at UE if it receives LP-WUS configuration via RRC. Proposal 4: LP-WUS feature is disabled at UE by network when the LP-WUS configuration is released with a RRC signaling.

Proposal 5: A new indication is configured by network to indicate the UE to start drx-ondurationTimer when UE does not detect a LP-WUS or LR is not able to monitor the LP-WUS monitoring occasion.

Proposal 5: A new indication is configured by network to indicate the UE to start drx-ondurationTimer (option 1-1) or lpwus_PDCCHMonitoringTimer (option 1-2) when UE does not detect a LP-WUS or LR is not able to monitor the LP-WUS monitoring occasion.