3GPP TSG-RAN WG2 Meeting #130	R2-2503911

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.17

Source:		Lenovo

Title:		DL and UL impacts of TDD pattern in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding TDD support in IoT NTN:

RAN2#129 Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN) RAN2#129bis Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

In this contribution we discuss some potential issues to support TDD mode in IoT-NTN from RAN2 perspective.

- Discussion

Due to the scheduling of configured TDD pattern, a downlink/uplink operation may overlap with non-D(ownlink)/non-U(plink) subframes, and how to handle these cases are still under RAN1 discussion. In this contribution we provide some discussion from RAN2 perspective.

Essential SIB (e.g., SIB31) acquiring impacted by TDD pattern

An overlap example of SI window reception with inactive downlink time (non-D subframes) at UE is shown in Figure 1. With NTN TDD pattern, it is possible that the SI window start time falls in the inactive downlink time. SI messages like SIB2 and SIB31 can exceed 208 bits and requires total 8 subframes to transmit a single SI transport block, while some of the 8 subframes are reserved for NPSS/NSSS/NPBCH/SIB-1. Therefore, the SIB transmission has to spread across two DL active time opportunities (i.e., a set of continuous x DL subframes).

Figure 1. An overlap example of SI window reception with inactive downlink time (non-D subframes) at UE

RAN1 have discussions on solutions including new periodicities to align with the TDD structure, postponement of a channel when it overlaps with non-U/D NB-IoT subframes, restriction of a channel to be fully confined within a single set of U/D NB-IoT subframe, or dropping (full or partial) of channels when they overlap with non-U/D NB-IoT subframes. From RAN2 perspective, the impact on UE SIB31 acquirasion (e.g., uplink synchronization, timer handling) shall be considered with any possible solution adopted by RAN1.

The reception timing of SI is depending on TDD resource configuration. More specifically, the reception could be postponed or even fail when overlapped with inactive downlink time (non-D subframes), while the acquirasion of SIB31 is essential to IoT-NTN UE for uplink synchronization.

In IDLE SIB31 acquiring is done before ephemeris expiration by UE implementation. With TDD pattern configured it is possible that an IDLE UE attempts to acquire SIB31 before ephemeris expiration, but does not succeed before expiration due to long DL inactive time, as shown in Figure 2.

Figure 2. SIB31 acquiring for IDLE UE when overlapped with DL inactive time (non-D subframes)

Proposal 1: UE in IDLE determines the timing of SIB31 acquirasion based on the DL active time derived from the TDD resource configuration.

The duration of SIB31 acquirasion before ephemeris expiration at least covers necessary DL active duration (e.g., 8 subframes for SIB31 transmission, or one or two DL active time opportunities before ephemeris expiration), e.g., to ensure UL sync without delay for RACH initiation or SDT. Meanwhile, it is reasonable that	the UE only monitors for SIB31 reception at the DL active time opportunities or subframes.

Proposal 2: UE in IDLE only monitors for SIB31 reception at the DL active time opportunities or subframes derived from the TDD resource configuration.

In CONNECTED SIB31 acquiring is controlled by T317 and T318:

- Upon receiving SIB31 UE starts T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

- T317 expiration triggers UL sync lost (suspending all UL operations) and starts T318 to acquire SIB31. T318 expiration triggers RLF.

With TDD pattern configured it is possible that a CONNECTED UE starts T318 upon T317 expiration, but fails to acquire SIB31 during T318 running due to long DL inactive time, as shown in Figure 3.

Figure 3. SIB31 acquiring for CONNECTED UE when overlapped with DL inactive time (non-D subframes)

UE in CONNECTED also handles the timer and behaviour for SIB31 acquirasion based on the DL active time derived from the TDD resource configuration. The following two options can be considered:

Proposal 3a: For UE in CONNECTED, if current ephemeris expires within DL inactive duration (or a non-D subframe), UE may start SIB31 acquirasion before current ephemeris expiration considering DL active time opportunities or subframes.

Proposal 3b: For UE in CONNECTED, if the expiration timing of the configured T318 is within DL inactive duration (or a non-D subframe), UE can continue SIB31 acquirasion considering DL active time opportunities or subframes delaying RLF.

Uplink reporting impacted by TDD pattern

From RAN2 perspective, due to the scheduling of configured TDD pattern, an uplink reporting could be delayed if triggered in UL inactive time or non-U subframes. The reporting could be time-sensitive including:

- Timing Advance Report (TAR) that includes the timing advance applied at UE and can be used by eNB for UL scheduling. The value varies over time.

- GNSS validity duration reporting that includes the remaining validity duration of the latest GNSS position fix result and can be used by eNB for GNSS measurement gap configuration. The value varies over time but can be easily derived.

Therefore, it may be necessary to discuss how to handle these reports triggered at a non-U subframe, including:

- Option 1: Postpone assembling the reporting element to the next UL active time opportunity or subframe to include the newest value.

- Option 2: Assemble the reporting element once triggered and report the triggering timing (time stamp) of the corresponding value.

- Option 3: Assemble the reporting element once triggered and report the variation of the corresponding value.

- Option 4: Assemble the reporting element once triggered and report an estimated value at the time of next UL active time opportunity or subframe.

Proposal 4: RAN2 to discuss how to handle an UL reporting triggered at a non-U subframe based on TDD resource configuration.

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and operation information exchange for S&F operations in IoT NTN. The following proposal are provided:

Proposal 1: UE in IDLE determines the timing of SIB31 acquirasion based on the DL active time derived from the TDD resource configuration.

Proposal 2: UE in IDLE only monitors for SIB31 reception at the DL active time opportunities or subframes derived from the TDD resource configuration.

Proposal 3a: For UE in CONNECTED, if current ephemeris expires within DL inactive duration (or a non-D subframe), UE may start SIB31 acquirasion before current ephemeris expiration considering DL active time opportunities or subframes.

Proposal 3b: For UE in CONNECTED, if the expiration timing of the configured T318 is within DL inactive duration (or a non-D subframe), UE can continue SIB31 acquirasion considering DL active time opportunities or subframes delaying RLF.

Proposal 4: RAN2 to discuss how to handle an UL reporting triggered at a non-U subframe based on TDD resource configuration.