3GPP TSG-RAN WG2 Meeting #130	R2-2503930

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.13.4

Source:	Nokia

Title:	Discussion on service continuity for multi-hop relay

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc discuss’ aspects of service continuity for sidelink Multi-hop relay.

# 2	Discussion

In the first releases of sidelink relay, separate SL-RSRP and SD-RSRP thresholds were defined since they may use different transmit powers, power control mechanism, and different resource pool. As of this, the SL-RSRP and SD-RSRP are not comparable with each other. The same reason shall be applied for defining the SL-RSRP or SD-RSRP threshold for U2N relay selection and re-selection.

In current TR38.331, selection and reselection of NR SL U2N relay is defined as:

#### 5.8.15.3 Selection and reselection of NR sidelink U2N Relay UE A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall: 1> if the UE has no serving cell; or 1> if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-RemoteUE-ConfigCommon/sl-RemoteUE-Config: 2> if the UE does not have a selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or NOTE 1: U2N Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. If SD-RSRP is used, the discovery procedure will be performed between the U2N Remote UE and the selected U2N Relay UE. 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers indicate not to use the currently selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers request the release of the PC5-RRC connection; or 2> if the UE has a selected NR sidelink U2N Relay UE, and sidelink radio link failure is detected on the PC5-RRC connection with the current U2N Relay UE as specified in clause 5.8.9.3: 3> perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs: 4> when evaluating the one or more detected NR sidelink U2N Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2N Relay UE ID and using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the sl-FilterCoefficientRSRP in SidelinkPreconfigNR (out of coverage), before using the SD-RSRP measurement results; 4> consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria; 3> if the UE detects any suitable NR sidelink U2N Relay UE(s): 4> consider one of the available suitable NR sidelink U2N relay UE(s) can be selected; NOTE 2: A candidate NR sidelink U2N Relay UE which meets all AS layer criteria defined in 5.8.15.3 and higher layer criteria defined in TS 23.304 [65] can be regarded as suitable NR sidelink U2N Relay UE by the NR sidelink U2N Remote UE. If multiple suitable NR sidelink U2N Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2N Relay UE. The details of the interaction with upper layers are up to UE implementation. NOTE 3: For L2 U2N Remote UEs in RRC_IDLE/INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE. Furthermore, L3 U2N Remote UE's selection on both cell and NR sidelink U2N Relay UE is also based on UE implementation. 3> else: 4> consider no NR sidelink U2N Relay UE to be selected. When evaluating the currently selected NR sidelink U2N Relay UE, the U2N Remote UE should apply layer 3 filtering as specified in 5.5.3.2 using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the sl-FilterCoefficientRSRP in SidelinkPreconfigNR (out of coverage), before using the SL-RSRP or SD-RSRP measurement results.

As seen in the above, the UE uses the same sl-RSRP-Thresh to trigger the SL discovery procedure for U2N relay reselection regardless of whether it is SL-RSRP or SD-RSRP of the current selected U2N relay UE measured by the remote UE. If the sl-RSRP-Thresh is configured according to SL communication transmission while the remote UE measures the SD-RSRP of the currently selected U2N Relay UE, the remote UE may not be triggered to perform the NR SL discovery procedure even if when necessary as the transmission power for SD-RSRP may be higher than the power-controlled SL-RSRP.

Although discussed in the previous releases, this issue is even more predominant in multi-hop relay, as the (re)selection procedure is more challenging, resource, and time consuming. Thus, we believe that in multi-hop relay, we should at least consider the difference between the two measurement results.

Proposal 1: RAN2 to discuss whether at least multi-hop service continuity should consider the SL-RSRP/SD-RSRP difference when comparing measurements for relay (re)selection.

Proposal 2: RAN2 to agree that the network may provide a separate SD-RSRP threshold for U2N relay reselection.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 to discuss whether at least multi-hop service continuity should consider the SL-RSRP/SD-RSRP difference when comparing measurements for relay (re)selection.

Proposal 2: RAN2 to agree that the network may provide a separate SD-RSRP threshold for U2N relay reselection.