3GPP TSG-RAN WG2 Meeting #130	R2-2503945

St. Julian’s, Malta, 19 - 23 May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and decision

Introduction

In this contribution, we provide our considerations on adaptation of common signal/channel transmissions.

Discussion

## 2.1 Adaptation of paging

In previous meetings, RAN2 has achieved the following progress for paging adaptation:

- On PF/PO calculation: Ns is extended to 8 and N is extended to T/32.

- On PEI: Separate PEI configuration is provided and R19 UEs only monitor the new configuration.

- On UE capability: A new UE capability for paging enhancement is added in UE-RadioPagingInfo.

During the discussion of [POST129bis][111][NES] (Ericsson), the following open issues related to paging adaptation are listed:

5.2.2.2.2 Editor’s note: FFS pharsing for paging adaptation pos only, that is those occasions that are not also configured for legacy. Consider e.g.: pagingAdaptation-NS Number of paging occasions per paging frame for paging adaptation. The UE supporting paging adaptation ignores this field in RRC_CONNECTED. pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. The UE supporting paging adaptation ignores this field in RRC_CONNECTED. DownlinkConfigCommonSIB Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation? UE-RadioPagingInfo Editor’s note: FFS details

In the following discussion, we provide our views on the FFSes related to paging configuration, while the capability detail can wait until other stage-3 aspects are made clear.

The first Editor’s Note in clause 5.2.2.2.2 is related to the following agreement made in RAN2#129bis:

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

Paging monitoring for RRC_CONNECTED UEs can be categorized as follows:

Case 1 (active BWP is initial BWP): similar to UEs in RRC_IDLE/RRC_INACTIVE, the UE monitors paging based on the common search space configured in initial BWP

Case 2 (active BWP is not initial BWP, and the active BWP is configured with common search space): UE monitors paging based on the common search space configured in the active BWP

Case 3 (active BWP is not initial BWP, and the active BWP is not configured with common search space): UE does not monitor paging, the updated SI is sent directly to the UE via dedicated RRC signalling

The NES gain of the RAN2#129bis agreement is targeted at Case 2, in which the network only send the paging DCI on legacy POs. Therefore, it is straightforward to let R19 UEs to ignore the additional paging configuration in RRC_CONNECTED.

Proposal 1: Add the clarification “The UE supporting paging adaptation ignores this field in RRC_CONNECTED” into the field descriptions of pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset.

However, a related question is, if the UE in RRC_CONNECTED only monitors legacy PO, it should monitor legacy PEI as well. In the last meeting, RAN2 agreed that:

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

RAN2 needs further clarification on whether the above agreements apply to RRC_IDLE/RRC_INACTIVE only, and what is the intended behaviour for RRC_CONNECTED UEs.

Proposal 2: RAN2 to discuss whether RRC_CONNECTED UE ignores the R19 PEI configuration.

On firstPDCCH-MonitoringOccasionOfPO, in existing spec the value range of depends on SCS and N (number of PFs in paging cycle T:

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

In R17, the parameter is extended considering the SCS of 480kHz:

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

To ensure backward compatibility, legacy firstPDCCH-MonitoringOccasionOfPO still needs to be broadcast for legacy POs, but additional values need to be configured for additional POs in the PF, i.e. firstPDCCH-MonitoringOccasionOfPO-r19. The value range needs to be extended: considering largest SCS of 480kHz and smallest N of T/32 (PF appears in every 32 frames), the maximum number of PDCCH monitoring occasions for this PF is 32 frames = 10240 slots = 143360 symbols in case of 480kHz SCS. Therefore, the existing maximum value of “(0.. 71679)” is not sufficient, it should be extended to “(0..143359)”.

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

Proposal 3: A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32.

In existing spec, PEI configuration comprises the following:

We think both pei-Config and pei-ConfigBWP are needed for R19 PEI configuration. Take firstPDCCH-MonitoringOccasionOfPEI-O as an example, this parameter is used to determine the PEI monitoring occasions after the UE determines the reference frame (based on pei-FrameOffset-r17). If firstPDCCH-MonitoringOccasionOfPEI-O is not configured, it is unclear how the UE locates the PEI, and reusing the same configuration from legacy PEI configuration largely restricts the PEI transmission flexibility. Similar to P1, the value range also needs to be extended.

Proposal 4: A separate pei-ConfigBWP is introduced, which contains pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O-r19. The value range of firstPDCCH-MonitoringOccasionOfPEI-O-r19 is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32.

According to existing spec, PEI can be associated to at most 2 PFs and at most 8 POs:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

The current restriction on the number of PF(s)/PO(s) associated to PEI assumes that there could be at most 4 POs in a PF. The number of POs associated to PEI need to balance the PEI overhead per PO and the paging latency/ UE power consumption. The following was agreed in RAN1#107:

Agreement Support mapping one PEI to POnumPerPEI PO(s) in one or multiple PF(s) - POnumPerPEI is a factor of (total PO number in a paging cycle) and configurable via SIB for the cell with the value range of {1, 2, 4, 8} - The Maximum number of PF associated with one PEI is up to 2 - Note: Maximum number of paging indication bits in DCI format 2_7 can be kept the same for any configuration of POnumPerPEI, e.g., by applying a smaller subgroupsNumPerP and a larger POnumPerPEI. - Note: Larger value of POnumPerPEI can reduce the average PEI overhead per PO, but there can also cause potentially larger paging latency and larger UE power consumption due to longer UE wake-up time before PO monitoring, which can be significant with large value of (T/N).

Observation 1: In the existing spec, PEI is associated to at most 2 PFs, at most 8 POs, based on the assumption that each PF has at most 4 POs. The restriction is a trade-off between PEI overhead and paging latency.

However, now that RAN2 has extended Ns to 8, PEI can only be associated to one PO if po-NumPerPEI-r17 is not extended. This causes the average PEI overhead per PF to be quite high. To still allow the PEI to be associated to 2 PFs, and also considering there are remaining bits in DCI format 2_7, POnumPerPEI can be extended to 16.

Proposal 5: Extend POnumPerPEI to 16 to maintain the same principle as R17 that PEI can be associated with 2 PFs.

## 2.2 Adaptation of RACH

In RAN2#129, the following agreements have been made for RACH adaptation:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

Since RAN2 has agreed not to incorporate the additional RACH resources into RACH partitioning framework, and the additional RACH resources will be used for CBRA, it is straightforward to add the additional RACH resources in BWP-ConfigCommon in the form of RACH-ConfigCommon. This can address the following issue raised in [POST129bis][111][NES] (Ericsson):

RACH adaptation: 1) FFS: How to configure additional RACH resource for RACH adaptation in RRC, with consideration of RAN1 input.

Proposal 6: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

On the other hand, RAN2 agreed that RACH partitioning with all the features and feature combinations are supported. It is still unclear how the RACH resource selection is performed when PRACH adaptation is used jointly with RACH partition features, for instance, how to enable RACH adaptation for a Redcap UE.

If RACH adaptation is enabled for Redcap UEs:

If the Redcap UE selects RA resources from Branch 1, it cannot benefit from RACH adaptation;

If the Redcap UE selects RA resources from Branch 2, it cannot apply early identification.

To solve this, either a Redcap dedicated RACH-ConfigCommon is added to BWP-UplinkCommon (i.e. add a RACH-ConfigCommonRedcapNES-r19 on top of RACH-ConfigCommonNES-r19) or a NES dedicated RACH-ConfigCommon is added to AdditionalRACH-Config-r17 (i.e. add a RACH-ConfigCommonNES-r19 on top of RACH-ConfigCommon):

The similar mechanism needs to be applied for other features (e.g. SDT, Slicing) if these features can also be applied jointly with RACH adaptation.

Proposal 7: RAN2 to down-select from the following for (e)Redcap UEs:

- Option 1: A Redcap-specific R19 RACH-ConfigCommon is added to BWP-UplinkCommon;

- Option 2: An NES-specific RACH-ConfigCommon is added to AdditionalRACH-Config-r17;

- Option 3: RACH adaptation is not supported for (e)Redcap UEs.

The similar approach applies to other RA features (e.g. SDT, Slicing).

According to RAN1 agreement, the additional RA resources can be adapted by DCI:

Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

In legacy RACH partitioning framework, the RACH resource set selection is performed after carrier selection, BWP selection and feature applicability determination. After RACH resource set selection (in which UE will choose from legacy RA resource set and R19 NES resource set), the UE performs RA type selection, initialize RA variables, and starts msg1 transmission.

If the RAR reception is not successful or contention resolution is not successful, the UE will select preambles to retransmit, until preambleTransMax is reached. However, in the context of RACH adaptation, it is possible that after UE selects the additional RA resources, the RA resource is deactivated before successful completion of RA procedure. In this case it is not preferred to let the UE keep retransmitting preambles consider the maximum number could be as many as 200:

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

Proposal 8: RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

In RAN1#120bis meeting, an LS was sent to RAN2 [1], asking RAN2 to confirm whether the use of 1-bit in short message is feasible:

Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

The motivation is to use the 1-bit to indicate whether the additional RO is available for PDCCH order triggered RACH. From our perspective, it is feasible to use one of the spare bits in short message to indicate the availability of additional RO.

Proposal 9: Reply to RAN1 that it is feasible to use one of the spare bits in short message to indicate the availability of additional RO.

## 2.3 Adaptation of SSB

In RAN2#129, the following agreements were made for SSB adaptation:

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

In RAN2#129bis, RAN2 further agreed that:

Agreements on common channel adaptation

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

RAN2 agreed to use an additional SMTC on SCell for SSB adaptation for serving cell measurements. In case of intra-frequency measurements, the SMTC configured in servingCellMO will be used for both serving cell and intra-frequency neighbour cells, there is no motivation to deviate from this principle and exclude intra-frequency neighbour cells. The additional SMTC is preconfigured to the UE, and will come into use upon reception of DCI 2_9.

Proposal 10: The additional SMTC configured on SSB for SSB adaptation is used for serving cell and intra-frequency neighbour cell measurements upon receiving DCI 2_9.

However, based on existing RAN1 agreement, it is not clear whether the time location for adapted SSB is based on RRC configuration or based on the timing of DCI 2_9. RAN1 only agreed that the adapted SSB will appear no earlier than slog m + d:

---- cited from RAN4#113 WF R4-2420098 Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

Further progress in RAN1 is expected before RAN2 discusses the details of SMTC determination (e.g. whether the SMTC offset is based on pre-configuration or based on DCI 2_9).

Another open issue is whether to have a unified L3 RRM measurement design for both OD-SSB and SSB adaptation. We think this is not possible given the current RAN2 and RAN4 progress. RAN2 has already agreed SSB adaptation will be based on SMTC, while RAN4 has agreed that OD-SSB will be based on OD-SSB pattern rather than MO configuration:

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

Proposal 11: RAN2 does not aim at a unified L3 RRM measurement design for both OD-SSB and SSB adaptation.

Conclusion

In this contribution, we discussed the adaptation of common signal/channel transmissions, and made the following proposals:

Proposal 1: Add the clarification “The UE supporting paging adaptation ignores this field in RRC_CONNECTED” into the field descriptions of pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset.

Proposal 2: RAN2 to discuss whether RRC_CONNECTED UE ignores the R19 PEI configuration.

Proposal 3: A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32.

Proposal 4: A separate pei-ConfigBWP is introduced, which contains pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O-r19. The value range of firstPDCCH-MonitoringOccasionOfPEI-O-r19 is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32.

Observation 1: In the existing spec, PEI is associated to at most 2 PFs, at most 8 POs, based on the assumption that each PF has at most 4 POs. The restriction is a trade-off between PEI overhead and paging latency.

Proposal 5: Extend POnumPerPEI to 16 to maintain the same principle as R17 that PEI can be associated with 2 PFs.

Proposal 6: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

Proposal 7: RAN2 to down-select from the following for (e)Redcap UEs:

- Option 1: A Redcap-specific R19 RACH-ConfigCommon is added to BWP-UplinkCommon;

- Option 2: An NES-specific RACH-ConfigCommon is added to AdditionalRACH-Config-r17;

- Option 3: RACH adaptation is not supported for (e)Redcap UEs.

The similar approach applies to other RA features (e.g. SDT, Slicing).

Proposal 8: RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Proposal 9: Reply to RAN1 that it is feasible to use one of the spare bits in short message to indicate the availability of additional RO.

Proposal 10: The additional SMTC configured on SSB for SSB adaptation is used for serving cell and intra-frequency neighbour cell measurements upon receiving DCI 2_9.

Proposal 11: RAN2 does not aim at a unified L3 RRM measurement design for both OD-SSB and SSB adaptation.

References

- R2-2503312, LS on DCI-based PRACH adaptation (R1-2503086; contact: Ericsson), RAN1