3GPP TSG-RAN WG2 Meeting #130	R2-2503954

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Further discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - Configuration for LP-WUS

Regarding the issue of how the NW provides the configuration to the UE, it can rely on the existing RRC measurement report. For example, the NW configures the UE to report measurement results based on some events and periodicity, then the NW can know whether it is suitable for the UE to start using LP-WUS functionality, e.g. the NW finds that the measurement result is good enough, or the change of the measurement result is small enough, the NW can configure the UE to use LP-WUS. However, the NW only knows the measurement results from MR. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

Observation 1: The NW only knows the measurement results from MR based on existing RRC measurement report. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

In this case, the UE can inform the NW to stop using LP-WUS since the UE knows the link quality of LR, and also indicate whether the LP-WUS can be used again if it finds that the LR link quality is good enough. For example, the UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS. How the UE determines the LR link quality is not good enough and sends the assistance information, it can be up to UE implementation without configuring additional conditions/thresholds.

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: When to send the assistance information is up to UE implementation, without additional configuration to the UE.

After the NW provides the LP-WUS configuration to the UE by RRC signaling, the UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality. Separate activation/deactivation signaling, e.g., L1/L2 signaling is not needed, since it won’t change dynamically.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

We understand enabling LP-WUS functionality does not mean UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time). Thus, one issue that needs discussion is whether the UE starts LP-WUS monitoring once it receives the LP-WUS configuration.

Observation 2: Enabling LP-WUS functionality does not imply UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time).

One option is to start LP-WUS monitoring immediately after receiving the LP-WUS configuration. It seems like LP-WUS configuration can trigger the UE enter LP-WUS monitoring status. However, if the UE applies LP-WUS monitoring ahead of the NW, ongoing services from the NW may be affected which causes data loss for the UE. On another hand, if the LP-WUS functionality takes effect on the NW before the UE, UE may miss the LP-WUS which leads to LR/MR state mismatch between the UE and the NW. Thus, it is beneficial that the UE and NW can align the starting point for monitoring LP-WUS, e.g. the UE and the NW start LP-WUS monitoring after a time period after the UE sends RRC reconfiguration complete message.

Another option is to keep the UE stay in C-DRX Active Time (i.e., using MR) after it receives the LP-WUS configuration. As the UE receives RRC reconfiguration message and needs to receive PDCCH scheduling PUSCH for sending RRC reconfiguration complete message, the UE can naturally stay in C-DRX Active Time. And there is no interruption for ongoing service if any. After the data transmission is finished and C-DRX Active Time ends, the UE goes back to LP-WUS monitoring according to the agreed UE’s behavior.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

  - UE behavior in case of LP-WUS not detected

In RAN2#129bis meeting, the following working assumption was reached.

- Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

For DCP, three “3>” were specified including:

- the 1st “3>” describes the case that DCP is detected;

- the 2nd “3>” describes the case that DCP occasion(s) are not able be monitored;

- the 3rd “3>” describes the case that DCP occasion(s) are able to be monitored but DCP is not detected.

2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

Regarding the case that LP-WUS occasion(s) are not able be monitored, the same principle can be applied to Option 1-2. To ensure the opportunity for timely data transmission, UE can start the new timer. Considering the duration of new timer won’t be too long, the additional power consumption is acceptable.

Proposal 5: (MAC-2) For Option 1-2, when the UE is not able to monitor the LP-WUS occasion(s), the UE starts the new timer (similar to DCP).

The reason why we have the case that DCP occasion(s) are able to be monitored but DCP is not detected, because the DCP is a DCI with bitmap indicating whether to wake-up or not, e.g. “1” indicates “wake-up” and “0” indicates “not wake-up”. Then if the DCP is not detected, a default understanding needs to be defined, which relies on ps-Wakeup configuration from the network. However, LP-WUS is a codepoint, if UE needs to wake up when the LP-WUS is not detected, it means UE will always wake up whether the LP-WUS is detected or not, which makes LP-WUS pointless. Thus, if the LP-WUS is not detected for a configured LP-WUS occasion, UE does not start the either drx-OnDurationTimer (for Option 1-1) or the new timer (for Option 1-2).

Proposal 6: If the LP-WUS is monitored but not detected for a configured LP-WUS occasion, UE does not start the drx-OnDurationTimer (for Option 1-1) or the new timer (for Option 1-2).

  - LP-WUS with CA

In RAN1#120bis meeting, RAN1 discussed CA with dual DRX groups with two alternatives, but there is no consensus in RAN1 which of the following options to support for this case in Rel-19.

- Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

Currently, legacy DCP and secondary DRX cannot be configured simultaneously. Some companies have concern on FR2 power consumption, thus, to balance the power consumption and standard work, we understand the compromise can be to support LP-WUS configured in PCell with secondary DRX at the same time, but LP-WUS indication is applicable to all serving cells in the cell-group without separate handling for different DRX groups/serving cells. With secondary DRX configured, a separate drx-OnDurationTimer and drx-InactivityTimer can be configured and the most power saving gain can be obtained, the power consumption saved by supporting separate control for LP-WUS wake-up itself is marginal.

Proposal 7: (MAC-1, RRC-3) For both Option 1-1 and Option 1-2, LP-WUS can be configured in PCell with secondary DRX at the same time, and LP-WUS indication is applicable to all serving cells in both DRX groups of the cell-group, without separate control for different DRX groups.

  - Coexistence with other features

    - Combination with Rel-16 DCP

In RAN2#126 meeting, we had the following agreement:

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The same principle can be applied to Option 1-2, since Option 1-2 can technically provide better performance for power saving and latency compared with DCP. We don’t see the motivation to configure both legacy DCP and Option 1-2, besides, such combination gives additional complexity for the specification.

Proposal 8: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

    - Impacts on SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 9: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 10: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Configuration for LP-WUS

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: When to send the assistance information is up to UE implementation, without additional configuration to the UE.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

UE behavior in case of LP-WUS not detected

Proposal 5: (MAC-2) For Option 1-2, when the UE is not able to monitor the LP-WUS occasion(s), the UE starts the new timer (similar to DCP).

Proposal 6: If the LP-WUS is monitored but not detected for a configured LP-WUS occasion, UE does not start the drx-OnDurationTimer (for Option 1-1) or the new timer (for Option 1-2).

LP-WUS with DC/CA

Proposal 7: (MAC-1, RRC-3) For both Option 1-1 and Option 1-2, LP-WUS can be configured in PCell with secondary DRX at the same time, and LP-WUS indication is applicable to all serving cells in both DRX groups of the cell-group, without separate control for different DRX groups.

Coexistence with other features

Proposal 8: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

Proposal 9: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 10: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

- References

R2-2501332 Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT) report.

3GPP TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2.