3GPP TSG-RAN2#129bis										            R2-2503974

St Julian, Malta, 19th – 23rd May, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	RLC enhancements for XR

Agenda item:	8.7.5

Document for: Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. .

In this contribution, we will further discuss the RLC re-transmission related enhancements and give our proposals.

# Discussion

Based on the current RLC running CR for XR[3], when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxObsoleteSDU is configured and the RLC SDU or a segment thereof has been submitted to the lower layers, the transmitting RLC entity will stop transmission or retransmission of the RLC SDU or the segment, thus the receiving PDCP entity cannot receive the discarded SDU.

Observation 1: Based on the current agreement, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxObsoleteSDU is configured, the receiving PDCP entity cannot receive the discarded SDU.

Based on the current specification[2], when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) or segment(s) thereof have been submitted by RLC to lower layers, it shall not trigger a PDCP SN gap report, thus the receiving PDCP entity cannot know that the SDU(s) have been discarded and will wait for the SDU(s) until the t-Reordering Timer expiring, this is not the intention of the PDCP SN gap report.

Observation 2: Based on the current specification, when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) have been submitted by RLC to lower layers, it shall not trigger the a PDCP SN gap report.

Since transmitting PDCP entity already considers the RLC entity information(e.g. whether the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers), it is reasonable that the transmitting PDCP entity consider the RLC parameter(e.g. stopReTxObsoleteSDU) to decide whether to trigger PDCP SN gap report.

Observation 3: Based on the current specification, transmitting PDCP entity already considers the RLC entity information, it is reasonable to also consider the RLC parameter(e.g. stopReTxObsoleteSDU) when necessary.

Proposal 1: If stopReTxObsoleteSDU is configured, PDCP SN gap report can be triggered even when the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers.

For SDU discard indication from upper layer (e.g. PDCP), there are two branches:

The first is [2]:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

The second is[3]:

##### 5.2.3.1.1 General ... If stopReTxObsoleteSDU is set to enabled, when receiving a discard indication for an RLC SDU with SN = x from the upper layer (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment for transmission or retransmission. Editor’s Note: FFS on the term, whether it should be “obsolete”, or “outdated”, or “discard”. Same as below.

Based on the above description, we can concluded that: if the RLC SDU or a segment thereof has been submitted to the lower layers, the transmitting side of an AM RLC entity shall not discard the indicated RLC SDU; the transmitting side of an AM RLC entity only stops the transmission or retransmission of the indicated RLC SDU.

Observation 4: Based on the current RLC running CR, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if the RLC SDU or a segment thereof has been submitted to the lower layers, it will not been discarded, and will not be transmitted.

Then for DL, if t-RxDiscard is not configured, the RLC SDU or RLC SDU segment(s) will stay in the transmitting window and lead the transmitting window stalling.

Observation 5: Based on the current RLC running CR, If the receiving side timer based discarding mechanism has not been activated, the transmitting window maybe stalling. It is a critical issue to be solved.

To avoid transmitting window maybe stalling, the following options can be considered:

Option 1: Transmitting window stalling is avoided based on t-RxDiscard mechanism. Considering that theUL transmitting window forwarding depends on UL receiving side timer based discarding mechanism, but t-RxDiscard is specified only for DL, the UL receiving side timer based discarding is up to network implementation. How to specify the dependency between the transmitting side unnecessary (re) transmission and receiving side timer based discarding should be considered.

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU indicated from upper layer (e.g. PDCP) to discard.

Proposal 2: RAN2 discuss which of the following option is used to deal with the Transmitting window stalling:

Option 1: Transmitting window stalling is avoided based on t-RxDiscard mechanism.

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU .

If Option 1 is agreed, RAN2 specify the dependency between the transmitting side unnecessary (re) transmission and receiving side timer based discarding.

If Option 2 is agreed, RLC SN gap report should be used to avoid the receiving window stalling.

Considering that the unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, it is more easily been coordinated, we prefer option2.

Observation 6: Unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, option 2 is more easy to be specified.

To support RLC SN gap report, similar as PDCP SN gap report, the RLC control PDU including the SN(s) of Discarded RLC SDU and/or Discarded RLC SDU segments is preferred.

Proposal 3: If option 2 is agreed, an RLC SN gap report is supported, which uses RLC control PDU including the SN(s) of Discarded RLC SDU(s) and/or Discarded RLC SDU segment(s).

# Conclusion

The following proposals are made:

Observation 1: Based on the current agreement, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxObsoleteSDU is configured, the receiving PDCP entity cannot receive the discarded SDU.

Observation 2: Based on the current specification, when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) have been submitted by RLC to lower layers, it shall not trigger the a PDCP SN gap report.

Observation 3: Based on the current specification, transmitting PDCP entity already considers the RLC entity information, it is reasonable to also consider the RLC parameter(e.g. stopReTxObsoleteSDU) when necessary.

Proposal 1: If stopReTxObsoleteSDU is configured, PDCP SN gap report can be triggered even when the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers.

Observation 4: Based on the current RLC running CR, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if the RLC SDU or a segment thereof has been submitted to the lower layers, it will not been discarded, and will not be transmitted.

Observation 5: Based on the current RLC running CR, If the receiving side timer based discarding mechanism has not been activated, the transmitting window maybe stalling. It is a critical issue to be solved.

Proposal 2 (RLC 10): RAN2 discuss which of the following option is used to deal with the Transmitting window stalling:

Option 1: Transmitting window stalling is avoided based on t-RxDiscard mechanism

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU

Observation 6: Unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, option 2 is more easy to be specified.

Proposal 3 (RLC 10): If option 2 is agreed, an RLC SN gap report is supported, which uses RLC control PDU including the SN(s) of Discarded RLC SDU(s) and/or Discarded RLC SDU segment(s).

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- 3GPP TS 38.321 V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification

- [POST129bis][505][XR] RLC running CR and open issues (vivo)